ABSTRACT

Report 139 pages, 25 figures, 11 tables, 63 sources, 18 app.

INTELLIGENT CONTROL SYSTEMS, COMPLEX OBJECT, SMART - TECHNOLOGY, ARTIFICIAL IMMUNE SYSTEMS, MODIFIED ALGORITHMS

The object of the research is control systems for complex objects of industrial automation in the oil and gas industry.

The goal of the research is to develop cognitive Smart - technologies, models, algorithms and software for creating effective intelligent control systems for complex nonlinear dynamic objects with parameter uncertainties for various applications based on the development of the latest approaches of artificial immune systems (AIS) and other artificial intelligence methods using modern computer facilities.

Work methods. During the research, there were used modified AIS algorithms.

Results of work and their novelty. For 2020, the following main results were obtained:

- there was created a system for diagnostics of industrial equipment on the basis of modified AIS algorithms and the AMDEC approach using modern microprocessor technology from Schneider Electric. The assessment of the degree of criticality of U700 equipment failures of the TengizChevroil enterprise was carried out on the basis of the calculation of the RPI risk priority index, the analysis of the criticality of equipment faults was carried out using the indicators of the degree of criticality and the probability of faults.

- there was developed an information optical Smart-technology of distance learning for the distributed control Experion PKS system by Honeywell for operators of the oil and gas industry based on the AIS approach and on an innovative methodology for the ergonomic design of cognitive mnemonic diagrams, taking into account the peculiarities of vision and psychophysiological characteristics of students.

- there was developed software for the implementation of a Smart-technology for creating intelligent complex control systems for various applications.

Field of application of the results: Industrial enterprises of the oil and gas industry.

РЕФЕРАТ

Есеп 139 б., 25 сурет., 11 кесте., 63 дереккөз, 18 қосымша.

ИНТЕЛЛЕКТУАЛДЫ БАСҚАРУ ЖҮЙЕЛЕРІ, КҮРДЕЛІ ОБЪЕКТ, SMART – ТЕХНОЛОГИЯ, ЖАСАНДЫ ИММУНДЫ ЖҮЙЕЛЕР, МОДИФИКАЦИЯЛАНҒАН АЛГОРИТМДЕР

Зерттеу объектісі - мұнай-газ саласындағы өндірістік автоматтандырудың күрделі объектілерін басқару жүйелері болып табылады.

Зерттеудің мақсаты – есептеу техникасының заманауи құралдарын пайдалана отырып жасанды иммундық жүйелердің (ЖИЖ) және басқа да жасанды интеллект әдістерінің негізінде әртүрлі қосымшаларды құру үшін параметлері белгісіз күрделі сызықтық емес динамикалық объектілерді басқарудың тиімді интеллектуалды жүйелерін құрудың когнитивті Smart – технологиясын, модельдерін, алгоритмдерін және бағдарламалық жасақтамасын жасау.

Жұмысты жүргізу әдісі. Зерттеу жүргізу барысында модификацияланған ЖИЖ алгоритмдері қолданылды.

Жұмыстың нәтижелері және олардың жаңашылдығы. 2020 жыл үшін келесідей негізгі нәтижелер алынды:

- Schneider Electric фирмасының заманауи микропроцессорлық техникасын пайдаланатын AMDEC тәсілдемесінің және ЖИЖ модификацияланған алгоритмдерінің негізіндегі өндірістік жабдықтардың диагностикалық жүйесі құрылды.  RPI тәуекелінің басымдығының индексін есептеу негізіндегі ТеңізШевройл кәсіпорнының У700 жабдығының істен шығу қаупінің деңгейін бағалау жүзеге асырылды, жабдықтың істен шығуының сыни деңгейі мен бұзылу ықтималдығының көрсеткіштерін қолдана отырып талдау жасалды.

- ЖИЖ негізінде мұнай - газ саласының операторларына арналған Honeywell фирмасының Experion PKS таратылған басқару жүйесін қашықтықтан оқытудың ақпараттық-оптикалық Smart-технологиялары жасалды және оқып-үйренушілердің психофизиологиялық сиппаттамалары мен көру ерекшеліктерін ескере отырып когнитивті мнемосұлбасының эргономикалық жобалау әдістемесі құрылды.

- Әртүрлі қосымшаларға арналған интеллектуалды күрделі басқару жүйелерін құру үшін Smart-технологиясын жүзеге асырудың бағдарламалық жасақтамасы құрылды.

Нәтижелерді қолдану саласы: Мұнай-газ саласының өнеркәсіптік кәсіпорындары.

TABLE OF CONTENTS

| INTRODUCTION................................................................................................................. | 8 |
| --- | --- |
| 1 Diagnostics of industrial equipment and prediction of faults based on modified algorithms of artificial immune systems and AMDEC methodology…………………... | 11 |
| - Problem statement………………………………………………………..………… | 12 |
| 1.2 Industrial equipment diagnostics system based on AMDEC……….......................... | 12 |
| 1.3 Development of a diagnostic system based on modified AIS algorithms and AMDEC..................................................................................................................... | 13 |
| - Modeling results........................................................................................................ | 18 |
| - Conclusion................................................................................................................. | 21 |
| 2 Development of mnemonic diagrams for complex objects control using the achievements of artificial intelligence and cognitive technologies…………………..... | 22 |
| 2.1 Problem statement ………………………….………….......…………...…………... 2.2 Research methods and algorithms……………………………………………….….. 2.3 Optical Cognitive Smart-technology of DL................................................................ 2.4 Modeling results.......................................................................................................... 2.5 Conclusion.................................................................................................................. | 23 24 25 28 33 |
| 3. Development of software for the implementation of a smart-technology for creation of a control system for technological processes of the oil and gas industry……………… | 34 |
| 3.1 Smart technology based on modified AIS algorithms ……………………………... 3.2 Implementation of a Smart-technology for creation of a control system for technological processes in the oil and gas industry................................................... 3.3 Conclusion.................................................................................................................. | 34 36 43 |
| CONCLUSION...................................................................................................................... | 44 |
| REFERENCES...................................................................................................................... | 48 |
| APPENDIX A Calendar plan for 2018-2020........................................................................ | 53 |
| APPENDIX B List of published works for 2020………………………………………….. | 59 |
| APPENDIX C Reprints of publications for 2020………………………………………….. | 66 |
| APPENDIX D List of used foreign information resources................................................... | 119 |
| APPENDIX E U700 Installation, Hardware Specification................................................... | 120 |
| APPENDIX F Specification of U700 valves......................................................................... | 121 |
| APPENDIX G Failures and ways of their elimination on U700…………………….…….. | 122 |
| APPENDIX К An example of equipment diagnostics based on the AMDEC approach….. | 123 |
| APPENDIX L Measurement range of U700 sensors............................................................ | 124 |
| APPENDIX M Description of U700 parameters................................................................... | 125 |
| APPENDIX N Database of readings from U700 sensors..................................................... | 126 |
| APPENDIX P Listing of the GA subprogram for searching the initial population for the AIS-CLONALG subprogram…………….………..………………….......... | 127 |
| APPENDIX R Listing of the AIS-NSA subprogram for determination of the local minima of quality criteria……………………..…………………………………… | 128 |
| APPENDIX S Listing of the AIS-CLONALG subprogram for determination of the parameters of the controllers………………….………………………….. | 129 |
| APPENDIX T Control system modeling schemes in the MATLAB environment……..…. APPENDIX U Certificates….……………………..………………………………………. | 132 133 |
| APPENDIX W Acts of implementation…………………………………………………… APPENDIX X Copyright certificates……….……………………………………………... | 136 138 |

DEFINITIONS, SYMBOLS AND ABBREVIATIONS

The following terms with the corresponding definitions, designations and abbreviations are used in the report:

| DB | – data base |
| --- | --- |
| DL | – distance learning |
| DC | – distillation column |
| AI | – artificial intelligence |
| IS | – intelligent system |
| МАS | – multi-agent systems |
| SS | – software support |
| AIS (Artificial Immune Systems) | – artificial immune systems |
| AMDEC (l'Analyse des Modes de Défaillances, de leurs Effets et de leur Criticité) | – methodology for equipment diagnostics to identify possible failures and their causes, as well as for the analysis of the consequences of failures for the system |
| CLONALG (Clonal Selection Algorithm) | – clonal selection algorithm |
| DCS (Distributed Control System) | – distributed control system |
| FPA (Flower Pollination Algorithm) | – flower pollination algorithm |
| GA (Genetic Algorithm) | – genetic algorithm |
| GWO (Grey Wolf Optimizer) | – grey wolf optimizer |
| JADE (Java Agent Development Framework) | – software development environment for multi-agent systems |
| K-nn (Nearest neighbor method) | - nearest neighbor method |
| MATLAB (Matrix Laboratory) | - software package for solving technical computing problems |
| NSA (Negative Selection Algorithm) | - negative selection algorithm |
| RF (Random Forest) | – random forest |
| ROC (Receiver Operating Characteristic) | – a graph that allows to assess the quality of a binary classification |
| UCI (UC Irvine Machine Learning Repository) | – repository |

INTRODUCTION

Assessment of the current state of the problem and its relevance. Modern industrial control systems for complex objects are created using the latest achievements in the field of microprocessor technology and on the basis of a set of technical means of leading manufacturers. Large industrial enterprises in the oil and gas, metallurgical, aerospace and other industries are automated taking into account the requirements for the reliability, safety and efficiency of the equipment. An important factor is the timely analysis and diagnostics of control systems, since even minor unpredictable faults in equipment operation can lead to an emergency, as well as to a loss of economic efficiency of production. Distributed enterprise control systems are overloaded with a stream of data, which are mostly archived and not further analyzed. Integration of the latest achievements in artificial intelligence (AI) is an effective solution to this problem. In turn, the bioinspired approach of AIS is rapidly developing, which has the following advantages: the ability to process a large amount of production data, the ability to parallel data processing, self-training, the presence of memory and prediction at the class boundary. Since at present there are no universal AI algorithms capable of equally efficiently performing predictions for various types of production data, the development of modified AI algorithms is effective.

Basis and initial data for the development of the topic. Grant financing of the MES RK is the basis for research on the project topic. The initial data for the project are scientific publications in rating journals, publicly available databases, technical documentation of leading manufacturers of industrial equipment, as well as real production data from Tengiz Chevroil.

Justification of the research need. The active development of new innovative bio-inspired approaches of AI and AIS opens wide opportunities for creating highly effective optimal prediction and control systems for complex objects and high-tech production lines in the oil and gas industry, which contributes to the significant development of the economy of Kazakhstan.

Information about the planned scientific and technical level of development. The proposed cognitive Smart-technology for creating effective intelligent control systems for complex objects using AIS ensures the implementation of the Industrialization 4.0 program at a high level, which is a very important factor in the development of Kazakhstan's industry.

Information about the methodological support of research work. In the methodological support of scientific research, there are used the theoretical foundations of immunology, chemometrics and chemoinformatics, molecular biology, artificial intelligence and information about modern microprocessor technology, which are widely used in production.

The relationship of this work with other research projects. The research is a continuation of a large cycle of work on grant financing projects of the Ministry of Education and Science of the Republic of Kazakhstan on the development of intelligent systems for prediction and control of complex objects in conditions of parameters uncertainty using innovative bio-inspired approaches of AI: No. 0115RK00538 (2015-2017), No. 0112RK00324 (2012-2014 yy).

Research goals and objectives. The goal of this project is to develop a cognitive Smart - technology, models, algorithms and software for creating effective intelligent control systems for complex nonlinear dynamic objects with parameter uncertainties for various applications based on the development of the latest achievements of artificial immune systems and other artificial intelligence methods using modern computing technologies that ensure the technological progress of the national economy and have high socio-economic importance.

Main objectives for 2018-2020:

- Development of an effective Smart-technology for creating dynamic intelligent control systems for complex objects based on a cognitive approach and the latest AI developments (artificial immune systems, swarm intelligence algorithms, neural networks, genetic algorithms, fuzzy set theory and multi-agent systems) for various applications.

- Synthesis of a multifunctional AIS, consisting of subsystems that implement the basic mechanisms and algorithms for the functioning of the human immune system (molecular recognition, clonal selection and negative selection) in order to assess the state and to predict the behavior of an intelligent system, to diagnose equipment, for decision-making support and to promptly correct the behavior of the system.

- Creation of new modified AIS algorithms using algorithms of swarm intelligence, neural and genetic algorithms, as well as developing software for their implementation in a multifunctional AIS.

- Development of a diagnostic system for industrial equipment based on the proposed modified AIS algorithms, AMDEC approaches (Analyze Des Modes de Défaillance set des leurs Effect set leur criticité, analysis of operating modes and faults, their effect and degree of criticality) and modern microprocessor technology.

- Creation of mnemonic diagrams for managing complex objects using the latest advances in artificial intelligence and cognitive technologies.

- Implementation of a multifunctional IMS based on a multi-agent approach using cognitive agents in the synthesis of intelligent control systems for various applications in industrial automation systems, technological processes in the oil and gas industry and education.

- Creation of mnemonic diagrams for complex objects control using the latest achievements in artificial intelligence and cognitive technologies.

- Implementation of a multifunctional AIS based on a multi-agent approach using cognitive agents in the synthesis of intelligent control systems for various applications in industrial automation systems, technological processes in the oil and gas industry and education.

According to the calendar plan for 2020, the first section is devoted to the development of a diagnostic system for industrial equipment based on AMDEC approach and modified AIS algorithms using the example of industrial equipment from Schneider Electric. The second section discusses issues related to the development of an information optical Smart- technology for distance learning on the Honeywell Experion PKS distributed control system for the oil and gas industry and the creation of an innovative methodology for ergonomic design of cognitive mnemonic diagrams, taking into account the peculiarities of vision and psychophysiological characteristics of students. The third section is devoted to the development of software for the implementation of a Smart-technology for creating a control system for technological processes in the oil and gas industry.

Scientific novelty lies in the development of a Smart-technology for the synthesis of intelligent control systems for complex objects based on multifunctional AIS, using new modified algorithms that implement all AIS directions for assessing the state of the control system based on intelligent analysis of multidimensional data, predicting behavior, diagnosing equipment, supporting decision making and operational correction of the system's behavior using multi-agent, ontological approaches and cognitive technologies for various technical and technological applications of the oil and gas industry and education, as well as the possibility of implementation of this technology into real production. There are no analogs of developments for this type of tasks.

The proposed final report is a continuation of the intermediate reports for 2018-2019:

- for 2018 year - inventory number № 0218РК00177;

- for 2019 year - inventory number № 0219РК00495.

1 Diagnostics of industrial equipment and prediction of faults based on modified algorithms of artificial immune systems and AMDEC methodology

Nowadays, the design, construction or modernization of industrial plants such as oil refineries is a costly process with a number of technical difficulties. Replacement of a set of technical means or modernization of old equipment leads to a stop of a complex of technological lines and to subsequently financial losses of the enterprise, since this process practically does not pay off. Maintaining expensive equipment in working mode, timely diagnostics and prediction of critical conditions is an urgent task.

There are developed a large number of models for analyzing equipment failures and their criticality, such as: AMDEC (l'Analyse des Modes de Défaillances, de leurs Effets et de leur Criticité) [1,2], HAZOP (Hazard and Operability Study), FTA (fault tree analysis) [3], etc.

The AMDEC approach, which represents a safety assessment methodology for determining faults at the level of hardware and software systems, has become widespread [4]. For example, the work [5] presents the AMDEC methodology for determining the service life and storage of rubber seals. The methodology aims to identify potential seals and O-ring faults during manufacturing at various phases of the product life cycle.

The AMDEC approach is successfully used by a large manufacturer of industrial equipment and software, Schneider Electric [6]. According to European legislation, Schneider Electric is guided by international standards in the field of equipment safety: EN/ISO 13849-1 (Safety of equipment - Safety components); EN62061 (Safety of equipment - Functional safety), etc. The assessment of the diagnostic capabilities of Schneider Electric equipment is carried out using the analysis of emergency modes and consequences. The standard describes four ranges of equipment diagnostic capabilities: no diagnostic capability (DC <60%); low level (60% <DC <90%); medium level (90% <DC <99%); high level (99% <DC).

In turn, the intellectualization of the process of industrial equipment diagnostics is a promising area of research. Among bioinspired AI methods, the artificial immune systems (AIS) approach, widely used for the development of diagnostic systems, has proven itself well. The work [7] presents an intelligent maintenance system based on AIS, built into the equipment, which performs the function of local diagnostics. The practical implementation of the diagnostic system was carried out using multi-agent systems.

Research [8] focuses on the application of AIS with continuous learning in order to diagnose voltage disturbances in electrical distributed systems. The work [9] provides an overview of the use of artificial immune systems for fault detection, equipment diagnostics, and elimination of failures.

Since there are no universal algorithms that meet the requirements of reliability and high accuracy of the prediction, it is advisable to create modified AIS algorithms to improve the efficiency of prediction models [10].

1.1 Problem statement

The statement of the research problem is formulated as follows: it is necessary to develop a diagnostic system for industrial equipment taking into account the quality standards and diagnostic capabilities of Schneider Electric equipment based on the AMDEC methodology and modified algorithms of artificial immune systems on the example of a real complex control object Unit 700 for gas fractionation at TengizChevroil.

1.2 Industrial equipment diagnostics system based on AMDEC

The AMDEC approach is a convenient methodology for identifying possible faults and their causes, as well as for analyzing the consequences of faults on the system. This approach was firstly developed in the USA as a standard MIL-STD-1629 “Procedures for Performing a Failure Mode, Effects and Criticality Analysis” [2, 11].

AMDEC methodology is an engineering analysis by experts in a specific subject area and has the following special capabilities: it allows to identify equipment faults and identify their causes; to assess the consequences of faults for the entire system; to assess the risks of system faults and to prioritize decision making; to take actions to solve the most serious problems; to archive analysis results data in a knowledge base.

Figure 1.1 shows a general view of the table based on the AMDEC methodology and a matrix form for assessing the criticality of a fault

Figure 1.1 - AMDEC methodology

Failure assessment and criticality analysis are based on the risk priority index indicator, which is calculated using the following formula:

(1.1)

where S – «severity» degree of criticality, assessed by a scale from 1 to 4 (1 - minimum degree of criticality; 2 - significant degree of criticality; 3 - critical degree; 4 - catastrophic degree); O - "occurrence" probability of occurrence, ranging from 1 to 4 (1 - very low probability of occurrence; 2 - low degree; 3 - medium degree of occurrence probability; 4 high degree of occurrence probability); D - "Detection" is the probability of not being detected, assessed by a scale from 1 to 4.

The AMDEC methodology is a convenient platform for developing a diagnostic system for industrial equipment based on modified AIS algorithms.

1.3 Development of a diagnostic system based on modified AIS algorithms and AMDEC

The quality of the prediction depends significantly on the nature of the analyzed data, the dimension and the formulation of the research problem. There are no universal algorithms that can predict results with the same accuracy on different datasets. It is possible to increase the predictive capabilities of models by selecting informative features and compiling an optimal data set.

Flower Pollination algorithm (FPA) is a promising optimization method for selection of informative features. The FPA algorithm was developed by the scientist Yang Xin-She in 2012 [12-14] and simulates the process of flowers pollination in nature. The method of flower pollination is successfully used for equipment diagnostics. For example, in [15], a new binary algorithm for chaotic flowers pollination is presented for diagnosing analog errors. FPA is based on the natural process of flowers pollination for further reproduction of the species. There are different ways of flowers pollination: biotic form (the process of transfer of pollen is carried out with the help of pollinators); abiotic form (self-pollinated flowers). Figure 1.2 shows the process of flowers pollination in nature.

Figure 1.2 - Flowers pollination process

Let consider the flower pollination algorithm taking into account the following provisions for global and local pollination [12-14]:

Remark 1. The process of global pollination is based on a biotic form. The biotic form is characterized by the transfer of pollen from the anther of one flower to the stigma of the pistil of another flower using a pollinator (insects, birds, etc.). This process is described using a Levy flight [12].

Remark 2. Local pollination is considered in the form of an abiotic form (self-pollination), when pollen is transferred from the anther on the stigma of the pistil of one flower.

Remark 3. The concept of flower constancy is introduced, in which individual pollinators visit certain types of flowers.

Remark 4. Local and global pollination occurs with a probability

At the initial stage of the algorithm there is carried out random generation of the initial population of flowers [13-15]. In order to obtain a new solution, the type of pollination must be determined with a probability  A random number  is generated. Two cases are considered: value  and :

- If , then the provisions of Remark 1 and Remark 3 are valid. Global pollination is carried out, described by the formula:

(1.2)

where  - solution  at the moment of time ; - current best solution; - scaling factor; - step size at a Levy flight.

- If , then the provisions of Remark 2 and Remark 3 are valid. Local pollination is carried out on the basis of the formula:

(1.3)

where - two randomly selected solutions; - random number in the interval .

Then the current best solution is updated and iterations continue until the stop criterion is reached. On the basis of the flowers pollination method, there is developed a modified FPA-AIS algorithm given below.

Algorithm.

STEP 1. Collection of data on parameters of equipment failures [16]. Formation of a database of equipment faults.

STEP 2. Selection of informative features based on the flower pollination algorithm.

STEP 2.1 Initialization of the initial population of flowers with a probability . Selection of the number of iterations.

STEP 2.2 Assessment of the initial population and search for the best solution .

STEP 2.3 Equation-based global/local pollination procedure (1.2, 1.3).

STEP 2.4 Updating positions.

STEP 2.5 Updating solutions taking into account the best solution.

STEP 2.6 Saving the current best solution.

STEP 2.7 Checking the reaching of a given number of iterations.

STEP 3 Formation of a database of informative features.

STEP 4. AIS-based pattern recognition problem solution [17].

STEP 5. Equipment condition prediction. Recommendations on decision making.

Additionally, previously developed algorithms are considered as modified AIS algorithms: RF-AIS [17] and GWO-AIS [18].

The modified RF-AIS algorithm consists of the following stages: 1 - formation of a database (DB) of equipment state parameters; 2 - reduction of non-informative parameters based on the Random Forest (RF) algorithm; 3 - diagnostics of the equipment state using immune network modeling [17, 18]; 4 - evaluation of the efficiency of the algorithm; 5 - prediction of the degree of criticality of the failure, decision-making on faults elimination.

Since the prediction results largely depend on the initial data, an additional modified GWO-AIS algorithm is used for the equipment diagnostics system. This algorithm is formed on the basis of the following stages: Stage 1 - formation of a database of object parameters; Stage 2 - reduction of non-informative parameters based on the Grey Wolf Optimization (GWO) method; Steps 3-5 are similar to the modified RF-AIS algorithm.

A block diagram of a diagnostic system for industrial equipment based on modified algorithms of artificial immune systems and the AMDEC approach is shown on figure 1.3.

The principle of operation of the diagnostic system is following. At the initial stage, data collection is carried out. The formed database of parameters of industrial equipment operation (readings from sensors, information about failures, etc.) is connected to the diagnostic system. Next, the process of identifying potential weak points of the object (the equipment under consideration) is carried out: analysis of the operating experience of the object, reliability testing, etc.

At the next stage, a list of failure modes is formed for further analysis, where internal and external causes of equipment failure can be considered. Further, data mining is carried out based on modified AIS algorithms. The type of the modified algorithm is selected and the severity of the equipment failure is determined on the basis of the selected algorithm.

Then are assessed the effectiveness of the modified algorithm using characteristics (Accuracy, Classification Error, Precision, Recall, F-measure, etc.) and the influence of the failure on the entire system. After analyzing the data by the modified AIS algorithms, there is made a comparative analysis of their effectiveness. The algorithm with the best predictive result is selected. After that, a prediction and decision-making to eliminate equipment faults is carried out.

Figure 1.3 - Block diagram of a diagnostic system based on modified AIS and AMDEC algorithms

1.4 Modeling results

The effectiveness of the developed diagnostic system is assessed on the basis of real production data of TengizChevroil. Considered [19] the technological process U700 of the complex technological line 2. The modeling was carried out on the basis of software from Schneider Electric, Unity Pro (current name EcoStruxure Control Expert). Gas fractionation is carried out on Unit 700. The gas purified from acidic components is separated into fuel gas and liquefied petroleum gas. The enlarged mnemonic diagram of the U700, developed in the Unity Pro software (Schneider Electric), is shown on Figure 1.4. Description of the U700 units is presented in Appendix E (Table E.1).

Figure 1.4 - Fragment of enlarged mnemonic diagram of U700

The current technical regulations provide general deviations from the technological process, as well as possible causes and solutions [19, 20]. Appendix F (Table F.1) shows a fragment of the U700 valve specification. The specification of failures and options for their elimination in accordance with technical regulations are presented in Appendix G (Table G.1). Based on Table B.1, as well as the daily report of the TengizChevroil coordinator, a database is formed for equipment diagnostics on the basis of the AMDEC approach (Appendix K, Table K.1).

The proposed AMDEC model is supplemented with data mining based on modified AIS algorithms (FPA-AIS, RF-AIS, GWO-AIS). Let's consider an example of industrial equipment diagnostics using the U700 as an example using modified AIS algorithms. The specification of a fragment of the database of the permissible range of U700 technological parameters is presented in Appendix L (Table L.1). The purpose and place of installation of the U700 sensors are described in Appendix M (Table M.1). According to the permissible range of U700 technological parameters, there is formed a database (Appendix N) of daily measurements from sensors and information about equipment failures (a fragment of the database is given in Table 1.1,). The size of the database  is 12800 copies.

Table 1.1 - Fragment of a database for equipment diagnostics

| Unit ID | S1 | S2 | S3 | S5 | … | Sn | Failure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unit 1 | 12345.0 | 6.56 | 6.87 | 7.0 | … | 400, 347 | Yes |
| Unit 2 | 11722.7 | 5.733 | 6.0 | 5.5 | … | 423,987 | No |
| Unit 3 | 11909.1 | 5.906 | 6.6 | 5.3 | … | 187,356 | No |
| … | … | … | … | … | … | … | … |
| Unit 128 | 11132.2 | 6.245 | 6.0 | 5.0 | … | 542.23 | Yes |

The reduction of non-informative parameters of the U700 is carried out using the FPA, GWO, RF methods. Figure 1.5 shows an example of selecting informative features based on FPA. Parameters S7, S11, S4 are significant, and S9, S12 and S10 must be reduced.

Figure 1.5 - Selection of informative features based on the FPA algorithm

The next step is to predict the equipment state on the basis of AIS. For example, the experts select 2 classes: Yes - equipment failure; No - the equipment is operating normally. The criticality of the failure is assessed by a point system in the range from 1 to 4 (1 - minimum criticality, 2 - pre-emergency state, 3 - emergency state, 4 - critical state). Table 1.2 presents a comparative analysis of the modeling results of the U700 parameter database using algorithms: FPA-AIS, RF-AIS, GWO-AIS, clonal selection (CLONALG), artificial immune recognition system (AIRS) and nearest neighbor method (K-nn ).

Table 1.2 - Assessment of the effectiveness of equipment diagnostics based on AI algorithms

| Assessment | FPA -AIS | RF-AIS | GWO -AIS | CLONALG | AIRS | K-nn |
| --- | --- | --- | --- | --- | --- | --- |
| Accuracy | 96,66% | 92,72% | 95,33% | 64,64% | 61,61% | 68,44% |
| Clas. error | 3,33% | 7,27% | 4,66% | 35,35% | 38,38% | 31,66% |
| Precision | 0,942 | 0,929 | 0,923 | 0,542 | 0,623 | 0,701 |
| Recall | 0,963 | 0,927 | 0,942 | 0,667 | 0,616 | 0,770 |
| F-measure | 0,951 | 0,927 | 0,933 | 0,598 | 0,599 | 0,723 |
| ROC area | 0,965 | 0,949 | 0,952 | 0,654 | 0,571 | 0,724 |

The following characteristics were selected as assessments: accuracy, classification error, precision, recall, F-measure, ROC - error curve. Figure 1.6 presents a comparative analysis of the modeling results based on the ROC (Receiver Operating Characteristic) indicator.

Figure 1.6 - Comparative analysis of the modeling results for ROC assessment

According to the research results in this example, the modified FPA-AIS algorithm showed the highest efficiency - 96.66%. Equipment diagnostics based on the AMDEC model and modified AIS algorithms are shown on figure 1.7.

The developed diagnostic system based on the AMDEC approach and modified AIS [21] algorithms combines the latest achievements of modern equipment diagnostic methods and artificial intelligence approaches.

Figure 1.7 - Equipment diagnostics based on modified AIS algorithms

The methodology allows the remote collection and processing of information for data analysis and gives recommendations for decision making in case of equipment failures. There is reduced the load on expert operators, who must manually fill out information about the state of equipment in the AMDEC model. The introduction of modified AIS algorithms due to the properties of autonomy, adaptability and self-organization allows to identify equipment faults not only within the framework of already known failures, but also in case of emergency situations.

1.5 Conclusion

The following main results were obtained:

- Created a system for diagnostics of industrial equipment on the basis of modified AIS algorithms and the AMDEC approach using modern microprocessor technology from Schneider Electric.

- Developed a block diagram of the functioning of the industrial equipment diagnostic system based on the AMDEC approach and modified AIS algorithms using FPA, RF, GWO methods.

- The degree of criticality of equipment failures U700 of TengizChevroil was assessed on the basis of the calculation of the risk priority index RPI, the criticality of equipment failures was analyzed using indicators of the degree of criticality and the possibility of failure, and an example of the specification of possible equipment failures was given. Therefore, the intellectualization of diagnostics of industrial equipment makes it possible to simplify data analysis, timely to identify failures and to assess their criticality.

2 Development of mnemonic diagrams for complex objects control using the achievements of artificial intelligence and cognitive technologies

At modern oil and gas enterprises, when managing complex technological processes, in the presence of large amounts of information and the need to process data in real time, the risks of a person making erroneous decisions significantly increase. An urgent task is the development of cognitive methods of perception of visual information from the monitor screen to improve the efficiency of work [22], since there is an increased load on human vision. Many papers have been published for various applications devoted to this problem [23]. Analysis of statistical production data shows that about 70% of all accidents are caused by human factors. Cognitive methods are actively used in the development of complex human-machine interfaces, which can reduce the percentage of emergency situations caused by the operator [24].

World experience [25] shows the integration of industry and innovation in the development of strategic development plans and implementation of the Industry 4.0 program. Therefore, it is especially important to use innovative cognitive optical technologies and modern methods of artificial intelligence in the development of graphic mnemonic diagrams containing signal images of equipment on displays of workstations, operator screens and dispatch consoles.

In addition, the remoteness of oil and gas facilities and the need for effective and high-quality training of technical personnel on modern complex equipment is one of the important tasks facing the management of high-tech industrial production. There is an acute issue of training professional engineering personnel to work with modern technologies and complex industrial equipment. World leaders in the production of equipment, such as Honeywell, Sсhneider Electric, etc., pay great attention to distance learning (DL) of technical personnel and practical work skills. The distributed control system (DCS) of Honeywell's Experion PKS is widely used in the oil and gas industry, which is a set of tools for solving various automation problems, from collecting and processing information to optimizing the operating modes of technological processes [26].

The use of artificial immune systems approach is promising in the development of distance learning information technology for Experion PKS equipment. This approach is actively used to solve a wide range of problems in pattern recognition, optimization, prediction, classification, diagnostics, clustering, information protection and education. For example, the study [27] discusses issues related to the problem of recognition of visual images in real time using a web camera based on the AIS algorithm. The advantage of using AIS is shown due to the use of an effective learning algorithm and parallel computing.

For example, the study [27] discusses issues related to the problem of recognition of visual images in real time using a web camera based on the AIS algorithm. The advantage of using AIS is shown due to the use of an effective learning algorithm and parallel computing.

Modified AIS algorithms are also widely used. The article [28] is devoted to diagnostics of faults in wireless sensor networks based on the clonal selection algorithm of AIS and the probabilistic approach of a neural network. The modeling results showed that this method is promising. In [29], there is proposed a hybrid immune algorithm (Hybrid-IA) for solving complex combinatorial optimization problems. The algorithm is developed on the principles of clonal selection and a local search procedure using a deterministic approach to refine the found solutions. The modeling results and comparison with various algorithms confirm the efficiency and reliability of the hybrid algorithm from the point of view of the best found solutions.

The article [30] describes various hybrid computational intelligence algorithms that combine intelligent and traditional methods. There is given a comparison of the efficiency of optimization methods such as: particle swarm algorithm (PSO), evolutionary algorithm (EA) and AIS for the selected test functions.

The proposed research is a continuation of a large cycle of work on distance learning on engineering specialties. For example, in [31], a data collection procedure (OPC technology, OLE for Process Control) was developed from a real industrial automation object for organizing information exchange with an intelligent control system based on artificial immune systems. This approach allows the organization of remote access to real equipment for students.

The developments [32] are devoted to specialized distance learning technology for people with visual impairments based on artificial intelligence and multi-agent approaches. Research for people with visual disabilities, testing and experiments to determine color perception were carried out on the basis of the Almaty branch of the public association "Kazakhstan Society of the Blind". The specifics of distance learning for operators with different psychotypes and color perception are considered in [33].

2.1 Problem statement

The purpose of the research is to develop an information optical Smart-technology [34] for distance personalized learning of the distributed control system Experion PKS from Honeywell, taking into account the peculiarities of vision and psychotype of students by adjusting the color presentation of educational material, dynamic presentation of information using cognitive-visual schemes and the development of appropriate cognitive mnemonic diagrams for monitoring and controlling equipment. Prediction of learning outcomes is carried out on the basis of modified algorithms of artificial immune systems, the results of which are used to promptly correct the learning process.

2.2 Research methods and algorithms

Application of the latest achievements in the field of artificial intelligence, psychology of color presentation of educational material, taking into account the peculiarities of vision, cognitive techniques based on psychophysical perception [35] of educational information and modern IT technologies, can significantly increase the level of professional training and the intensity of training operators on high-tech equipment. Mnemonic diagrams are designed to display the functional and technological diagram of the control object, to monitor the state of the object and to signal all faults. Since mnemonic diagrams reflect the current state of systems and especially emergency modes of technological processes, the work of operators is associated with long-term observation of them, which leads to significant loads on the visual apparatus. It is advisable to use red and purple colors as little as possible when creating mnemonic diagrams. High ergonomic requirements are imposed [36] at developing mnemonic diagrams. The mnemonic diagram should not contain redundant information, and the elements that are especially important for the control and management of the object should be highlighted (by color, brightness, size, etc.). For example, a brightness contrast between signs and the background is desirable over 65%. Of particular importance are signals that are used when the state of an object changes, for example, "open-closed". These states should also be clearly distinguished by color, shape, etc. Increased requirements are imposed on special alarm signals. The signal intensity should be 30-40% higher than during normal operation, it is also possible to use an intermittent signal, which is very effective for attracting attention, since the human eye actively reacts to movement.

Functions of the operator of the Honeywell station console are to monitor and control the process by means of mnemonic diagrams [37]; notification and control of process signals; requesting reports on signals, graphical trends and data analysis; output (on screen and printing) of historical process data and trends; monitoring the state of the system equipment, etc. Usually 4 types of mnemonic diagrams are used: Level 1 (mnemonic diagram of the general level of the plant), Level 2 (mnemonic diagram of a general overview of the process unit of the plant: heat exchangers, pumps, etc.), Level 3 (operating displays, contain all the regulators and indicators for performing routine operations, and for emergency response), Level 4 (detailed interactive mnemonic diagrams of complex equipment and displays of program sequences). All items of equipment that are used on the mnemonic diagrams of Honeywell are standard (for example, C - regulator, G - pump). Colors also identify equipment status, allowing the operator to immediately identify unusual situations. For example, the color of the mnemonic icons: green - Normal, Running, Open; red - Alarm, etc. The following colors are used for the status of the regulator: red - Alarm; flashing red - unacknowledged alarm, etc.

2.3 Optical Cognitive Smart-technology of DL

Figure 2.1 shows a block diagram of Experions PKS optical cognitive smart-technology for distance learning of operators.

Figure 2.1. - Smart-technology for distance learning of Experions PKS operators

The use of optical technology based on the psychophysiology of information perception on the display screen, including color perception and visual features, makes it possible to increase the effectiveness of teaching on complex experimental PKS technological equipment using specially designed cognitive mnemonic schemes for training and the corresponding presentation of educational material.

The use of the results of psychological research to identify patterns of perception of information and cognitive technologies is an urgent problem in the creation of innovative DL technologies. The visual perception of information from a computer monitor screen or a specialized display is characterized [38] by certain preferred strategies for moving the eyes, depending on the preferred starting point and trajectory of movement. Experimental studies have shown that for effectively selection of signal structures on the information panel, it is more rational to choose the center of the screen or place information from left to right in the central part of the screen. The dependence has also been proven: the larger the object is, the less time it takes to identify it.

Human color perception plays an important role in the process of teaching complex industrial equipment. Color [39] is characterized by the following basic qualities: color background (wavelength), saturation (part of the main background plus impurities of other colors), brightness or lightness (degree of closeness to white). Also, all colors are divided into achromatic (colorless: grey colors of different lightness) and chromatic (color: blue, green, red, yellow).

When creating cognitive mnemonic schemes, it is necessary to take into account the peculiarities of color perception, which depend on age, gender, visual acuity, physical and emotional state of a person. To determine the objective individual characteristics of an operator, his psychological and physiological state, various texts are used, for example, based on the method of color choices developed by the Swiss psychologist Max Luscher [40]. For various psychotypes, a certain color is preferable [33]: for a choleric person is blue (color wavelength 440-485 Nm); for a sanguine person is green (550-565 Nm); for the melancholic is yellow color (565-590 Nm); for phlegmatic person is red (565-590 Nm).

A fairly large percentage of people have vision problems to one degree or another, ranging from various stages of myopia, hyperopia (farsightedness) and to problems with color perception (color blindness): protanopia, deuteranopia and trinatopia. Various types of color blindness usually affect men 10% and very few women 1%. Special photoreceptors of the retina in the form of cones are used to recognize shades of three primary colors: red, blue, and green [41]. Pronatopia is characterized by a weakened perception of the red color due to the absence of erythrolab pigment in the retinal cones. People (pronatopes) with these vision features do not distinguish green shades from red ones.

With deuteranopia, the green color is usually not perceived (there is no chlorolab pigment), so the light green color appears to be dark red, violet as blue, purple as Grey. Diagnostics of deuteranotopes is carried out using Rabkin's polychromatic tables, when numbers or objects are depicted against a background consisting of circles of different colors. People with a violation of the color perception of blue shades are called tritanomals (tritanopes). Vision pathology is caused by the absence of photoreceptors responsible for the perception of blue color or the corresponding cyanolab pigment in the cones. Rabkin's tables, which depict pictures with numbers of a certain color, while the background has a different color, help to define this category of people.

Tritanomals see all shades of blue and purple in shades of red or green. Testing and determining various types of color blindness of the operator is especially important to consider when working with emergency panels.

Visual acuity also affects a person's color perception. For example, with myopia [32], a person's picture is projected in front of the retina, and as a result, vision is good near and poor in the distance. Almost every third person has different stages of myopia. For this category of person, it is recommended to use calm tones of pale yellow or pale green (color wavelength 640-590 Nm). In hyperopia (farsightedness), the picture is projected after the retina, which gives good vision in the distance and poor vision in the vicinity. This feature of vision is widespread in the elderly. For hyperopia, more intense bright yellow or bright orange colors are recommended (color wavelength 580-550 Nm). With myopia, the operator perceives important information better in the upper left corner, and with hyperopia in the lower right corner.

The considered DL information technology is implemented in the class of multi-agent systems using the JADE multi-agent platform [42]. The use of this software is determined by the following advantages: the possibility of optimal use of computing resources, self-organization, reliability, scalability, the ability for agents to work autonomously, expanding with new modules, as well as effective interaction between agents. The development of the knowledge base is carried out on the basis of the ontological approach [42, 43].

The algorithm for the functioning of an information optical cognitive Smart-technology of DL based on artificial immune systems used to solve the problem of pattern recognition and predicting learning outcomes is given below.

Algorithm:

STEP 1. Creation of a knowledge base and a database (DB) of individual student descriptors based on various methods, questionnaires and testing.

STEP 2. Creation of an optimal immune network based on solving the problem of identifying informative descriptors using optimization algorithms (for example, Random Forest [44]).

STEP 3. With the use of experts, students are classified according to knowledge levels, visual features, psychophysiological state, etc.

STEP 4. Multidimensional data processing by the modified immune network algorithm AIS [45, 46].

STEP 5. Development and distance learning (based on the formed individual trajectories of training) for operators of practical work with cognitive mnemonic diagrams using the peculiarities of color perception, depending on the state of the student's visual apparatus and its psychophysiological characteristics.

STEP 6. Comprehensive assessment of knowledge, prediction of the quality of individual distance education received and operational management of learning processes.

The table below shows a comparative analysis of the modeling results based on the proposed immune network algorithm of AIS [47] and other algorithms of artificial immune systems: AIRS1, AIRS2, CLONALG.

Table 2.1 - Comparative analysis of algorithms

| Efficiency | AIRS1 | AIRS2 | CLON-ALG | AIS |
| --- | --- | --- | --- | --- |
| Accuracy | 89% | 77% | 78% | 93,7% |
| Classification Error | 11% | 23% | 22% | 7% |
| Precision | 0,894 | 0,766 | 0,822 | 0,933 |
| Recall | 0,890 | 0,770 | 0,78 | 0,930 |
| f-measure | 0,891 | 0,767 | 0,786 | 0,928 |
| Total Time | 0,07s | 0,11 s | 0,06 s | 0,06 s |

Modeling results for algorithms AIRS1, AIRS2, CLONALG were obtained using the WEKA software product. The table shows that the best result was shown by the AIS immune network algorithm. The assessment procedure by homologous proteins improves the recognition of peptides at class boundaries with almost the same structure, but belonging to different classes. The proposed Smart-technology based on the AIS immune network algorithm can be used for various applications, including for recognition of visual images.

  - Modeling results

As an example for the development of cognitive mnemonic diagrams, let’s consider the technological process of gas purification in a distillation column (DC) unit, which is widely used in the oil and gas industry. Distillation columns have a multi-section design [48, 49], consisting of: a vertical unit, inside which trays are located, the main task of which is the separation of components by contact of gas and liquid phases; a steam boiler for heating the liquid (reboiler); condenser to lower the temperature of the steam in the upper part of the column in order to obtain condensate and to collect it into a tank for irrigation. Figure 2.2 shows the technological process of gas purification in the DC.

Figure 2.2 - Technological diagram of the gas purification process

in the distillation column

According to the technological diagram of the gas purification process, figure 2.3 shows a simple mnemonic diagram for monitoring and controlling the operator of the gas purification process in the DC. Noteworthy is the Grey neutral color scheme that is commonly used on Honeywell mnemonic diagrams.

Figure 2.3 – Simple Honeywell mnemonic diagrams.

Researches on visual perception of information on mnemonic diagrams for students and teachers of JSC "Kazakh-British Technical University" were carried out. The survey involved 162 people, 44.4% of them were women and 55.6% were men aged 18 to 31 years. The results of the experiment made it possible to identify patterns in the field of focusing human vision on the operator screen. A distance method of questioning and a test consisting of questions on the topic "Cognitive perception of information" has been developed. Developing cognitive mnemonic diagrams, for the first time, the peculiarities of the perception of information of color blind people, as well as people with the right or left hand as the main one, are taken into account. As a result of the experiment, it was determined that vision problems exist in 67.3% of respondents, of which 34.6% with weak myopia, 27.8% with severe myopia and 3% with hyperopia. In addition, 1.9% of the respondents have color blindness. The respondents (regardless of the state of vision) were presented HMI displays with operator screens and asked to indicate which graphic object was defined in the first place. Figure 2.4 offers a variant of the mnemonic diagram (divided conditionally into seven areas) to determine the focus of students' attention.

Figure 2.4 - Mnemonic scheme variant for determining focus
on seven areas

Most people with normal visual acuity [50, 51] perceive information as follows: the information located on the display in the upper right corner (33% of attention), in the upper left corner of the training field (28%), in the lower right (23%) and in lower left corner (16%).

Let’s consider the survey results that show the percentage of areas that get attention in the first place. The center of the mnemonic diagram attracts the most attention (68.5%), then the upper left corner (10.9%), the top area in the middle of the screen (7.3%), etc. The bottom left corner attracts the least attention (1.2%).

Further on figure 2.5, there is proposed a variant of the mnemonic scheme (conventionally divided into four areas) to determine the focus of attention.

Figure 2.5 - Mnemonic scheme variant for determining focus
on four areas

Figure 2.6 shows the percentage of attention focus. It was determined that there was a pronounced attention to the upper left corner of the operator screen (45.5%) and the upper right corner (42.4%). Thus, the bottom area of the screen is non-informative and not intended to contain critical information.

Figure 2.6 - Percentage of focusing attention on four areas

Let’s consider statistics for people with vision problems. Figure 2.7 presents statistics for people with myopia.

Figure 2.7 - Focusing attention on the mnemonic diagram for people with myopia

As can be seen from the pie chart, people’s attention with myopia is focused on the upper left area of the HMI display (45.6% of respondents) and slightly less on the upper right area (39.8%). Based on the results of the survey analysis, we can conclude that the location of the most important information on the mnemonic diagram for people with myopia is preferably located in the upper left corner of the monitor.

Also, results were obtained on the perception of graphic information of senestrals (left-handed) and dextrals (right-handed) people. From the 162 respondents, 4.9% were senestrals, 92% were dextrals and 3.1% were two-handed. Depending on whether the person is a senestral or dextral (for a mnemonic diagram with four objects), the statistics was following: people with the left hand focused their attention on the upper right corner in 87.5% of cases. At the same time, among respondents - dextrals, on the contrary, the upper left corner prevails in 47.3% of cases.

In order to search for the most suitable range of light, the respondents were presented with two variants of the design of the mnemonic diagram: standard Grey (variant 1, Figure 2.3) and color (variant 2, Figure 2.8). The students were asked to determine which variant is most appropriate for long-term visual contact. According to the results of the survey, it was revealed that 81.8% of the respondents preferred the color version of the mnemonic diagram in blue tones, which carries a lower load on the visual apparatus, which is confirmed by previous studies [11, 12].

Since more than half of the students have varying degrees of myopia, it is relevant to develop cognitive mnemonic diagrams taking into account these features of vision. Figure 2.9 shows an example of the developed cognitive mnemonic diagram for operators with myopia.

Figure 2.8 - Cognitive mnemonic diagram for determining the best color range

Figure 2.9 - Cognitive mnemonic diagram for people with myopia

The most important information for people with myopia, according to the experiments, is located in the upper left corner. However, if the student is a senestral, then it is better to shift important information to the upper right corner. Thus, the development of cognitive mnemonic diagrams based on the peculiarities of human visual information processing can significantly reduce the risks of emergency situations during long-term monitoring of several consoles.

  - Conclusion

The developed DL approach with the use of modern cognitive visual teaching methods, using specialized cognitive mnemonic diagrams [18], allows to intensify the process of training of service personnel and to provide high quality training of technical specialists in the oil and gas industry in the shortest possible time. The proposed method of ergonomic design of cognitive mnemonic diagrams can be used to create complex industrial mnemonic diagrams, which will reduce the load on vision and improve the psychological comfort of the operator.

3 Development of software for the implementation of a smart-technology for creation of a control system for technological processes of the oil and gas industry

In this section of the project, there was developed software for the algorithm, for which the mechanisms and advantages of Smart-technologies are used: artificial immune system, AIS, and its clonal and negative selection algorithms. Technological processes of the oil and gas industry are considered as an object of control. In order to create systems for optimal control of technological processes, it is necessary to synthesize such devices for the control object as optimal typical controllers.

In the project of 2019 [52], there was solved the problem of synthesizing the optimal control of a complex process control system for the oil and gas industry based on the clonal selection algorithm, CLONALG [53]. This stage of the project is a continuation of the use of the clonal selection algorithm for artificial immune systems, but, in contrast to the previously developed one, there has been formed a modified AIS algorithm with using the genetic algorithm, GA. Algorithms that combine the advantages of the AIS algorithm and the GA algorithm are faster and more accurate than each separately: GA or AIS approach [54-56]. The modification of AIS is that the GA is used to form the initial population of the AIS, that is, the initial values of the regulator. Also at this stage of the project, the CLONALG algorithm is considered in conjunction with the negative selection algorithm, NSA, which made it possible to solve the problem of finding a global minimum, excluding local minima [57].

3.1 Smart technology based on modified AIS algorithms

The development of software for the implementation of the modified Smart-technology algorithm, including the mechanisms and advantages of the algorithms of the artificial immune system and the genetic algorithm, is used to solve the problem of synthesizing the optimal control of a complex process control system in the oil and gas industry [58].

The problem of synthesizing the optimal control of a complex system is associated with the problem of finding the minimum values of the quality criterion due to the requirements for technological processes, and the search for the corresponding parameters of typical controllers that provide the required minimum [59]. In the course of solving this problem, it is necessary to obtain the parameters of the regulators that provide the global minimum of the quality criterion.

In order to form the initial population of the AIS algorithm, that is, the initial values of the parameters of a typical controller, there is used the genetic algorithm GA, which has better characteristics than traditional regulator tuning algorithms. The operation of the modified AIS-GA algorithm is shown on figure 3.1.

Generation of the initial population - the initial

values of the regulator parameters

based on the GA algorithm

Population cycle for calculating the optimal

parameters of regulators

based on AIS algorithms:
iterative algorithm for calculating the minimum

values of quality criterion

Figure 3.1 – Operation of the modified AIS-GA algorithm

The CLONALG [52] and NSA [57] algorithms are used to implement a Smart-technology of AIS. The CLONALG algorithm is used to solve a multicriteria optimization problem for the specified quality criterion, but, at the same time, there may be several local minima. The theory of negative selection and the NSA algorithm are used in this work to solve the problem of finding unwanted local minima and selecting the global minimum.

Within the framework of biological theory, negative selection is used to explain the functioning of the immune system, as the ability to distinguish own cells from almost any foreign cells and molecules. The theory of negative selection solves the problem of searching for anomalies (uncharacteristic changes) of an object. The basic algorithm based on the theory of negative selection - NSA, created by Forrest [60], includes a mechanism for determining the permissible deviation from the usual behavior of the system as a whole. The negative selection algorithm uses a process for generating negative system positions. The initial population is generated randomly, but, further, the negative positions of the system are eliminated [61, 62]. The result of this whole process is the detected anomalies and an interest in the framework of detecting local minima.

In biological systems, a negative selection process is used prior to clonal selection. In this work, the principle of negative selection is also used to form an algorithm for the initial finding of optima before the clonal selection algorithm for finding the global minimum.

In order to detect local minima of the quality criterion of the control system, there was developed software for the negative selection algorithm - the AIS-NSA subprogram (Figure 3.2), after which the AIS-CLONALG subprogram is implemented.

AIS-NSA subprogram:

Population cycle for calculating the parameters of regulators for local minima of the objective function based on the NSA algorithm

GA subprogram:
Generation of the initial population - the initial values of the regulator parameters based on the GA algorithm

AIS-CLONALG subprogram:

Population cycle for calculating the parameters of regulators for global minima of the objective function based on the CLONALG algorithm

Figure 3.2 – Modified AIS-GA algorithm subprograms

During the implementation of the AIS-NSA subprogram, in order to find the global minimum value of the optimization problem of a complex process control system for the oil and gas industry, the following steps of the algorithm were performed:

STEP 1. Probabilistic formation of the population.

STEP 2. Calculation of the affinity of each member of the population with the objective function.

STEP 3. Selection of the members of the population with the worst characteristics (negative). Finding local minima.

STEP 4. Checking the obtained population for compliance with the quality criterion requirements.

Based on the exclusion of the found local minima, the problem of finding the global minimum using AIS-CLONALG is solved more efficiently.

Formed by the modified AIS-GA algorithm, it is the basis for obtaining the results of the implementation of a Smart-technology for creating a control system for technological processes in the oil and gas industry.

3.2 Implementation of a Smart-technology for creation of a control system for technological processes in the oil and gas industry

The implementation of the modified algorithm for solving the problem of synthesizing the optimal control system for technological processes in the oil and gas industry is carried out for a complex object of the oil and gas industry - a distillation column for the process of natural gas purification from various impurities.

The mathematical model of the distillation column, developed at the previous stages of the project [63], describes a two-dimensional control system. In this case, - the transfer functions of the object connecting the inputs and outputs of the two-dimensional system [10] with two control circuits:

.                                         (3.1)

where  – transfer functions of two isolated subsystems;

– transfer functions of interconnections between subsystems.

Problem statement for a complex system. To create a complex control system for stabilizing the processes of the distillation column (3.1), in order to achieve the desired values of the output signals.

The synthesis of the control system for gas purification in the distillation column is determined by finding the control actions for which the PI control law is selected:

(3.2)

where  – mismatch errors between desired and output signals.

Transfer functions of PI-controller (3.2) have the form [60]:

(3.3)

An integral quadratic criterion was chosen as quality criteria for adjusting the regulators, which ensures the solution of the stabilization problem:

.                                        (3.4)

Quality criteria (3.4) correspond to local regulators, , which are in separate circuits and for them there is used the procedure for calculating the integral quadratic estimate [52].

Problem statement for subsystems. To create a control system for the distillation column processes based on software that implements Smart-technology algorithms for the synthesis of typical PI-controllers (3.3) of isolated subsystems of a system (3.1), minimizing quality criteria (3.4).

To calculate the parameters of the regulators, , that provide the minimum to the criteria (3.4), there was formed the software of the modified AIS-GA algorithm [13] in the MATLAB environment: GA (Appendix P), AIS-NSA (Appendix R), AIS-CLONALG (Appendix S).

In accordance with the properties of the considered control system for gas purification in the distillation column (3.1) and the system requirements for PI-controllers, we formulate the conditions for the values of the criteria (3.4):

(3.5)

and there are limitations on the changes range in the controllers parameters for the system stability.

The parameters of the control system of the distillation column processes for the implementation of algorithms in the MATLAB environment are presented in Table 3.1.

Table 3.1 – Parameters of the control system (3.1) and quality criteria (3.4)

| Parameters | Values |
| --- | --- |
| Gain coefficients | ; ; ; |
| Time constants | ; ; ; |

The result of the GA subprogram (Appendix P) of the initial population search for the AIS subprogram is shown on figure P.1.

In accordance with the obtained values (Appendix P), the initial populations, that is, the initial values of the PI-controllers of the AIS-GA algorithm have the following values:

(3.6)

In order to search for local minima and to determine the range of finding the global minimum of quality criterion, there is implemented the AIS-NSA subprogram - the negative selection algorithm (Appendix R). On the basis of the presented algorithm, for the quality criterion (3.4), there was developed software for the implementation of the negative selection technology for creating an intelligent control system in the MATLAB environment. The result of the AIS-NSA subprogram with a wide range of PI-controller parameter values is shown on figures 3.3 and 3.4.

Figure 3.3 – Results of the AIS-NSA subprogram for the first control circuit

The presented results were obtained only for the first control circuit - for the second circuit the results are similar.

In accordance with figure 3.3, we can conclude about the range of parameters for which there are many local minima. Based on the AIS-NSA algorithm, it is possible to ignore unwanted local minima (Figure 3.4) and restrict the population in order to find the regulator parameters that would satisfy conditions (3.5).

Figure 3.4 – Results of the search for the minima of the first control circuit

During the implementation of the AIS-NSA software, it was concluded that this mechanism makes it possible to recognize and ignore unwanted extremes when searching for the optimal values of the quality criterion.

In order to search for the global minima of the quality criteria (3.4), there is used a subprogram of the AIS-CLONALG algorithm, with a generated population of initial values of the regulator parameters based on the GA and AIS-NSA subprograms. The corresponding software in the MATLAB environment is presented in appendix P, during the implementation of which the values of typical PI-controllers were obtained.

Figure 3.5 shows the results of the AIS-CLONALG subprogram for minimizing the quality criterion (3.4) for the first circuit with the initial values of the population formed on the basis of the GA and AIS-NSA algorithms.

x=P1 = 100.8357   y=I1 = 76.0759   fx = -0.2876

Figure 3.5 – Results of the AIS-CLONALG subprogram for the first circuit

Received PI-controller of the first control circuit with the parameters:

.                                                (3.7)

There was obtained the PI-controller of the second control circuit with parameters too:

.                                       (3.8)

On the basis of the obtained software results in the form of parameters of typical controllers, in the MATLAB Simulink environment there is formed a scheme for modeling a control system for a distillation column along two isolated circuits (Appendix T). At the same time, modeling circuits with three types of PI-controllers are collected:

- a regulator tuned on the basis of the AIS-GA algorithm (3.7), (3.8);

- a regulator tuned on the basis of the AIS algorithm [52]:

(3.9)

- a regulator tuned on the basis of the GA algorithm (3.6).

In accordance with the created schemes, the results of modeling the first circuit (Appendix T) of a complex control system of the distillation column (Figure 3.6) were obtained in the form of a transient process.

Figure 3.6 – Modeling results of the first circuit of the control system

According to the transient processes, it can be concluded that with the application of the AIS-GA regulator, the assessments of the quality of the transient process of the closed-circuit control system of the first circuit have better indicators than with the AIS-regulator and with the GA-regulator: overshoot is equal to zero; the settling time is the smallest (Tset=10); the rise time is the smallest; steady-state error is zero.

The modeling results of the second circuit (Appendix T) of the complex control system of the distillation column (Figure 3.7) in the form of a transient process were also obtained. Here, estimates of the quality of the transient process with a regulator tuned by the AIS-GA algorithm also give the best indicators: overshoot is equal to zero; the settling time is the smallest (Tset = 8); the rise time is the smallest; steady-state error is zero.

Based on the results obtained, we can conclude about the effectiveness of using the modified AIS-GA algorithm for the synthesis of typical PI-controllers of control systems in order to achieve the best speed and accuracy of the system processes.

Figure 3.7 – Modeling results of the second circuit of the control system

In accordance with the statement of the problem for the synthesis of a complex system, it is necessary to supplement the regulators with decoupling devices that compensate for the influence of interconnections (Figure 3.8).

Control device

Complex

control

object

AIS-GA regulators

Decoupling

devices

Figure 3.8 – Control system for technological processes of the oil and gas industry, developed on the basis of a Smart-technology

For this, the coefficients of the decoupling device are calculated according to the methodology presented at the previous stages of the project [52]. For a complex object (3.1), with regulators (3.9), the decoupling devices are:

The modeling results of a complex control system for the distillation column (Appendix T) with regulators (3.9) and a decoupling device are shown on Figure 3.9 in the form of transient process.

Figure 3.9 – Modeling results of a complex control system for the distillation column processes

Based on the transient processes of the first and second circuits of the complex control system for the distillation column in the oil and gas industry, it can be concluded that the problem of stabilizing technological processes based on Smart-technologies has been solved.

3.3 Conclusion

The main results were obtained:

- Developed software for the implementation of Smart-technology algorithms to solve the problem of synthesizing an optimal complex control system for technological processes in the oil and gas industry using the example of a distillation column for the process of purifying natural gas from various impurities (Appendix X).

- In the MATLAB environment, there was formed the software of the modified AIS-GA algorithm, consisting of the GA, AIS-NSA, AIS-CLONALG subprograms.

- Developed a subprogram of the modified AIS-GA algorithm based on the genetic algorithm to generate the initial values of typical controllers.

- Developed a AIS-NSA subprogram to recognize and ignore unwanted local minima when searching for optimal values and obtaining a range of changes in regulator parameters.

- Developed a AIS-CLONALG subprogram, as a result of which transient processes of a complex system with the best indicators of quality assessments were obtained.

CONCLUSION

During the implementation of the project on the development of a cognitive Smart-technology for intelligent systems for complex objects control based on artificial intelligence approaches, according to the calendar plan (Appendix A) for 2020, the following main results were obtained:

- Developed a system for diagnostics of industrial equipment based on modified algorithms of artificial immune systems and the AMDEC approach using modern microprocessor technology from Schneider Electric to maintain expensive equipment in working order, timely analysis and prediction of critical conditions.

- Created a block diagram of the functioning of the industrial equipment diagnostic system based on the AMDEC approach and modified algorithms of artificial immune systems (AIS) using FPA (Flower Pollination algorithm), RF (Random Forest) and GWO (Grey Wolf Optimization) methods. Expanding the AMDEC methodology through the use of intelligent prediction allows to analyze a large amount of production data and to reduce the complexity of the process of diagnosing the state of equipment.

- Carried out an assessment of the criticality degree of U700 equipment failures of the TengizChevroil enterprise based on the calculation of the RPI risk priority index, the analysis of the criticality of equipment failures was carried out using indicators of the degree of criticality and the probability of failure, as well as an example of the specification of possible equipment failures. The proposed diagnostic system allows to identify the main types of failures: physical jamming, vibrations, incorrect start/stop; short circuit; exceeding the permissible limits, etc.

- Developed mnemonic diagrams for complex objects control using the achievements of artificial intelligence and cognitive technologies.

- Created a technique for the ergonomic design of cognitive mnemonic diagrams, taking into account the peculiarities of vision and psychophysiological characteristics of students.

- Developed an information optical Smart-technology of distance learning of the distributed control system Experion PKS from Honeywell for oil and gas operators based on the approach of artificial immune systems and the use of cognitive mnemonic diagrams.

- Developed software for the implementation of a Smart-technology for creating intelligent complex control systems for various applications of industrial automation systems, technological processes in the oil and gas industry and education.

- Developed software for the implementation of a Smart-technology algorithms to solve the problem of synthesizing an optimal complex control system for technological processes in the oil and gas industry using the example of a distillation column for the purification of natural gas from various impurities.

- In the MATLAB environment, there was formed the software of the modified AIS-GA algorithm, consisting of the GA, AIS-NSA, AIS-CLONALG subprograms.

- Developed a subprogram of the modified AIS-GA algorithm based on the genetic algorithm in order to generate the initial values of typical regulators.

- Developed a subprogram AIS-NSA to recognize and ignore unwanted local minimum when searching for optimal values and obtaining a range of changes in regulator parameters.

- Developed a subprogram AIS-CLONALG, as a result of which transient processes of the system with the best indicators of quality assessments were obtained.

- There was received a copyright certificate for the developed software (Appendix B).

In 2020, based on the research results, 20 scientific articles were published in journals and materials of prestigious international conferences (Appendix A), including 5 articles (quartile Q2, Q3, Q4) with an impact factor in the Thomson Reuters and Scopus databases.

During the implementation of the project in 2019, the following main results were obtained:

- Developed an intelligent control system based on a Smart-technology and a multi-agent approach.

- Developed a multifunctional artificial immune system based on a multi-agent approach.

- Developed a module for a multifunctional multi-agent AIS using a modified algorithm of grey wolves optimization and AIS on the example of the operation of a medium-pressure absorber of technological Unit 300 for gas purification from acidic components at TengizChevroil, as well as a comparative analysis of the effectiveness of the algorithm.

- Developed a module for a multifunctional multi-agent AIS based on a modified cooperative particle swarm algorithm with inertia weight (CPSOIW) and tested on standard UCI Machine Learning datasets.

- Developed an architecture and described the functions of cognitive agents for the synthesis of an intelligent control system (there was proposed the architecture of interaction of cognitive agents based on the Belief-Desire-Intention (BDI) model in the framework of the Model Driven Architecture (MDA) concept and described the functions of cognitive agents on the example of a medium pressure absorber).

- Developed scenarios of interaction of cognitive agents based on the analysis of predicted events during the operation of a complex object (medium pressure absorber).

- Developed a sophisticated intelligent control system for the distillation of gas through the distillation column of the oil and gas industry based on a Smart-technology. There was solved the problem of synthesis of optimal control using the algorithm of AIS based on clonal selection, which was selected as a result of a comparative analysis with other algorithms of Smart-technology (genetic algorithm and an ant colony optimization algorithm). For the implementation of a Smart-technology for the synthesis of a complex intelligent control system, there were obtained the results of the decoupling procedure.

During the implementation of the project for 2018, the following main results were obtained:

- Developed a Smart-technology for creating intelligent control systems for complex objects based on a cognitive approach and the latest developments in artificial intelligence.

- Conducted an analytical review of the state of Artificial Immune Systems. Developed the theoretical foundations for creating a multifunctional AIS. Developed a multifunctional AIS based on modified algorithms of swarm intelligence, cognitive technologies and a multi-agent approach.

- Considered the main control mechanisms based on distributed control systems of Honeywell DCS enterprises, as well as the principles of data transmission and storage for working with predicted events. Proposed an architecture of an intelligent system based on a modified AIS algorithm (clonal selection) for integration with Honeywell DCS.

- Developed a modified inertia weight particle swarm algorithms (IWPSO) and a cooperative particle swarm algorithm (CPSO) for the multifunctional AIS.

- Obtained the results of modeling of a modified particle swarm algorithm (IWPSO and CPSO) based on real production data of the oil and gas company TengizChevroil for diagnostics of industrial equipment (based on daily measurements of readings from the sensors of the U300). Conducted a comparative analysis of the modeling results with the classical particle swarm algorithm (PSO).

- Developed an innovative AIS-based cognitive Smart-technology for distance learning for visually impaired engineers in a Honeywell laboratory of common use for training industrial equipment using Experion PKS distributed control system. The use of a cognitive approach makes it possible to provide high-quality personalized distance learning, depending on the type of the central nervous system of students (choleric, melancholic, etc.), as well as the characteristics of vision, taking into account the psychotype.

For three years, based on the results of the research, it was published:

- Monograph (Appendix A).

- There are 63 articles in journals and materials of prestigious international conferences, 11 of which (quartile Q1, Q2, Q3, Q4) with an impact factor in the Thomson Reuters and Scopus databases (Appendix A).

- There was received 3 certificates of state registration of rights to the copyright object (for computer programs).

- There was received an act of introducing the results of scientific research on the project into the educational process of the Kazakh-British Technical University.

- There was received an act of introducing the results of scientific research on the project into the educational process of Satbayev University.

Scientific novelty lies in the development of a Smart-technology for the synthesis of intelligent control systems for complex objects based on multifunctional AIS, using new modified algorithms that implement all AIS directions for assessing the state of the control system based on intelligent analysis of multidimensional data, predicting behavior, diagnosing equipment, supporting decision making and operational correction of the system's behavior using multi-agent, ontological approaches and cognitive technologies for various technical and technological applications of the oil and gas industry and education, as well as the possibility of implementation of this technology into real production. There are no analogs of developments for this type of tasks.

REFERENCES

- Cabarbaye A. Sûreté de Fonctionnement & Optimisation des systèmes. – СAB Innovation, 2017. - 243 p.

- Lyonnet P., Marc T., Rasario, T. Fiabilité, diagnostic et maintenance des systèmes. - Paris, France, 2012. – 377 p.

- Fuentes-Bargue J.L., González-Cruz M., González-Gaya C., Baixauli-Pérez, M.P. Risk Analysis of a Fuel Storage Terminal Using HAZOP and FTA // Intern. J. of Environmental Research and Public Health. – 2017. – Vol. 14, №705. – Р. 1-26.

- Ahram T., Karwowski W., Vergnano A., Leali F., Taiar R. Intelligent Huma Systems Integration. - Modena, Italy. – 2020. – 1312 p.

- Chandrasekaran С.V. Storage and Service Life of Rubber Seals // Rubber Seals for Fluid and Hydraulic Systems. - Elsevier Inc, 2010. - 148 с.

- Gerin M. Guide des solutions d'automatisme: Schémathèque. - Schneider Electric, 2007. – 104 р.

- Zuccolotto M., Fasanotti L., Cavalieri S., Pereira C.E. Artificial Immune Intelligent Maintenance System – Diagnostic Agents // Proceeding of IFAC. – 2014. – Vol. 47, №3. – Р. 7116-7121.

- Lima F., Lopes M., Lotufo A., Minussi C. An artificial immune system with continuous-learning for voltage disturbance diagnosis in electrical distribution systems // Expert systems with applications. – 2016. – Vol. 56. – P.131-142.

- Bayara N., Darmoulb S., Hajri-Gabouja S., Pierrevalc H. Fault detection, diagnosis and recovery using Artiﬁcial Immune Systems: A review // Engineering application of artificial intelligence. – 2015. – Vol. 46. – P. 43-57.

- Padmanabhan S., Chandrasekaran M., Ganesan S., Khan M. and Navakanth P. Optimal Solution for an Engineering Applications Using Modified Artificial Immune System // Materials Science and Engineering. – 2017. – Vol. 183. – P. 1-65.

- Carlson C.S. Effective FMEAs. - John Willey & Sons, 2012. – 453 р.

- Yang X.-S. Flower Pollination Algorithm for Global Optimization // Lecture Notes in Computer Science. – 2012. – Vol. 7445. – P. 240-249.

- Yang X.-S. Flower Pollination Algorithms. Nature-Inspired Optimization Algorithms. – 2014. – 173 p.

- Abdel-Basset M. Flower pollination algorithm: a comprehensive review // Artificial intelligence review. – 2019. – Vol. 52. - P. 2533-2557.

- Cui W., He Y., Zhao D., Zhu J. A Novel Binary Chaotic Cloud Flower Pollination Algorithm for Analog Fault Diagnosis // Proceeding of IEEE 3rd Advanced Information Technology, Electronic and Automation Control Conf. – 2018. – P. 1175-1179.

- Samigulina G.A., Samigulina Z.I. Development of industrial equipment diagnostics system based on modified algorithms of artificial immune systems and AMDEC approach using schneider electric equipment // Proceeding of the Intern. Conf. on Industrial Engineering, Applications and Manufacturing. – 2020. – P. 1-5.

- Samigulina G.A., Samigulina Z.I. Modified immune network algorithm based on the Random Forest approach for the complex objects control // Artificial Intelligence Review. – 2019. - Vol. 52, №4. – P. 2457–2473, 2019.

- Samigulina G.A., Samigulina Z.I. Development of Smart-Technologies for prediction and control of complex objects based on modified algorithms of artificial immune systems. - Science Book Publishing House Yelm, WA, USA, 2020. – 224 p.

- Technological regulations of U700, KTL2. TengizChevroil LLP, TP-KTL-1-700-13. – 2014 – P. 127. (in Russian).

- Permanent technological schedule for the process of extracting LPG at U-700. TengizChevroil, TP-SGP-700-11. -2017 – P. 134. (in Russian).

- Samigulina G.A., Samigulina Z.I. Diagnostic system for industrial equipment in the oil and gas industry based on modified algorithms of artificial immune systems and AMDEC approach // Mater. scientific. conf. of IICT MES RK "Modern problems of informatics and computing technologies". - Almaty, 2020. - P. 303-309. (in Russian).

22 Kothe C.A., Makeig S. BCILAB: A platform for brain-computer interface development // J. Neural Eng. – 2013. – Vol. 10(5). – P. 056014 // DOI:10.1088/1741-2560/10/5/0560141- 25.

23 Major T., Conrad J. A survey of brain computer interfaces and their applications // Proceeding of conference Southeastcon, IEEE. – Lexington, 2014. – P. 1-9 // DOI:10.1109/SECON.2014.6950751.

24 Filippis L., Gaia E., Guglieri G., Re M., Ricco C. Cognitive Based Design of a Human Machine Interface for Telenavigation of a Space Rover // J. of aerospace technology and management. – 2014. – Vol. 6, № 4. – P. 415-430.

25 Gioacchino de Candia. Industry 4.0 and its aberrations. – 2019 // https://www.researchgate.net/publication/337949690_Industry_40_and_its_aberrations: 14.01.2020.

26 Par'ev G.V. Centralized control system for technological objects based on the Unified operator room of LUKOIL-Volgogradneftepererabotka // Automation in industry. - 2015, - No. 7. - P. 41-47. (in Russian).

27 Mechelson R.M. The use of an artificial immune system for recognition of visual images // Computer Optics. - 2018. – Vol. 42, No. 1. - P.113-117. (in Russian)

28 Mohapatra S., Khilar P.M., Swain R.R. Fault diagnosis in wireless sensor network using clonal selection principle and probabilistic neural network approach // International Journal of Communication Systems. - 2019. - № 32(12) // DOI:10.1002/dac.4138.

29 Cutello V., Oliva M., Pavone M., Scollo R.A. A Hybrid Immunological Search for the Weighted Feedback Vertex Set Problem. A Chapter in book: Learning and Intelligent Optimization. -2020 // DOI: 10.1007/978-3-030-38629-0_1.

30 Burczynski T., Kus W., Beluch W., Szczepanik M. Intelligent Computing Techniques. Chapter in book: Intelligent Computing in Optimal Design. – 2020 // DOI: 10.1007/978-3-030-34161-9_3.

31 Samigulina G.A., Samigulina Z.I. Intelligent System of Distance Education of Engineers, based on Modern innovative Technologies // Proceeding of the II Intern. Conf. on Higher Education Advances; J. Procedia - Social and Behavioral Sciences. – Valencia, Spain: Elsevier, 2016. – Vol. 228. – P. 229-236.

32 Samigulina G.А., Shayakhmetova A.S., Nyusupov A.T. Innovative intelligent technology of distance learning for visually impaired people // Open engineering. – 2017. - № 7. – C. 444-452.

33 Samigulina G.A., Lukmanova Zh.S. Cognitive Smart-technology of distance learning in modern industrial automation equipment // Bulletin of NTU "KhPI". - Kharkiv, 2018. - No.2(1318). - P.160-170. (in Russian).

34 Krishnamurthy E.V., Kris Murthy V. On engineering smart systems // Proceeding KES of the 9th Intern. Conf. on Knowledge-Based Intelligent Information and Engineering Systems. – Australia, 2005. – Vol. 3. – P. 505-512.

35 K. V. Beloskova, S. L. Artemenkov Experimental study of the order of perception of text information on the display screen // PsyJournals. - 2010. - P. 230-234. (in Russian).

36 GOST R IEC 60073-2000. Human-machine interface. Marking and designations of controls and control devices. - M. 2000, No.348-st. (in Russian).

37 TCO GRK. TengizChevroil. Haneywell GUS Basic Principles Training. 3SG / 3VP. - 127 p. (in Russian).

38 Alexandrov Yu.I. Psychophysiology: textbook. – SPb, – 2018 - 464 p. (in Russian).

39 Ronnier Luo. Encyclopedia of Color Science and Technology. – Springer Science+Bisiness Media, NY, 2016. – 457 p.

40 Spasennikov V.V. The phenomenon of color perception in ergonomic research and color consulting // Ergodesign. - 2019. - No. 2. - P.51-59. (in Russian).

41 Pathology of tritanopia - I cannot distinguish blue. - 2016 // https://ofthalm.ru/tritanopija-tritanomalija.html/: 01.15.2020. (in Russian).

42 Samigulina G.A., Samigulina Z.I., Lukmanova Zh.S. Cognitive Smart technology of distance learning of Experion PKS distributed control system for oil and gas industry using ontological approach // News of the Academy of sciences of the Republic of Kazakhstan. Series of Geology and Technical Sciences. – Almaty, 2020. - № 1(439). – P. 23-31.

43 A.a. №1882. Development of an intellectual control system for distance education based on immune network modeling (computer program) / Samigulina GA, Samigulina ZI; publ. 27.12.2010. (in Russian)

44 Samigulina G.A., Samigulina Z.I. Modified immune network algorithm based on the Random Forest approach for the complex objects control // Artificial Intelligence Review. – 2019. – Vol. 52(4). – P. 2457-2473.

45 Tarakanov A., Nicosia G. Foundations of immunocomputing // Proceeding of the 1-st IEEE Symposium of Computational Intelligence. – Honolulu, 2007. – P. 503-508.

46 Samigulina G.A. Development of the decision support systems on the basis of the intellectual technology of the artificial immune systems // Automatic and remold control. – Springer, 2012. – Vol. 74, № 2. - P. 397-403.

47 A.a. №1601. Data_Analyzer (HC program) / Samigulina G.A., Samigulina Z.I.; published 4.12.2013.

48 Minh V.T., Rani A.A. Modeling and control of distillation column in a petroleum process // Mathematical problems in engineering. - Perak: Hindawi Publishing Corporation, 2009. // https://doi.org/10.1155/2009/404702: 14.06.2020.

49 Chinjiang Liu. Optimal design of high-rise building wiring based on ant colony optimization // Cluster Computing. – Springer, 2018. – P. 1-8.

50 Garyaev A.V., Garyaeva T.P. Psychological and physiological features of visual perception of information and their consideration when creating educational presentations. Bulletin of PSPU. - 2008. - No. 4. - P. 106-113. (in Russian).

51 Klingberg T. Training and plasticity of working memory // Trends in Cognitive Sciences. – 2010. – Vol. 14, - № 7. – P. 317-324.

52 Development of a cognitive Smart-technology for intelligent control systems of complex objects based on artificial intelligence approaches: Project report on the topic (interim report) / NC STE: Head Samigulina G.A.; per. Shiryayeva O.I. – Almaty, 2019. – 182 p. – № GR 0118РК00188. – Inv. №0219РК00495. (in Russian).

53 Shiryayeva O., Samigulin T. CLONALG application to the PID-controller synthesis of MIMO-systems in oil and gas industry // Informatyka, Automatyka, Pomiary w Gospodarce i Ochronie Środowiska. – Lublin, 2019. – № 3. – P. 50-53 // DOI: 10.35784/iapgos.235.

54 Ali M.O., Koh S.P., Chong K.H. Design a PID Controller of BLDC Motor by Using Hybrid Genetic-Immune // Modern Applied Science. – 2011. – Vol. 5, № 1. – P.75-85.

55 Jia Lu, Stankevich L.A. Immune network with a genetic tuning algorithm for PID control // Telecommunication systems and computer networks. – 2009. – No. 4. – P. 27-36. (in Russian).

56 Khoie M., Salahshoor K., Ehsan N., Sedigh A.K. PID Controller Tuning using multi-objective optimization based on fused genetic-immune algorithm and immune feedback mechanism. – Springer-Verlag Berlin Heidelberg, 2012. – 276 p.

57 Shiryayeva O.I., Samigulin T.I. Development of software for the implementation of negative selection technology for creating an intelligent control system for the oil and gas industry // Mater. of conf. "Satpayev readings -2020". – Almaty: KazNRTU named after K.I. Satpayev, 2020. – P. 245-248. (in Russian).

58 Shiryayeva O.I., Samigulin T.I. Implementation of SMART-technology for designing optimal systems based on modified algorithms // Herald of the National Technical University "KhPI". Series of "Informatics and Modeling". – Kharkov: NTU "KhPI". – 2020. – № 1(3). – P. 41-49. (in Russian).

59 Shiryaeva O.I., Samigulin T.I. Basic concepts of the development of an artificial immune system for a class of complex systems // Bulletin of KazNRTU. – 2019. – No. 5. – P. 501-504. (in Russian).

60 Forrest S. Principles of a Computer Immune System // Proceeding of the Second New Security Paradigms Workshop. – 1997. – P. 75-82.

61 Ayara M., Timmis J., Lemos R., Castro L., Duncan R. Negative Selection: How to Generate Detectors // Proceeding of the 1st International Conference on Artificial Immune Systems. – UK, 2002. – Vol.1. – P. 89-98.

62 Marciniak J., Wawryn K., Widulinski P. An Artificial Immune Negative Selection Algorithm to Control Water Temperature in the Outlet of the Chamber // International conference on signals and electronic systems. – Poland, 2018. – P. 236-241.

63 Samigulin T.I., Shiryaeva O.I. Development of a Smart-control system for a complex technological process in the oil and gas industry using bioinspired algorithms // Bulletin of KBTU. – 2019. – T. 16, Issue. 4. – P. 164-171. (in Russian).

APPENDIX A

Calendar plan for 2018-2020 уу.

Appendix 1.13

to the Agreement № from 2018

on grant financing

TECHNICAL SPECIFICATIONS AND CALENDAR PLAN

Under Agreement No. 211 dated March 19, 2018

1. Republican state enterprise on the right of economic management "Institute of Information and Computing Technologies" of the Science Committee of the Ministry of Education "Science of the Republic of Kazakhstan

1.1 By priority: №3. Information, telecommunication and space technologies, scientific research in the field of natural sciences.

1.2 By sub-priority: 3.1 Intelligent information technologies. Intelligent control and decision-making systems (including in real time).

1.3 On the topic of the project: No.AP05130018 "Development of a cognitive Smart - technology for intelligent systems for complex objects control based on artificial intelligence approaches."

1.4 The total amount of the project is 45,000,000 (forty-five million) tenge, including with a report by years, for the execution of work in accordance with paragraph 3:

- for 2018 - in the amount of 15,000,000 (fifteen million) tenge;

- for 2019 - in the amount of 15,000,000 (fifteen million) tenge;

- for 2020 - in the amount of 15,000,000 (fifteen million) tenge.

2. Characteristics of scientific and technical products by qualification characteristics and economic indicators

2.1 Direction of work: Development of an innovative information Smart-technology for complex objects control based on modified artificial intelligence algorithms.

2.2 Scope of application: technical, technological and socio-economic complex objects of control.

2.3 Final result:

- for 2018: there will be developed a Smart-technology for creating intelligent systems for complex objects control based on a cognitive approach and the latest developments in artificial intelligence. Journal article: J. Artificial Intelligence review. (Thomson Reuters, Scopus impact factor - 2,627).

- for 2019: there will be developed an intelligent control system based on a Smart-technology and a multi-agent approach. Certificate of authorship.

- for 2020: there will be developed software (software) for the implementation of a Smart-technology for creating AIS for various applications of industrial automation systems, technological processes of the oil and gas industry and education. Monograph in a foreign publishing house. Publications will be published in publications indexed in Scopus or Web of Sciences with a non-zero impact factor.

2.4 Patentability: It is possible to obtain a copyright certificate.

2.5 Scientific and technical level (novelty): A high level of novelty. The project proposes the creation of a fundamentally new multifunctional AIS based on new modified AI algorithms implemented using multi-agent and cognitive technologies.

2.6 The use of scientific and technical products is carried out: by the executor.

2.7 Type of application of the result of scientific and (or) scientific and technical activities:
Cognitive Smart-technology can be used in industrial automation systems, technological processes in the oil and gas industry and education.

3. Name of work, terms of their implementation and results

| Task, stage code | Name of work under the Agreement and the main stages of its implementation | Period of execution |  | Expected result |
| --- | --- | --- | --- | --- |
|  |  | beginning | ending |  |
| 1 | Development of a Smart - technology for creating intelligent systems for complex objects control based on the cognitive approach and the latest AI developments. | January 2018 | November 1, 2018 | Smart technology will be developed for creating intelligent systems for complex objects control based on a cognitive approach and the latest AI developments. |
| 1.1 | Analytical review of the state of the AIS. Development of theoretical foundations for creating a multifunctional AIS. | January 2018 | June 1, 2018 | An analytical review of the state of the AIS will be carried out. Theoretical foundations will be developed for the creation of a multifunctional AIS |
| 1.2 | Creation of new modified algorithms for multifunctional AIS. | July 2018 | November 1, 2018 | New modified algorithms for multifunctional AIS will be created. An article will be published in the journal: J. Artificial Intelligence review. (Thomson Reuters, Scopus Impact Factor -2.627). |
| 2 | Development of an intelligent control system based on Smart-technology and a multi-agent approach. | January 2019 | November 1, 2019 | An intelligent control system will be developed based on a Smart-technology and a multi-agent approach. |
| 2.1 | Development of a multifunctional AIS based on a multi-agent approach | January 2019 | June 1, 2019 | A multifunctional AIS will be developed based on a multi-agent approach. |
| 2.2 | Development of an architecture and description of the functions of cognitive agents for the synthesis of an intelligent control system. | July 2019 | November 1, 2019 | Architectures and descriptions of the functions of cognitive agents for the synthesis of ICS will be developed. The copyright certificate will be received. 1 publication will be published in a domestic scientific publication with a non-zero impact factor. |
| 3. | Development of software (SW) for the implementation of a Smart-technology for creating AIS for various applications of industrial automation systems, technological processes of the oil and gas industry and education. | January 2020 | November 1, 2020 | Software will be developed for the implementation of a Smart-technology for creating AIS for various applications of industrial automation systems, technological processes in the oil and gas industry and education. |
| 3.1 | Creation of a diagnostic system for industrial equipment based on modified AIS algorithms and AMDEC approaches using modern microprocessor technology. | January 2020 | June 1, 2020 | A diagnostic system for industrial equipment will be created based on AIS and AMDEC algorithms on modern microprocessor technology |
| 3.2 | Creation of mnemonic diagrams for complex objects control using the achievements of AI and cognitive technologies | July 2020 | November 1, 2020 | Mnemonic diagrams will be created for complex objects control using the achievements of AI and cognitive technologies. 1 publication will be published in a publication indexed in Scopus or Web of Sciences with a non-zero impact factor. Publications will be published in domestic and foreign scientific journals with a non-zero impact factor. The monograph will be published in a foreign publishing house. |

Calendar plan for 2018-2020 уу.

APPENDIX B

List of published works for 2020 уear

Foreign:

1 Samigulina G., Massimkanova Zh. Development of Modified Cooperative Particle Swarm Optimization with Inertia Weight for feature selection // Cogent Engineering. - 2020. - Vol. 7, Issue 1 // https://doi.org/10.1080/23311916.2020.1788876. (Thomson Reuters, Cite Score Scopus – 3,1; SJR Scopus – 0,323; quartile Q2; percentile 73%).

- Samigulina G.A., Samigulina Z.I., Lukmanova Zh.S. Cognitive Smart technology of distance learning of Experion PKS distributed control system for oil and gas industry using ontological approach // News of the Academy of sciences of the Republic of Kazakhstan. Series of Geology and Technical Sciences. – Алматы, 2020. - № 1(439). – P. 23-31. (Thomson Reuters, Cite Score Scopus – 0,8; SJR Scopus 0,209; quartile Q3; percentile 25%).

- Samigulina G., Samigulina Z. Design of technology for prediction and control system based on artificial immune systems and the multi-agent platform JADE // Chapter in book: Agent and Multi-Agent Systems: Technology and Applications. - Split, Croatia. January 2020. – P. 143-153 // DOI: 10.1007/978-981-15-5764-4_13. (Thomson Reuters, Cite Score Scopus – 0,9; SJR Scopus – 0,180; quartile Q3, percentile 28%).

- Samigulina G.А., Massimkanova Zh.A. Development of Smart-technology for Forecasting Technical State of Equipment based on Modified Particle Swarm Algorithms and Immune-Network Modeling. Chapter in book: Computational and Experimental Simulations in Engineering. Mechanisms and Machine Science. – Japan, 2020 // DOI: 10.1007/978-3-030-27053-7_26. (Cite Score Scopus – 0,8; SJR Scopus – 0.172; quartile Q4; percentile 25%).

- Samigulina G.A., Samigulina Z.I. Development of the industrial equipment diagnostics system based on modified algorithms of artificial immune systems and the AMDEC approach using Schneider Electric equipment // Proceeding of 2020 Intern. Conf. on Industrial Engineering, Applications and Manufacturing. – Sochi, Russia: IEEE, 2020 // DOI:10.1109/ICIEAM48468.2020.9111977. (Scopus).

- Samigulina G.A., Samigulina Z.I. Development of Smart-technologies for prediction and control of complex objects based on modified algorithms of artificial immune systems: Monograph. – Yelm, WA, USA: Science Book Publishing Ноuse, 2020. – 224 p.

- Samigulina G.A., Samigulina Z.I. Development of cognitive agents for a Smart control system // ITSEC: information technologies in science, education and control. - M, 2019 - №. 4(14). - P. 39-43. (IF RSCI - 0,966, in Russian).

- Samigulina G.A., Samigulina Z.I. Development of cognitive agents for a Smart-system for forecasting and managing complex objects // Mater. of XXVII All-Russian seminar "Neuroinformatics, its applications and data analysis". - Krasnoyarsk, 2019 - P.96-99. (not included in the report for 2019, in Russian).

- Samigulina G.A., Samigulina Z.I. Cognitive Smart Technology of Distance Learning for a Distributed Control System for the Oil and Gas Industry // Mater. of XI All-Russian scientific and technical conf. with int. participation "Robotics and Artificial Intelligence". - Zheleznogorsk, 2019. - P. 396-399. (in Russian).

- Shiryayeva O., Samigulin T. Аpplication of clonal selection algorithm for the PID-controller synthesis of MIMO-systems in oil and gas industry // Informatyka, Automatyka, Pomiary w Gospodarce i Ochronie Środowiska. – Lublin, 2019. – №3. – P. 50-53 // DOI:10.35784/iapgos.235.

- Shiryaeva O.I., Samigulin T.I. Multicriteria optimization of an intelligent control system based on modified algorithms // Bulletin of NTU "KhPI". - Kharkov, 2020. - No. 1(3). - P. 41-49. (IF RSCI - 0,129, in Russian).

Domestic:

- Samigulina G.A., Masimkanova Zh.A. Development of software in JADE for a multi-agent system based on a cooperative algorithm for a swarm of particles with a weight of inertia // AUEC Bulletin. - Almaty, 2020 - No. 1 (48). - P. 124-130. (IF KZ – 0,154, in Russian).

- Samigulina G.A., Samigulina Z.I. Diagnostic system for industrial equipment in the oil and gas industry based on modified algorithms of artificial immune systems and AMDEC approach // Mater. of scientific. conf. IICT MES RK "Modern problems of informatics and computing technologies". - Almaty, 2020. - P. 303-309. (in Russian).

- Samigulina G.A., Masimkanova Zh.A. Development of a knowledge base for a multi-agent system based on a cooperative particles swarm algorithm of with inertia weight // Mater. V Intern. scientific and practical conf. "Informatics and applied mathematics". – Almaty: IICT, 2020. – P. 106-112. (in Russian).

- Shiryaeva O.I., Samigulin T.I. Basic concepts of the development of an artificial immune system for a class of complex systems // Bulletin of KazNRTU. - 2019. - No. 5. – P. 501-504. (IF KZ - 0.045, not included in the report for 2019, in Russian).

- Shiryaeva O.I., Samigulin T.I. Development of a Smart-control system for a complex technological process in the oil and gas industry using bioinspired algorithms // Bulletin of KBTU. - 2019. - T. 16, Issue. 4. - P. 164-171. (IF KZ - 0.047, not included in the report for 2019, in Russian).

- Shiryaeva O.I., Samigulin T.I. Development of software for the implementation of negative selection technology for creating an intelligent control system for the oil and gas industry // Mater. of conf. "Satpayev readings -2020". - Almaty: KazNRTU named after K.I. Satpayev, 2020. - P. 245-248.

- Shiryaeva O. I. Synthesis of the oil and gas industry control system based on artificial immune systems and negative selection algorithm // Mater. V Intern. scientific and practical conf. "Informatics and applied mathematics". – Almaty: IICT, 2020. – P. 100-105. (in Russian).

- Shiryaeva O.I. Development of software for the implementation of Smart - technologies for creating a control system for technological processes in the oil and gas industry // Mater. of scientific. conf. IICT MES RK "Modern problems of informatics and computing technologies". - Almaty, 2020. - P. 325-330. (in Russian).

- A. a. №11354. Implementation of a Smart-technology for creating a control system for technological processes in the oil and gas industry / O.I. Shiryaeva, T.I. Samigulin; publ. 07/14/2020.

List of published works for 2019 уear

21 Samigulina G.A., Samigulina Z.I. Development of Smart technology for Complex Objects prediction and Control on the Basis of a Distributed Control System and an Artificial Immune Systems Approach // Advances in Sciences, Technology and Engineering Systems Journal. – 2019. – Vol. 4, № 3. – P.75-87. (Cite Score Scopus – 0,6; SJR Scopus – 0,14; quartile Q3, percentile 22%).

22 Samigulina G.A., Samigulina Z.I. Development of Smart-technology for Complex Objects Control based on Approach of Artificial Immune Systems // Materials of 2018 Global Smart Industry Conference (GloSIC). – Chelyabinsk, 2018 // DOI:10.1109/glosic.2018.8570142:14.05.2019. (Scopus, not included in the report for 2018).

23 Samigulina G.A., Massimkanova Zh.A. Development of Smart-technology for Forecasting Technical State of Equipment based on Modified Particle Swarm Algorithms and Immune-Network Modeling // Abstracts of The International Conference on Computational and Experimental Engineering and Sciences. – Tokyo, 2019. – P. 68-69.

24 Samigulina G.A., Lukmanova Zh.S. Cognitive Smart-technology of distance learning for modern industrial automation equipment // Bulletin of the National Technical University "Kharkov Polytechnic Institute". - Kharkiv, 2018. - No. 42 (1318). - P. 160-170. (IF RSCI – 0,129, not included in the report for 2018, in Russian).

25 Samigulina G.A., Masimkanova Zh.A. Development of software for a cooperative algorithm for a swarm of particles with a weight of inertia for a multifunctional artificial immune system // Collection of articles for XLIX Intern. scientific. readings (in memory of V.V.Petrov). - M., 2019 - P. 19-22. (in Russian).

26 Samigulina G.A., Aldibekova A.N. Artificial immune systems: modified algorithms for clonal selection // Collection of articles of international. scientific-practical conf. for Science-intensive technologies and intelligent systems. - Samara, 2018 - P. 68-70. (not included in the report for 2018, in Russian).

27 Samigulin T.I., Samigulina G.A. Development of software for complex object control based on the ant colony algorithm // Mater. of IX All-Russian scientific and technical conf. with int. participation "Robotics and Artificial Intelligence". - Zheleznogorsk, 2018 - P. 119-123. (not included in the report for 2018, in Russian).

28 Samigulina G.A., Masimkanova Zh.A. On the construction of an optimal set of descriptors based on a modified cooperative algorithm for a swarm of particles with the weight of inertia // Abstracts of the 9th International scientific and technical conf. "PIM-2019". - Kharkov-Odessa, 2019 - P. 16. (in Russian).

29 Masimkanova Zh.A. Samigulina G.A. Development of a multi-agent prediction system based on a cooperative algorithm for a swarm of particles and artificial immune systems // XI Annual Intern. conf. of young scientists "Modern techniques and technologies in scientific research." - Bishkek, 2019 - P. 113-115. (in Russian).

30 Shiryaeva O.I., Samigulin T.I. Development of an artificial immune control system for a multidimensional object of the oil and gas industry // Bulletin of the National Technical University "Kharkov Polytechnic Institute". - Kharkiv, 2019. - No. 13 (1338). - P. 155-163. (IF RSCI – 0,129, in Russian).

31 Shiryayeva O. Features of the Smart-technologies application to the synthesis of MIMO-systems in oil and gas industry // Materials of the 17th International scientific conference «Information technologies and management 2019». – Riga, 2019. – P. 46-47.

32 Samigulina G.A., Masimkanova Zh.A. Multifunctional multi-agent Smart-system based on modified algorithms of swarm intelligence // Bulletin of KBTU. –2019. - T.16, Issue. 2. - P. 157-164. (IF KZ – 0,047, in Russian).

33 Samigulina G.A., Masimkanova Zh.A. Development of software based on a cooperative particle swarm algorithm for a multi-agent Smart-system. Bulletin of KazNRTU named after K.I. Satpayev. - 2019. - No. 3 (133). - P. 605-610. (IF KZ – 0,045, in Russian).

34 Samigulina G.A., Samigulin T.I. Development of a Smart - technology for complex object control using the ant colony algorithm // Bulletin of the Almaty University of Energy and Communications. - 2019. - No. 1(44). - P. 98-105. (IF KZ – 0,154, in Russian).

35 Samigulina G.A., Aldibekova A.N. Klondy íríkteu negízínde zhasandy immundy zhүyyelerdíң zhetíldírílgen algoritmderíníң zamanaui zhaġdayy // Science News of Kazakhstan. - 2019. - No. 2. - P. 64-71. (IF KZ – 0,127, in Kazakh).

36 Samigulina G.A., Samigulina Z.I. Modified optimization Grey wolves algorithm for Smart-technology for prediction and complex objects control // Mater. of scientific. conf. IICT "Modern problems of informatics and computing technologies". - Almaty, 2019 - P. 280-286. (in Russian).

37 Samigulina G.A., Masimkanova Zh.A. Development of a modified cooperative algorithm for a swarm of particles with a weight of inertia for multifunctional artificial immune systems // Mater. of scientific. conf. IICT "Modern problems of informatics and computing technologies". - Almaty, 2019 - P. 286-291. (in Russian).

38 Shiryaeva O.I., Samigulin T.I. Analysis of the modeling results of the processes of the oil and gas industry based on Smart-technologies // Proceedings of the Satpayev Readings "Innovative technologies - the key to successfully solving fundamental and applied problems in the ore and oil and gas sectors of the economy of the Republic of Kazakhstan." - Almaty, 2019 - P. 270-274. (in Russian).

39 Shiryaeva O.I., Samigulin T.I. Comparative analysis of the adjustment of the regulators of the control system of the gas distillation process through the distillation column based on Smart-technologies // Mater. of scientific. conf. IICT KN MES RK "Innovative IT and Smart-technologies", dedicated to the 70th anniversary of prof. Utepbergenova I.T. - Almaty, 2019 - P. 270-273. (in Russian).

40 Shiryaeva O.I., Samigulin T.I. Modeling and decoupling of a complex system with optimal CLONALG-controllers // Mater. of int. scientific-practical conf. "Computer Science and Applied Mathematics". - Almaty: IICT, 2019 - p. 526-533. (in Russian).

41 Samigulina G.A., Samigulina Z.I. Development of a multifunctional artificial immune system based on cognitive agents and a model-based approach for a Smart-technology for prediction and complex objects control of the oil and gas industry // Mater. of int. scientific-practical conf. "Computer Science and Applied Mathematics". - Almaty: IICT, 2019. - P. 470-476. (in Russian).

42 Samigulina G.A., Masimkanova Zh.A. Development and description of functions of cognitive agents for a modified cooperative algorithm for a swarm of particles with a weight of inertia // Mater. of int. scientific-practical conf. "Computer Science and Applied Mathematics". - Almaty: IIVT, 2019. - P. 465-470. (in Russian).

43 А.a. № 2313. MCPSO (Multi-agent Cooperative Particle Swarm Optimization) / G.A. Samigulina, Zh.A. Masimkanova; publ. 03/15/2019. (in Russian).

44 А.a. № 836. АССО (Ant Colony for Complex Objects) / G.A. Samigulina, T.I. Samigulin; publ. 06.12.2018. (not included in the report for 2018, in Russian).

List of published works for 2018 уear

45 Samigulina G.A., Samigulina Z.I. Modified immune network algorithm based on the Random Forest approach for the complex objects control // Artificial Intelligence Review. – Springer, 2018. – P. 1-17. (Electronic version in 2018, printed version in 2019, №52(4), P. 2457-2473; IF WoS - 5.747, Cite Score Scopus – 9,1; SJR – 1,228; quartile Q1; percentile 89%).

46 Samigulina G.А., Massimkanova Zh.A. Multi-agent System for Forecasting Based on Modiﬁed Algorithms of Swarm Intelligence and Immune Network Modeling // Proceeding. of the 12th Intern. Conf. Agents and Multi-agent Systems: Technologies and Applications (KES-AMSTA-18). – Australia: Springer, 2018. – P. 199-208. (Thomson Reuters, Cite Score Scopus – 0,9; SJR Scopus 0,18; quartile Q3; percentile 28%).

47 Samigulina G.A., Shayakhmetova A.S., Nyusupov A.T. Innovative intelligent technology of distance learning for visually impaired people // Open engineering. – De Gruyter, 2017. – Vol. 7, Issue 1. – P. 444-452. (Cite Score Scopus – 1.2; SJR Scopus – 0,22; quartile Q3; percentile 35%; not included in report for 2017).

48 Samigulina G.A., Nyusupov A.T., Shayakhmetova A.S. Analytical review of software for multi-agent systems and their applications // News of the Academy of sciences of the Republic of Kazakhstan. Series of Geology and Technical Sciences. – Almaty, 2018. - №3 (429). – P. 173-181. (Thompson Reuters, Cite Score Scopus – 0,1; SJR Scopus 0,209; quartile Q4).

49 Samigulina G.A., Samigulina Z.I. Cognitive Smart-technology of immune network modeling based on ontological and multi-agent approaches // Mater. of XIV int. scientific-practical conf. "Days of Science". - Prague, 2018. - Issue. 6. - P. 58-60. (in Russian).

50 Samigulina G.A., Samigulina Z.I. Development of a multifunctional artificial immune system based on modified algorithms and a multi-agent approach // Materials of the XIII Intern. Scientific and Practical conf. "Science without borders-2018". - UK: Sheffield science and education LTD, 2018. - Vol. 10. - P. 60-62. (in Russian).

51 Samigulina G.A., Samigulin T.I. Review of modern approaches of artificial intelligence for control systems of complex objects // Problems of Informatics. - 2018. - No. 3 (40). - P. 4-20. (IF RSCI – 0,182, in Russian).

52 Samigulina G.A., Shayakhmetova A.S., Nyusupov A.T. OWL-model of a multi-agent Smart- system of distance learning for people with visual disabilities // Open Education. - M., 2017. - No. 6. - P. 49-56. (IF RSCI – 0,601, not included in the report for 2017, in Russian).

53 Samigulina Z.I., Aldemkhan Zh.M. Prospects for the use of industrial equipment from Schneider Electric to control complex objects // Mater. of X int. scientific-practical conference "Topical issues of modern science". - Tomsk, 2018. - Part 1(4). - P. 152-156. (in Russian).

54 Samigulina GA, Lukpanova Zh. Distance learning for visually impaired people based on cognitive SMART technology // Mater. of XXV All-Russian seminar "Neuroinformatics, its applications and data analysis". - Krasnoyarsk, 2018. - P. 112-116. (in Russian).

55 Samigulina G.A., Masimkanova Zh.A. Construction of an optimal data set based on modified algorithms of ant colonies and an ontological approach for immune network modeling // News of Science of Kazakhstan. - Almaty, 2018. - No. 3 (137). - P. 12-20. (IF KZ - 0,127, in Russian).

56 Samigulina ZI, Omirgaliev R. Development of software for industrial automation systems of complex objects based on microprocessor technology // Bulletin of KBTU. - Almaty, 2018 - T. 15, Issue 1. - P. 114-119. (IF KZ – 0.047, in Russian).

57 Samigulina G.A., Samigulina Z.I. Development of a multi-agent multifunctional Smart-system for complex objects control based on artificial immune systems // Bulletin of the National Engineering Academy of the Republic of Kazakhstan. - Almaty, 2018. - No. 3 (69). - P. 21-25. (IF KZ – 0,142, in Russian).

58 Samigulina G.A., Lukpanova J. Development of cognitive personalized smart technology for distance learning for people with impaired vision in engineering specialties // Bulletin of KazNRTU. - Almaty, 2018. - No. 5(129). - P. 48-53. (IF KZ – 0,045, in Russian).

59 Samigulina G.A., Samigulina Z.I. Development of innovative cognitive Smart-technology of distance learning for modern industrial equipment in the framework of the concept of digitalization of the industry of Kazakhstan // Mater. of round table "Digitalization of industry - the basis of the fourth industrial revolution." - Almaty, 2018 - P. 88-91. (in Russian).

60 Samigulina GA, Lukpanova Zh. Cognitive Smart Technology for Personalized Distance Learning of Visually Impaired People in Engineering Specialties in the Oil and Gas Industry // Mater. of scientific. conf. IIVT "Modern problems of informatics and information technology". - Almaty: IICT, 2018. - P. 244-248. (in Russian).

61 Samigulina G.A., Masimkanova Zh.A. Diagnostics of industrial equipment based on a modified particle swarm algorithm and the approach of artificial immune systems // Mater. of scientific. conf. IICT "Modern problems of informatics and information technology". - Almaty: IIVT, 2018. - P. 248-253. (in Russian).

62 Samigulina Z.I., Aldibekova A.N. Intelligent complex control system
objects based on the clonal selection algorithm // Mater. of scientific. conf. IICT "Modern problems of informatics and information technology". - Almaty: IIVT, 2018. - P. 253-258. (in Russian).

63 Samigulina G.A., Masimkanova Zh.A. Prediction of the technical state of industrial equipment based on modified algorithms of a swarm of particles and immune network modeling // Mater. of int. scientific-practical conf. "Computer Science and Applied Mathematics". - Almaty: IICT, 2018. - Part 1. - P. 296-300. (in Russian).

APPENDIX C

Publication reprints for 2020 у.

1 Samigulina G., Massimkanova Zh. Development of Modified Cooperative Particle Swarm Optimization with Inertia Weight for feature selection // Cogent Engineering. - 2020. - Vol. 7, Issue 1 // https://doi.org/10.1080/23311916.2020.1788876. (Thomson Reuters, Cite Score Scopus – 3,1; SJR Scopus – 0,323; quartile Q2; percentile 73%).

- Samigulina G.A., Samigulina Z.I., Lukmanova Zh.S. Cognitive Smart technology of distance learning of Experion PKS distributed control system for oil and gas industry using ontological approach // News of the Academy of sciences of the Republic of Kazakhstan. Series of Geology and Technical Sciences. – Алматы, 2020. - № 1(439). – P. 23-31. (Thomson Reuters, Cite Score Scopus – 0,8; SJR Scopus 0,209; quartile Q3; percentile 25%).

- Samigulina G., Samigulina Z. Design of technology for prediction and control system based on artificial immune systems and the multi-agent platform JADE // Chapter in book: Agent and Multi-Agent Systems: Technology and Applications. - Split, Croatia. January 2020. – P. 143-153 // DOI: 10.1007/978-981-15-5764-4_13. (Thomson Reuters, Cite Score Scopus – 0,9; SJR Scopus – 0,180; quartile Q3, percentile 28%).

- Samigulina G.А., Massimkanova Zh.A. Development of Smart-technology for Forecasting Technical State of Equipment based on Modified Particle Swarm Algorithms and Immune-Network Modeling. Chapter in book: Computational and Experimental Simulations in Engineering. Mechanisms and Machine Science. – Japan, 2020 // DOI: 10.1007/978-3-030-27053-7_26. (Cite Score Scopus – 0,8; SJR Scopus – 0.172; quartile Q4; percentile 25%).

- Samigulina G.A., Samigulina Z.I. Development of the industrial equipment diagnostics system based on modified algorithms of artificial immune systems and the AMDEC approach using Schneider Electric equipment // Proceeding of 2020 Intern. Conf. on Industrial Engineering, Applications and Manufacturing. – Sochi, Russia: IEEE, 2020 // DOI:10.1109/ICIEAM48468.2020.9111977. (Scopus).

- Samigulina G.A., Samigulina Z.I. Development of Smart-technologies for prediction and control of complex objects based on modified algorithms of artificial immune systems: Monograph. – Yelm, WA, USA: Science Book Publishing Ноuse, 2020. – 224 p.

- Samigulina G.A., Samigulina Z.I. Development of cognitive agents for a Smart control system // ITSEC: information technologies in science, education and control. - M, 2019 - №. 4(14). - P. 39-43. (IF RSCI - 0,966, in Russian).

- Samigulina G.A., Samigulina Z.I. Development of cognitive agents for a Smart-system for forecasting and managing complex objects // Mater. of XXVII All-Russian seminar "Neuroinformatics, its applications and data analysis". - Krasnoyarsk, 2019 - P.96-99. (not included in the report for 2019, in Russian).

- Samigulina G.A., Samigulina Z.I. Cognitive Smart Technology of Distance Learning for a Distributed Control System for the Oil and Gas Industry // Mater. of XI All-Russian scientific and technical conf. with int. participation "Robotics and Artificial Intelligence". - Zheleznogorsk, 2019. - P. 396-399. (in Russian).

- Shiryayeva O., Samigulin T. Аpplication of clonal selection algorithm for the PID-controller synthesis of MIMO-systems in oil and gas industry // Informatyka, Automatyka, Pomiary w Gospodarce i Ochronie Środowiska. – Lublin, 2019. – №3. – P. 50-53 // DOI:10.35784/iapgos.235.

- Shiryaeva O.I., Samigulin T.I. Multicriteria optimization of an intelligent control system based on modified algorithms // Bulletin of NTU "KhPI". - Kharkov, 2020. - No. 1(3). - P. 41-49. (IF RSCI - 0,129, in Russian).

12 Samigulina G.A., Masimkanova Zh.A. Development of software in JADE for a multi-agent system based on a cooperative algorithm for a swarm of particles with a weight of inertia // AUEC Bulletin. - Almaty, 2020 - No. 1(48). - P. 124-130. (IF KZ – 0,154, in Russian).

- Samigulina G.A., Samigulina Z.I. Diagnostic system for industrial equipment in the oil and gas industry based on modified algorithms of artificial immune systems and AMDEC approach // Mater. of scientific. conf. IICT MES RK "Modern problems of informatics and computing technologies". - Almaty, 2020. - P. 303-309. (in Russian).

- Samigulina G.A., Masimkanova Zh.A. Development of a knowledge base for a multi-agent system based on a cooperative particles swarm algorithm of with inertia weight // Mater. V Intern. scientific and practical conf. "Informatics and applied mathematics". – Almaty: IICT, 2020. – P. 106-112. (in Russian).

15 Shiryaeva O.I., Samigulin T.I. Basic concepts of the development of an artificial immune system for a class of complex systems // Bulletin of KazNRTU. - 2019. - No. 5. – P. 501-504. (IF KZ - 0.045, not included in the report for 2019, in Russian).

- Shiryaeva O.I., Samigulin T.I. Development of a Smart-control system for a complex technological process in the oil and gas industry using bioinspired algorithms // Bulletin of KBTU. - 2019. - T. 16, Issue. 4. - P. 164-171. (IF KZ - 0.047, not included in the report for 2019, in Russian).

- Shiryaeva O.I., Samigulin T.I. Development of software for the implementation of negative selection technology for creating an intelligent control system for the oil and gas industry // Mater. of conf. "Satpayev readings -2020". - Almaty: KazNRTU named after K.I. Satpayev, 2020. - P. 245-248.

- Shiryaeva O. I. Synthesis of the oil and gas industry control system based on artificial immune systems and negative selection algorithm // Mater. V Intern. scientific and practical conf. "Informatics and applied mathematics". – Almaty: IICT, 2020. – P. 100-105. (in Russian).

- Shiryaeva O.I. Development of software for the implementation of Smart - technologies for creating a control system for technological processes in the oil and gas industry // Mater. of scientific. conf. IICT MES RK "Modern problems of informatics and computing technologies". - Almaty, 2020. - P. 325-330. (in Russian).

- A. a. №11354. Implementation of a Smart-technology for creating a control system for technological processes in the oil and gas industry / O.I. Shiryaeva, T.I. Samigulin; publ. 07/14/2020.

APPENDIX D

List of used foreign information resources

In scientific research on this project, the following databases were used:

- Springer.com;

- Elsevier.com;

- Clarivate.com;

- eLIBRARY.ru;

- Scopus.com.

APPENDIX E

U700 Installation, equipment specification

Table E.1 – Description of U700 units

| Equipment | Function |
| --- | --- |
| R720/721/725 | gas dehydration reactors |
| D700 | demethanization column |
| D701 | deethanization column |
| D703 | depropanization column |
| D704 | debutanizing column |
| GC-701 | commercial gas compressor, etc. |
| TRC-70005 | D700 top temperature sensor |
| LICAHL-70001/LICAHL-70002 | D700 column level sensor |
| PRCHL-70001 | D700 column pressure sensor |
| TI70022/TI-70025 | D701 column pressure sensor |
| LICAHL-70010/LICAHL-7009 | D700 column pressure sensor |
| PIC-70007 | D701 column pressure sensor |
| TI-700432 | D703 column temperature sensor |
| LAH-700421/LAL-700421 | D703 column level sensor |
| PAH-700422/PAL-700422 | D703 column pressure sensor |
| TRC-70034 | D704 column temperature sensor |
| LAH-700424/LAL-700424 | D704 column level sensor |
| PAH-700441 | D704 column pressure sensor |

APPENDIX F

Specification of U700 valves

Table F.1 - Fragment sheets of U700 valves

| Valve installation place | № of valve | Manufacturer | Notes |
| --- | --- | --- | --- |
| D700 | PSV-700077A PSV-700077B | JBS-34-B | 72 bar |
| D701 | PSV-700075A PSV-700075B | Crosby JBS35BSP | 26 bar |
| D703 | PSV-700073A PSV-700073B | Crosby JBS-35-OR-K-SP | 20 bar |
| R720 | PSV-720002A PSV-720002B | SAPAG Y 8103F ALCBA | 73 bar |
| D704 | PV-70012 PDV-70013 | EWT Globe-21114 | normally open normally closed |
| GC-701 | PV-70848A PV-70948A | REGLOB 2" REGLOB 2" | normally closed normally closed |
| R720/721/725 | FV-70061 | REGLOB 6" | normally closed |
| … | … | … | … |

APPENDIX G

Failures and ways of their elimination on U700

Table G.1 – Search for cause of the problem and ways to resolve them

| Deviations from technological process | Causes of occurrence | Way of their elimination |
| --- | --- | --- |
| High content of mercaptans in commercial gas | High temperature or low pressure in columns D-700 and D-701. Insufficient reflux supply to column D-701. Malfunction of the control circuits of the columns D-700 and D-701. | 1. Bring in accordance with the technological map. 2. Provide the required reflux flow rate. 3. Check the control circuits. |
| GC-701 Compressor stop | Extremely high temperature at the input. | If one signal is received, check the devices. When 2, 3 signals arrive, take action. |
| High / low level on the plates in D-700 (LICAHL-70001; LICAHL-70002) | Failure of the control circuit or G-701. Hydrates in lines or on plates. Valves LV-70001 are closed or LV-70002 at low level. | 1.Check the control circuits and valve positions. 2. Check the operation of the pumps, if necessary, turn on the backup. 3. Add methanol. 4. Check the supply of desulfurized gas to the installation. |
| … | … | … |

APPENDIX K

An example of equipment diagnostics based on AMDEC approach

Table K.1 – Diagnostics of the condition of the U700 equipment based on AMDEC

APPENDIX L

Measurement range of U700 sensors

Table L.1 - Fragment of the DB of the permissible range of technological parameters of U700

| Unit ID | Sensor 1 | Sensor 2 | Sensor 3 | … | Sensor N |
| --- | --- | --- | --- | --- | --- |
| R-720/721/ 722 | FRC-20002 | PDT-720007 | PDT-720016 | … | TIAHL-720003 |
|  | 12861 nm3/h | 7 bar | 7 bar | … | 0-400 Со |
| B-720 | PICAHL-720011 | TICAH-20001 | TAHH-720020 | … | TIAH-720023 |
|  | 2,5 bar | 0-300 Со | 0-500 Со | … | 0-500 Со |
| D-701 | FI-700009 | FIC-700089 | LICAHL-00010 | … | TRCHL-700024 |
|  | 25.1-103.1 m3/h | 90 m3/h | 0-100 % | … | 0-200 Со |
| GC-701 | TRAH-740701/3 | TRAH-7405 | RAH-740509/11 | … | SAH-740521 |
|  | 75-90 Со | 0,3-0,4 mm | 110-125 Со |  | 7-10 g |
| … | … | … | … | … | … |

APPENDIX M

Description of U700 parameters

Table M.1 - U700 parameter specification

| Sensor | Function | Notes |
| --- | --- | --- |
| 2 | 3 | 4 |
| FRC-20002 | regeneration gas consumption | gas flow control |
| PDT-720007 | differential pressure R721 | alarm |
| PDT-720016 | differential pressure R720 | alarm |
| PDT-720031 | differential pressure 722 | alarm |
| TIAHL-720002 | input temperature R720 | alarm |
| TIAHL-720003 | input temperature R721 | alarm |
| PICAHL-720011 | fuel gas pressure at the input | pressure regulation |
| TICAH-720001 | gas temperature at the output | regulation |
| TAHH-720020 | flue gas temperature | alarm |
| TAHH-720021 | pipe wall temperature | alarm |
| TIAH-720022 | pipe wall temperature | alarm |
| TIAH-720023 | pipe wall temperature | alarm |
| FI-700009 | bottom product consumption | indication |
| FIC-700089 | reflux rate | consumption regulation |
| LICAHL-700010 | liquid level in a cube | level regulation |
| PDI-700008 | pressure drop across on plates | indication |
| PIAHH-700062 | gas pressure at the output | temperature regulation |
| TI-700022 | temperature on the plate | temperature regulation |
| TRCHL-700024 | temperature on the plate | temperature regulation |
| TRAH-740701/3 | temperature of the shaft support bearings | compressor reducer |
| TRAH-7405-08 | axial bearing temperature | compressor reducer |
| GAH-740505/6 | axial shaft displacement | compressor reducer |
| RAH-740509/11 | pinion shaft bearing temperature | compressor reducer |
| SXAH-740511 | pinion shaft vibration | compressor reducer |
| SXAH-740515 | radial vibration of the gearbox drive shaft | compressor reducer |
| SAH-740521 | body vibration | compressor reducer |

APPENDIX N

Database of readings from U700 sensors

Table N.1 – Fragment of a database of readings from U700 sensors

Figure N.1 - Visualization of DB readings from sensors

APPENDIX P

Listing of the GA subprogram for searching the initial population

for the AIS-CLONALG subprogram

IntCon = 1;

rng default % For reproducibility

fun = @shiryayeva;

A = [-1 0; 0 -1];

b = [0; 0];

x = ga(fun,2,A,b)

% first circuit quality criterion ISE1

function z=shiryayeva(xx)

z=((1+0.0042.*xx(:,1)).^2.+0.0042.*xx(:,2).*1.9588+(0.0042.*xx(:,1)).^2)./(2.*0.0042.*xx(:,2).*(1+0.0042.*xx(:,1)))-(xx(:,1)./xx(:,2));

end

% second circuit quality criterion ISE2

function z=shiryayeva(xx)

z=((1+0.0072.*xx(:,1)).^2.+0.0072.*xx(:,2).*1.9588+(0.0072.*xx(:,1)).^2)./(2.*0.0072.*xx(:,2).*(1+0.0072.*xx(:,1)))-(xx(:,1)./xx(:,2));

end

The calculation results of the GA algorithm for two control circuits are shown on Figure N.1.

Figure P.1 – Results of calculations of the GA algorithm for two control circuits

APPENDIX R

Listing of the AIS-NSA subprogram for determination

of the local minima of quality criteria

% Initial condition

N    = 100;                  % Population size

Ab   = cadeia(N,44);         % antibodies population

gen  = 50;                   % number of generations

pm   = 0.5;                  % mutation probability

d    = 0.3;                  % population that undergoes random permutations

beta = 0.5;                  % multiplier controlling the number of clones

% N	   -> number of clones

% beta -> multiplying factor

% ind  -> best individuals

% Ab   -> antibody population

% T   -> temporary population

% pcs  -> final position of each clone

% pma  -> initial value

% pmr  -> control rate

% itpm -> iterations for restoring

% AIS-CLONALG subprogram for determining the parameters of regulators

% Function to optimization ISE1

f =@shiryayeva1

varMin = -200; % lower population limit for the first circuit (P1) for NSA

varMax = 250; % upper population limit for the first circuit (P1) for NSA

varMiny = -200;% lower population limit for the first circuit (I1) for NSA

varMaxy = 250; % upper population limit for the first circuit (I1) for NSA

fbest = 0; % Global f best (minimum) by NSA

% Function to optimization ISE2

f =@shiryayeva2

varMin = -200; % lower population limit for the first circuit (P2) for NSA

varMax = 250; % upper population limit for the first circuit (P2) for NSA

varMiny = -200;% lower population limit for the first circuit (I1) for NSA

varMaxy = 250; % upper population limit for the first circuit (I1) for NSA

fbest = 0; % Global f best (minimum) by NSA

APPENDIX S

Listing of the AIS-CLONALG subprogram for determination

of the parameters of the regulators

% Initial condition

N    = 100;                  % Population size

Ab   = cadeia(N,44);         % antibodies population

gen  = 50;                   % number of generations

pm   = 0.5;                  % mutation probability

d    = 0.3;                  % population that undergoes random permutations

beta = 0.5;                  % multiplier controlling the number of clones

% Function to optimization ISE1

f =@shiryayeva1

varMin = 37.4554; % initial population, P1, obtained at GA for the first circuit

varMax = 120; % upper limit, P1, obtained by NSA for the first circuit

varMiny = 73.2435;% initial population, I1, obtained at GA for the first circuit

varMaxy = 74;% upper limit, I1, obtained by NSA for the first circuit

fbest = 0;

% Function to optimization ISE2

f =@shiryayeva2

varMin = 37.4554; % initial population, P2, obtained at GA for the first circuit

varMax = 120; % upper limit, P2, obtained by NSA for the first circuit

varMiny = 73.2435;% initial population, I2, obtained at GA for the first circuit

varMaxy = 74;% upper limit, I2, obtained by NSA for the first circuit

fbest = 0;

x = meshgrid(linspace(varMin, varMax, 61));

y = meshgrid(linspace(varMiny, varMaxy, 61))';

vxp = x;

vyp = y;

vzp = f([x(:),y(:)])

vzp = reshape(vzp,size(x));

x = decode(Ab(:,1:22),varMin,varMax)

y = decode(Ab(:,23:end),varMiny,varMaxy)

fit = f([x(:),y(:)]);

figure

imprime(1,vxp,vyp,vzp,x,y,fit)

pma  = pm;

itpm = gen;

pmr  = 0.8;

vfx   = zeros(gen,1);

PRINT = 1;

for it = 1:gen

x = decode(Ab(:,1:22),varMin,varMax);

y = decode(Ab(:,23:end),varMiny,varMaxy);

fit = f([x(:),y(:)]);

[a,ind] = sort(fit);

valx = x(ind);

valy = y(ind);

imprime(PRINT,vxp,vyp,vzp,x,y,fit);

[T,pcs] = reprod(N,beta,ind,Ab);

M = rand(size(T,1),44) <= pm;

T = T - 2 .* (T.*M) + M;

T(pcs,:) = Ab(fliplr(ind),:);

x = decode(T(:,1:22),varMin,varMax);

y = decode(T(:,23:end),varMiny,varMaxy);

fit = f([x(:),y(:)]);

pcs = [0 pcs];

for i = 1:N

[~,bcs(i)] = min(fit(pcs(i)+1:pcs(i+1)));

bcs(i) = bcs(i) + pcs(i);

end

Ab(fliplr(ind),:) = T(bcs,:);

nedit = round(d*N);

Ab(ind(end-(nedit-1):end),:) = cadeia(nedit,44);

pm = pmcont(pm,pma,pmr,it,itpm);

vfx(it) = a(1);

end

x  = valx(1)

y  = valy(1)

fx = vfx(end)

figure

semilogy(vfx)

title('Minimization')

xlabel('Iterations')

ylabel('Best f(x,y)')

grid on

txt2 = ['F Best: ', num2str(fbest)];

text(0,1,txt2,'Units','normalized',...

'HorizontalAlignment','left','VerticalAlignment','bottom');

txt3 = ['F Found: ', num2str(fx)];

text(1,1,txt3,'Units','normalized',...

'HorizontalAlignment','right','VerticalAlignment','bottom');

function imprime(PRINT,vx,vy,vz,x,y,fx)

if PRINT == 1

meshc(vx,vy,vz)

hold on

title('Minimization')

xlabel('x')

ylabel('y')

zlabel('f(x,y)')

plot3(x,y,fx,'k*')

colormap jet

drawnow

hold off

pause(0.1)

end

end

function [T,pcs] = reprod(N,beta,ind,Ab)

T = [];

for i = 1:N

cs(i) = round(beta*N);

pcs(i) = sum(cs);

T = [T; ones(cs(i),1) * Ab(ind(end-i+1),:)];

end

end

function pm = pmcont(pm,pma,pmr,it,itpm)

if rem(it,itpm) == 0

pm = pm * pmr;

if rem(it,10*itpm) == 0

pm = pma;

end

end

end

function z = decode(Ab,varMin,varMax)

Ab = fliplr(Ab);

s = size(Ab);

aux = 0:1:21;

aux = ones(s(1),1)*aux;

x1 = sum((Ab.*2.^aux),2);

z = varMin + x1' .* (varMax - varMin)/(2^22 - 1);

end

function Ab = cadeia(n1,s1)

Ab = 2 .* rand(n1,s1) - 1;

Ab = hardlim(Ab);

end

% first circuit ISE1

function z=shiryayeva1(xx)

z=(((1+0.0042.* xx(:,1)).^2.+0.0042.* xx(:,2).*1.9588+(0.0042.* xx(:,1)).^2)./(2.*0.0072.* xx(:,2).*(1+0.0042.* xx(:,1)))- xx(:,1)./ xx(:,2));

end

% second circuit ISE2

function z=shiryayeva2(xx)

z=(((1+0.0072.* xx(:,1)).^2.+0.0072.* xx(:,2).*1.9588+(0.0072.* xx(:,1)).^2)./(2.*0.0072.* xx(:,2).*(1+0.0072.* xx(:,1)))- xx(:,1)./ xx(:,2));

end

APPENDIX T

Control system modeling schemes in MATLAB environment

APPENDIX U

Certificatas

APPENDIX W

Acts of implementation

APPENDIX X

Copyright certificates