Investigators list

ABSTRACT

##### Report 189 р., 36 fig., 14 tab., 42 ref., 11 apps.

Wool, sheep, shearing station, artificial insemination station, transportation, wool storage, containers, customer specification, design documentation

Research object is the technology, equipment and installation mobile artificial insemination station (MAIS), a mobile shearing station (MSS), wool storage equipment (WSE).

Research purpose is creating a technology and system of machines for high-quality wool production qualifying modern requirements in the distant sheep farming conditions.

Research results: experimental samples MAIS, MSS, WSE; test and implementation results confirmed by acts; approved technical specifications and sets of design documentation for the design of prototypes; published recommendations, scientific articles (13), of which 2 with impact factor, 8 in journals recommended by the SC MES RK; 4 patents for inventions of the Republic of Kazakhstan, 4 copyright certificates for intellectual property; 3 reports at international conferences.

Research novelty: structural and technological schemes, technical solutions and parameters of MAIS, MSS, WSE; new methodology for insemination, shearing, transport and storage of bales, contributing to the cost-effective operation of the industry, including primary wool processing factories (WPF). The effect is achieved: by carrying out the insemination process directly in the flocks; minimization of the spread of the grade of wool in bales, by grading and dividing animals into groups immediately before shearing; the use of container storage technology, delivery of wool bales from MSS to the factory and their subsequent storage at the factory in containers without intermediate reloading of the bales, which, while maintaining the integrity of the bales, practically eliminates wool loss, and reduces the labor intensity of loading and unloading operations. New shearing machine design, use of a heat pump and microprocessors for container wool storage technology.

Methodological base: review and analysis of literature and patents, a systematic approach to the formation of a vertically integrated production technology, harvesting and storage of wool, analysis of system elements and their relationship within a specific organizational structure.

Scope - farms, private subsidiary farm, wool primary processing factories, working in the field of wool production and procurement.

Content

|  | p. |
| --- | --- |
| DEFINITIONS, DESIGNATIONS AND ABBREVIATIONS |  |
| Introduction | 1 |
| THE MAIN PART OF THE RESEARCH REPORT | 3 |
| 1 Analysis of the current situation | 3 |
| 1.1 State of the industry | 3 |
| 1.2 Substantiation of Research Direction | 5 |
| 2 Research program and methodology | 8 |
| 2.1 Research Agenda 2018-2020 | 8 |
| 2.2 Methodology for performing R&D | 8 |
| 3 Research results | 11 |
| 3.1 Research and development of a mobile sheep artificial insemination station | 11 |
| 3.1.1 Literature review, justification of technology and technical solution | 11 |
| 3.1.2 Constructive and technological solutions and principles of operation of the proposed MAIS | 12 |
| 3.1.3 Substantiation of the choice of structures and materials | 16 |
| 3.1.4 Research and justification of patterns | 18 |
| 3.1.5 Results and discussion | 21 |
| 3.1.6 Analysis and conclusion of test results | 26 |
| 3.2 Research and development of a mobile shearing station | 27 |
| 3.2.1 Literature review, justification of technology and technical solution | 27 |
| 3.2.2 Results of research and testing of MSS | 27 |
| 3.2.3 Description of device technology and wool press | 31 |
| 3.2.4 Results and discussion | 34 |
| 3.2.5 Analysis and conclusion of test results | 40 |
| 3.3 Research and development of wool storage equipment | 41 |
| 3.3.1 Literature review, justification of technology and technical solution | 41 |
| 3.3.2 Results of research and testing of WSE | 42 |
| 3.3.3 Development of software and hardware for the climate control system in the container storage chamber for wool bales | 51 |
| 3.3.4 Analysis and conclusion of test results | 53 |
| 4 Research Feasibility Study | 54 |
| CONCLUSION | 59 |
| REFERENCES | 61 |
| APPENDIX A Calendar work plan (Russian , English) | 64 |
| APPENDIX B Technical tasks | 70 |
| APPENDIX C Acts of economic tests of installations | 86 |
| APPENDIX D Protocols of economic tests of installations | 89 |
| APPENDIX E Acts of installation | 129 |
| APPENDIX F List of publications | 132 |
| APPENDIX G Copies of Published Articles | 135 |
| APPENDIX H Copies of patents and copyright certificates for the protection of intellectual property | 141 |
| APPENDIX I Information about the online consultation | 145 |
| APPENDIX J Copies of drawing documentation of installations | 146 |
| APPENDIX K New Zealand Mission Report | 178 |

DEFINITIONS, TERMS AND ABBREVIATIONS

The following definitions, terms and abbreviations apply:

Vertical integration is in modern agriculture in most cases there is such a chain: the product collection, its processing, sorting, packaging, storage, transportation and, finally, the product sale to the final consumer. The production organization, in which all or several links of a similar chain are controlled, will be vertically integrated.

MAIS – mobile artificial insemination station;

SAIS – stationary artificial insemination station;

MSS – mobile shearing station;

WSE – wool storage equipment;

WP – wool press;

ES – experimental sample;

P – prototype;

NCE - National Chamber of Entrepreneurs

PPW - primary processing of wool

NASEC – National Agrarian Science and Educational Centre

MS – mobile system

SLSS - system of labor safety standards

RTD - regulatory and technical documentation;

F&L – fuel and lubricants;

К – capital costs for the MSS production;

– materials cost;

– cost of purchased parts, components and assemblies;

– auxiliary equipment cost;

– production workers basic salary;

– production workers additional salary;

– charge for renovation, repair and machinery and equipment maintenance;

– total amount of occupancy cost;

– cost of fuel and lubricants;

– other direct operating costs;

W – number of the sheared sheep heads per year;

– economic effect of the MSS using;

– payback period;

– specific operating costs for shearing one sheep.

INTRODUCTION

Relevance of the topic. The basis for competitive development are modern e energo -, resource-saving technology and and automated zatsi I laborious process s [1, 2, 3] . Grazing sheep in Kazakhstan e - priority sector, which contribute to the traditional specialization of the local population, favorable climatic conditions and government support. As a result of the reforms, a new economic system was formed , which is at the stage of formation . For this, a legislative base has been created , the Laws of the Republic of Kazakhstan: "On Veterinary Medicine" [ 4 ], "On State Regulation of the Development of the Agro-Industrial Complex and Rural Areas" [ 5 ], "On Pedigree Livestock Breeding" [ 6 ], "On the Protection of Breeding Achievements" [ 7 ], "On pastures" [ 8 ].

The demand for the industry's products are meat, milk, wool, fur and sheepskin coats which remains stable both in the domestic and foreign markets [9, 10, 11]. To date, due to the lack of special technical means and machines, more than 70% of the grown wool is irretrievably lost, about 25% is subjected to simple washing and is exported from the country at bargain prices. Kazakhstan loses more than ten billion tenge annually on the cost of wool alone. Breeding work to preserve especially valuable fine-fleece and semi-fine fleece livestock is not carried out in the flocks of private household plots, where up to 80% of the livestock is located.

The purpose of this program is to create a technology and a system of machines for the production of high-quality wool in the conditions of distant sheep breeding, which meets modern requirements.

Program objectives

1 Research and development of technology and equipment for mobile artificial insemination stations.

2 Research and develoёpment of technology and equipment of mobile shearing stations.

3 Research and development of technology and equipment for wool storage.

The implementation of the target program (program IRN : BR06249240) was carried out by SPC of Agroengineering LLP, with the participation of scientists from the sheep breeding Research institute after Medeubekov, a branch of the Kazakh Research Institute of Livestock and Forage Production LLP and the Kazakh National Agrarian University NJSC.

The choice of topics for the target program and events was carried out in agreement with farmers, specialists of the Chamber of Sheep Breeders of the Republic of Kazakhstan, NCE "Atameken", agricultural administrations of regional and district akimats, agribusiness.

The review and analysis of the state of sheep breeding in the republic was carried out on the basis of statistical sources [12, 13] and the experience of animal husbandry and sheep breeding in developed countries. First of all, Australia, New Zealand, where similar technological and technical problems are being solved [14].

In 2018, a literature review and patent research were carried out, optimal design and technological solutions were substantiated, technical requirements for the design of MAIS, MSS, WSE were developed and approved. Sketch drawings of experimental samples have been developed.

In 2019, experimental samples were manufactured and tested, and test reports were drawn up for MAIS, MSS, WSE. Terms of reference for the design of prototypes, design documentation were developed, technical and economic calculations were performed. The expected annual savings in direct operating costs from the implementation of the proposed system of machines should amount to 8415 thousand tenge, of which the share of savings from the use of MSS - up to 36%, MAIS - up to 3% and MSS - up to 38.6% and 22.4% - by reducing the cost of manual labor for sorting wool at PPW.

In 2020, prototypes of MAIS, MSS, WSE were manufactured and introduced into production. Acts of introduction of prototypes were drawn up. A recommendation was issued on the use of the developed machines in the farms of the Republic of Kazakhstan. Published 6 scientific articles, including 2 articles, 2 applications for invention were submitted and a PhD thesis was prepared.

The novelty of the research is design and technological schemes, technical solutions and parameters of MAIS, MSS, WSE protected by patents of the Republic of Kazakhstan [31-33], a new design of a shearing machine [34], the use of a heat pump and microprocessors in container technology for storing wool, protected by a patent  and copyright certificates for intellectual property [35-39]

The methodological base was provided by: materials of literature sources and patents, a systematic approach in the formation of production technology, harvesting and storage of wool, theoretical and experimental research and testing of installations.

According to the research results, 13 scientific articles were published, of which 8 were published in the journal from the list recommended by the Science Committee of the Ministry of Education and Science of the Republic of Kazakhstan, 2 in an international journal from the Scopus database, 2 articles were submitted to an international cited journal (under examination). 4 patents of the Republic of Kazakhstan for invention and 4 certificates for copyright were received, an application for an invention was filed. Recommendations on the main results of R&D under the Program were issued. The results were reported and approved at the II Congress of Kazakhstan Sheep Breeders, held by the Ministry of Agriculture of the Republic of Kazakhstan, NASEC, (August 28-29, 2019), at seminars-meetings on the problems of sheep breeding development in Zhanarka District, Karaganda Region (June 10-11, 2019) and the Research Institute of Sheep after Medeubekov on the topic: “Innovative technologies in sheep breeding” (April 10-12, 2019). 2 online consultations for farmers on sheep breeding machines were conducted.

Inventory numbers of reports for 2018 and 2019 are 0218РК01252 and 0219РК00540, respectively.

The main part of the research report

1 Analysis of the current situation

1.1 State of the industry

By 2021, the number of sheep is planned to be increased by 2.8 million and brought to 18.3 million, of which up to 50% will be kept in the farm. In sheep breeding, despite market reforms, traditional technology has been preserved. The availability of pastures allows for the practice of cost-effective free grazing.

The total farmland used by agricultural producers is 79.7 million hectares. Of these, as can be seen in their diagrams (Figure 1) - 51% control large agricultural enterprises, 48% of households and only 1% of private households [15].

Figure 1 - Distribution of farmland by farm category

At the same time, agricultural holdings contain 5–6% of the sheep population (Figure 2).

Figure 2 - Structure of sheep population, 2009

Large farms represent a separate production model. They account for 20-24% of the total sheep population.

Households owns 48% of the land. They account for 25-30% of the total livestock.

Private farms own 1% of farmland, but they have more than 40% of the sheep population, each of which keeps less than 50 animals.

Only 8% of the sheep population is concentrated in relatively large farms, where the number of sheep exceeds 2000 heads. Such farms contain the bulk of the sheep breeding stock for the production of fine wool.

In private farms, grazing is carried out in a common herd, which is formed by several (5–30) neighboring private farms, where each of them owns from one to 50 animals. Fodder and hay are partly purchased and partly harvested by private farms themselves in hayfields using leased equipment (sometimes owned by one or several private farms). Feed (wheat, corn, barley, bran and plant residues) is purchased from specialized agricultural and flour mills. During the stall period, the daily ration of the animal is 0.5–1 kg. Shearing, as a rule, is done by shepherds in the open air using scissors once or twice a year (2–4 kg of wool per animal). Wool pressing is not practiced.

Households are focused on commodity production. They account for 25-30% of the total sheep population. The farm itself is usually located 5–30 km from the settlement. They lease small plots (1-10 ha) of arable land, which they use mainly for the production of forage (oats, alfalfa, corn). Some farms are members of production cooperatives, and some activities (for example, sowing forage crops) are carried out jointly with other members. The technological base is poorly developed. Sheep is sheared once or twice a year (usually in spring, in some cases in autumn). In the northern regions, coarse and (or) semi-coarse wool, as a rule, remains unsold due to low market activity, in the southern and eastern regions it is stockpiled and periodically sold.

Large farms, where 20-24% of the livestock are concentrated, keep from 2000 to 12 thousand sheep (mostly outbred). Their breeding is the main activity of the farms. Seasonal grazing of sheep is practiced in desert and semi-desert areas - in remote pastures. Grazing is carried out all year round, a stock of hay is prepared for feeding. The cultivation of forage crops is not practiced. Feed is periodically purchased from specialized enterprises. Shearing is done with electric clippers, but due to the lack of equipment, subsequent classification of wool is usually not done. Often, these farms have their own wool pressing devices and accept wool from households and small farms, which is then shipped to primary wool processing factories or felting and felt mills. They tend to be highly profitable and plan to expand. They contain only 20-24% of the livestock, but the importance of this group of farms is growing rapidly.

Large enterprises contain from 3000 to 20 thousand sheep. There are about 50 farms specializing in breeding fine-wool or semi-fine-wool breeds. This group of producers also includes pedigree enterprises specializing in the breeding of fine wool breeds and are largely subsidized by the state. Typically, these are partnerships, with some of them controlled by the state or state-owned companies. They practice distant pasture grazing on summer high-altitude and desert pastures, located at a distance of 50-250 km from the farm. Such enterprises, as a rule, use the adjacent arable land and have their own fodder base, where they grow fodder crops, harvest hay, and produce concentrated feed. Shearing are carried out at special points created and equipped during the Soviet era; the classification is carried out by experienced specialists. Most of the agricultural enterprises are unprofitable, a significant part of them are on the verge of bankruptcy. Only a few of them were able to switch to modern management methods and adapt to the conditions of the free market. The technical and technological base of agricultural enterprises is poorly developed, the qualifications of specialists and managers, as a rule, are low.

According to experts, large farms are the most prepared for the future development. They have established sustainable economic activities. Unlike others, they show a desire for expanded reproduction.

The material and technical base, which is mainly used by large farms, is physically worn out and morally obsolete. In small farms and private household plots, the main processes are performed manually.

Accordingly, the selection and breeding work is poorly carried out, it is not possible to prepare and preserve the grown wool on time and with high quality. Due to the lack of minimal household comfort, it is impossible to solve the problem of personnel rejuvenation, who are especially not satisfied with living in traditional yurts, without the Internet. Due to the lack of means of control and protection of sheep kept in pastures, the shepherd&#39;s working day is not limited to grazing, but continues almost around the clock. The existing constructions of sheepfolds do not provide a normal microclimate. Designed according to uniform requirements for livestock buildings, they do not take into account the peculiarities of keeping a large number of sheep, which leads to diseases and damage to wool.

The problem of animal diseases is acute. Not equipped with special mobile systems, veterinary services are not able to cover the control of the movement of animals and products of private household plots, farms.

The problem of forage procurement is one of the most important. Hay harvesting lacks about 50% of metabolic energy and almost 80% of vitamins, up to 30-35% of protein. Hay is not an effective feed due to its rapid deterioration during storage (within six months) up to 60% of the protein is "eroded". Carotene deficiency is also often observed in feed, which also negatively affects health.

1.2 Substantiation of Research Direction

Until recently, sheep breeding in Kazakhstan was based on large farms. Accordingly, the infrastructure consisted of stationary points and installations. As for the mobile sites, they did not receive due attention.

As it’s commonly known, all agricultural machines and technical means are divided into two large groups according to the principle of operation - stationary and mobile or mobile. Among mobile machines, tractors are used, which are mainly used for forage harvesting in an aggregate with forage harvesting equipment.

At the same time, in pasture animal husbandry, where the main production activity is associated with moving, both inside and outside the farm, the use of mobile systems (MS) is relevant. A new direction of MS can be used as a base for robotization.

Therefore, one of the objectives of this Program is to increase competence and formulate requirements for the nomenclature and parameters of mobile systems used for the production of priority types of livestock products, based on the criteria for innovative development of the economy and the material and technical equipment of agriculture.

These and other tasks arising from them are of certain interest in terms of the effective use of MS, the study of their characteristics and capabilities in relation to the conditions of existing farms and “smart farms”.

It should be noted that the gradual development of the idea of "smart farms" is as inevitable as the mechanization of agricultural production processes in the 20th century. Over the past decades, agricultural automation has developed into an independent branch of science and technology, covering the theory, principles of construction and methods of using automated control systems in agriculture, working with minimal or no human participation.

At present, the demanded MSs are mobile artificial insemination stations (MAIS), mobile shearing stations (MSS), mobile product storage equipment (MPSE).

For pasture sheep breeding, the following are also relevant: equipment for pasture water supply (PWS), collapsible mobile sheepfolds (CMS) with an improved microclimate, mobile shepherd houses (MSH), which should replace yurts, service and procurement centers (PC), especially for wool preparation , mobile milking machines (MMM), universal transport and power facilities of high cross-country ability, equipped with all kinds of manipulators, based on all-terrain vehicles and quads.

It is important that these measures will contribute to solving an acute personnel problem, as well as increasing the prestige of the livestock breeder (shepherd cattle breeder) profession. It is alarming that despite the measures taken, the process of population migration to cities continues. If the situation continues like this, there will be no one to work in the village.

Research on the development of mobile shearing stations, conducted by Russian scientists in the early 90s, was due to similar factors [16]:

- the number of farms with a high concentration of livestock has significantly decreased and the number of farms with a small number of livestock has increased, as evidenced by statistical data on the development of agriculture in the country;

- farms of various forms of ownership engaged in sheep breeding, including newly created farms, have a relatively small number of sheep, as a rule, dispersed over a large territory;

- the current situation makes it unprofitable, even impossible to use for machine shearing of sheep and the mechanization of other processes in sheep breeding, previously developed for large farms, efficient high-performance serial equipment. Also, specialized enterprises such as "Aktyubinskselmash" practically stopped producing machines for sheep breeding.

- a common disadvantage of the known autonomous mobile shearing stations based on a tractor and a trailer is the impossibility of simultaneously transporting technological equipment and shearers. When using, it is necessary to additionally involve transport to deliver the service personnel to the cutting site.

The purpose of the developed Program is due to the above mentioned socio-economic conditions and is determined by state programs and tasks [1, 2].

The purpose of the research is to create a technology and a system of machines for the production of high-quality wool in the conditions of distant sheep breeding, which meets modern requirements.

Program objectives:

- to research and create the technology and equipment for mobile artificial insemination stations (MAIS).

- to research and create the technology and equipment of mobile shearing stations (MSS)

- to research and create wool storage technology and equipment (WSE).

Taking into account the relative redistribution of the sheep population by the categories of farms in Kazakhstan, technologies and mobile technical means will be developed that will allow the delivery of service personnel and technological equipment for shearing and insemination of sheep directly to the locations of flocks, as well as equipment for transporting and storing sheared wool.

2 Research program and methodology

2.1 Research Agenda 2018-2020

- Literature review and analysis of technologies and equipment MAIS, MSS, WSE (2018);

- substantiation of the design and technological schemes of MAIS, MSS, WSE (2018);

- development and approval of technical requirements (TR) and sketch drawings for the design of experimental samples of MAIS, MSS, WSE (2018);

- continuation of the analysis of scientific and technical literature;

- production and testing of experimental samples, processing, analysis and generalization of research and test results, drawing up test reports for MAIS, MSS and WSE (2019);

- development of technical specifications for the design of prototypes of MAIS, MSS and WSE (2019);

- development of design documentation for prototypes of MAIS, MSS, WSE (2019);

- completion of design documentation based on the results of testing prototypes (2019);

- production of prototypes, drawing up acts of introduction of prototypes (2019);

- publication of recommendations on the use of the developed machines in the farms of the Republic of Kazakhstan, filing 2 applications for an invention, obtaining 4 certificates for intellectual property, publication of 12 scientific articles, including 2 articles with an impact factor in publications included in the database Scopus (2018-2020);- submission of research results for discussion of international and republican scientific and practical conferences, seminars, consultations;

- preparation of submission of interim annual reports and the final report for 2018-2020 in accordance with STATE STANDARDS 7.32-2001 (2018-2020).

2.2 Methodology for performing R&D

2.2.1 Analysis of scientific and technical literature

The analysis of scientific and technical literature was carried out on the basis of a literature review, by abstract and technical journals: "Technique and equipment for the village", "Technics in agriculture", "Mechanization and electrification of agriculture", "Bulletin of agricultural science of Kazakhstan", "Agriculture Abroad ”, catalogs of equipment of foreign companies, as well as international databases Springer, Elsever, Scopus, Thompson Reuters, Sciencedirect, Kazakhstan Virtual Science Library.

Current patent research was carried out in accordance with ST RK STATE STANDARDS R 15.011-2005, according to sources: CIS, Russia, USA, Japan, Australia, New Zealand, European Patent Office. The patent research report has been drawn up in accordance with STRK STATE STANDARDS R.15.011-2005 [16]

2.2.2 Development of technical requirements and tasks

Technical requirement and task - the main document for the design of a technical object. The technical task further (TT) establishes the main purpose of the developed object, technical requirements, characteristics, quality indicators and technical and economic requirements, instructions for performing the necessary stages of creating documentation (design, technological, software, etc.) and its composition, as well as special requirements. Technical requirements (TR) and TT were developed based on the results of research and testing, the content and list of indicators of which are determined by the requirements of the relevant STATE STANDARDS [17].

2.2.3 Development and revision of plant drawings

The drawings were developed and refined based on the results of experiments and tests of installations in accordance with STATE STANDARDS 2.601-2006 "Unified system for design documentation" [18].

The designers agreed on preliminary layout diagrams, basic technical and technological solutions with the task leaders. The assembly drawing and general view were approved and signed by the manager.

2.2.4 Manufacturing and testing of plant samples

The production of experimental samples (ES) was carried out by employees with the involvement of experimental production specialists according to the developed sketch drawings and under a service contract, under the supervision of the head of the experimental design group, designer and event manager. The tests were carried out in economic conditions, according to the developed programs and test methods. The list of assessments performed during the tests included [19]:

1 Technical parameters (technical expertise)

2 Functional indicators

3 Energy

4 Product safety and ergonomics

5 Operational and technological

6 Reliability

7 Economic

During the technical examination, the following is carried out: examination of the machine design; assessment of the quality of the machine; assessment of conformity of design documentation to State standards; assessment of the availability and speed of assembly, dismantling and adjustment work of units and working bodies; preliminary assessment of compliance with the requirements of safety, labor and environmental protection [20].

The lists and parameters of the ES functional indicators are given in the technical requirements for the developed machines [21].

The energy assessment of agricultural machines and aggregates was carried out at operating modes in which technological operations are stably performed [22].

The measuring instruments have been checked or calibrated in accordance with the requirements of national standards.

The assessment of the safety and ergonomics of products depending on the type of products was carried out in accordance with the relevant STATE STANDARDS [19] and the rules [23].

The nomenclature of indicators and methods of operational and technological assessment were established in accordance with STATE STANDARDS R 52778. Operational and technological [24].

Reliability tests were carried out in real operation conditions or accelerated according to the current normative document, in real operation conditions of products or as close as possible to them within the values specified in the technical documentation for the product [25].

The nomenclature of indicators and economic assessment were carried out in accordance with STATE STANDARDS R 53056 [26], by comparison with the base case (analogue). The best examples of foreign equipment used in zonal agricultural technologies were taken as the base. As the initial information for the economic assessment, the test results obtained by the compared parameters of technology, in comparable conditions, were used.

2.2.5 Publication of articles, filing applications for inventions, obtaining certificates of intellectual property.

Articles should be published in journals included in the list of the Science Committee of the Ministry of Education and Science of the Republic of Kazakhstan, in the scientific and technical journal "International Agroengineering" LLP "SPC Agroengineering" in the materials of international conferences. Applications for an invention were drawn up in accordance with the Rules for the preparation, execution and consideration of an application for an industrial design [27].

Scientific reports were carried out in accordance with approved programs and methods of R&D, in accordance with the requirements of STATE STANDARDS 7.32-2001 [28].

3 Research results

3.1 Research and development of a mobile sheep artificial insemination station

3.1.1 Literature review, justification of technology and technical solution

The results of the literature review, the choice of the direction of research and the substantiation for the constructive and technological solution and the principles of functioning of the MAIS were detailed in the interim R&D report for 2018 [21].

Artificial insemination is the most advanced method of sheep reproduction, providing a more efficient use of breeding resources. 20-30 sheep or up to 1400-1800 ewes are inseminated with sperm received from a ram in one cage per season. At the same time, the need for rams is significantly reduced and you can use high-class producers, tested for the quality of offspring. Using for artificial insemination of ewes of thoroughbred rams-producers, the improvement of the breed and productive qualities of sheep is achieved.

For the organization of artificial insemination of sheep, special points are equipped in the conditions of summer-camp keeping of sheep. The station premises is a mobile plank shed with an area of 45-50 m2, divided into three parts: an arena for receiving semen from breeders, a small (3 ... 4 m2) laboratory and a section for fixing ewes and their insemination. Next to the insemination station, we set up sheds with sheds and feeders for breeder rams and test rams.

The review covered more than 200 literature sources, patent materials. 14 technologies and devices were identified as the closest analogs and prototypes, among which the following deserve attention.

A circular three-machine installation, on which the insemination time of one sheep does not exceed 30 seconds, the installation is serviced by 4 people. An electric motor is used to rotate the platform circle with machine tools. Some authors propose to change the designs of the standard stations and equip a permanent insemination area in three twin sheds. At the same time, 15 ewes spend no more than 5 minutes for insemination. In some farms, due to the lack of sufficient manpower and the lack of appropriate mechanized equipment, primitive methods of insemination of sheep are used. In this case, all the ewes selected in the hunt are driven into a split.

For remote, hard-to-reach areas, or without permanent premises, mobile artificial insemination stations were created. They allowed for a more rational and maximum use of natural pastures, changing locations, which is important from an ecological point of view. For these purposes, a mobile artificial insemination chamber has been designed, which has a collapsible frame and a removable felt covering, sheathed with a waterproof fabric. The chamber equipment consists of: a set of tools, an iron furnace with pipes, a folding machine, folding corrals. The machine was installed in a pen outside the chamber; at the time of insemination, the sheep was pressed against a special window in the chamber wall.

Animals for insemination must be well secured in special pens. Otherwise, it is impossible to protect animals from injury. Due to poor fixation on the pen, the recesses of sheep often reach 30%. For these purposes, a double rotating machine has been developed, where on average 1 minute is spent on inseminating the uterus. Also known is a two-section rotating machine, which consists of two machines mounted on a circle with a diameter of 140 cm. Other designs of rotating machines are known. Labor productivity on such installations reaches an average of 90 sheep per hour. Those. increases in comparison with the previously used method by 1.4 times.

There is flow method of insemination, which was used in stationary points. For these purposes, a 6-machine ring-conveyor unit was developed, where the productivity was 180-200 heads / hour. A group machine was used, where you can simultaneously fix not one, but 10-15 or more ewes at once. In addition, it creates an imitation of herd communication of animals, which reduces their stress level and increases fertility.

However, the known points are tied to the stationary conditions of keeping sheep and do not correspond to the distant-pastry character of sheep breeding.

For intensive restoration of the breeding stock, mobile stations of artificial insemination are most acceptable, Mobile stations can be run by a farmer. If the farmer is not able to carry out the process in a qualified manner, the item can be handed over to a competent specialist who will provide the appropriate services, according to the plan of selection and breeding work with the herd of this breed.

The presence of such points will make it possible to rationally use pasture resources, especially remote from settlements, and in a short time to cover farms, personal subsidiary farms, including those remote from the center, with breeding work. At the same time, the point must be equipped with the necessary devices and equipment, meet the zootechnical requirements, climatic conditions. The structure should be transported by a car, easy to operate and maintain, compact when packed, easily assembled and disassembled.

3.1.2 Constructive and technological solutions and principles of operation of the proposed MAIS samples

Figure 3 shows a general view (a) and a diagram of the first sample of MAIS (b) in a state deployed for operation with the location of the main equipment.

(a)

Figure 3 - Scheme (a) and general view (b) MAIS-1 with the location of the main equipment

The station has a arena 1, a laboratory 2, pens for keeping a batch of sheep 3 and rams 4 selected for insemination, two tambours for non-inseminated 5 and inseminated animals 6, a trailer 7 for transporting a set of equipment MAIS-1, a shower cabin 8 and a toilet cabin 9. To maintain adequate sanitary and hygienic conditions, the inner walls of the arena and laboratory are made of light-colored plastic sheeting. This will allow you to clean and wipe the coating surface daily. The floor of the arena is also made of mesh plastic, which should provide a secure grip on the legs of the animals and be easy to clean. The premises are equipped with side windows 10 that provide the necessary lighting for work. The laboratory is equipped with the necessary devices 11, a sink 12 for washing instruments and working tools, a heater with a water heater 13, which runs on wood, dung. Pens and vestibules are equipped with collapsible fences 14 and gates 15 for the passage of personnel, admission and release of animals. Corral for keeping a batch of sheep selected for insemination 3 and vestibule 5 have a common fence and are separated by a movable fence 16. In the center of the arena, an insemination machine is installed 17. As the number of the first sheep decreases and the number of inseminated sheep increases, it moves, providing an equal density of placement of sheep.

The works are carried out in the following sequence.

The trailer brings the MAIS-1 accessory equipment to the location of the flock. After that, assembly and putting into working condition is carried out: the main arena, laboratories with accessories, corrals, a solar power plant with electrical connections, shower cabins, and a toilet.

Before carrying out the main activities, a heater with a water heater 13 is switched on, the main rooms are heated. The heat output of the heater is calculated from the condition of maintaining the temperature inside the arena at the level of 200C. Artificial insemination is carried out by skilled workers. Usually, insemination technicians are prepared from broodstock herders.

In the morning, before the animals are driven to pastures, insemination technicians beat off the sheep that have come into the hunt from the flock and collect them in the paddock 3. With a herd of 500 heads, the daily average number of such sheep during the insemination period reaches 30-40. lots of 10-15 heads are brought into the vestibule for keeping inseminated sheep 6.

Preliminarily, semen is taken from the rams located in pen 4, which is stored in the laboratory at an appropriate warm temperature. One dose of semen taken from one ram is enough for 10-15 heads.

After that, the inseminated sheep are brought into the arena. The selected sheep are placed in a special pen. At the same time, the internal air temperature in the arena must be maintained within 18-23 degrees. Inseminators, using special tools, carry out the insemination process. After that, the sheep is taken to the vestibule of the inseminated animals 6.

Further, semen is taken from the second ram and the cycle of insemination of the batch of sheep is repeated.

Figure 4 shows the MAIS-2 diagram. Unfolded, working position of the station, with side views, in plan, transport position and drawings of individual units are shown.

| MAIS-2 side view | Transport position MAIS-2 |
| --- | --- |
| Umbrella assembly (pos. 6) | Laboratory cabin (LC) (pos. 1) |
| View of MAIS-2 in plan | Awning canopy (pos. 11) |

Figure 4 - MAIS-2 design diagram

The structure of the station is collapsible and consists of: a cab-laboratory (LC) 1 transported on a trailer, an arena 2, where sheep are inseminated, a folding umbrella 6 that serves as a roof for the arena, as well as a corral 11 located along the back wall of the cabin 1, where rams-producers are kept. The outer fence of the arena 2 is made of three rows of barrier boards 5, and is equipped with gates 3 and 4 for entering and exiting the inseminated sheep from the arena. Umbrella 6 consists of a central pillar 7 extending from its upper end of trusses 8, awning 9, as well as supporting posts 10 installed under the outer ends of the trusses. The outer fence of the corral 11 is made of three rows of barrier boards 14 and is closed from above by a retractable canopy 12, such as an awning which is supported by 13 posts.

The laboratory cabin 1 is equipped with a folding work table, the necessary set of working tools and instruments for performing the insemination process, a tabletop gas stove, dishes for heating water and washing instruments, a photovoltaic battery for a lighting lamp, charging a cell phone and a folding trestle bed for an inseminator technician.

The indoor arena is divided into three parts: the first - for keeping the sheep selected for insemination; the second - for keeping inseminated sheep; the third is where the insemination process takes place. The areas are divided among themselves by portable partitions. A prerequisite for the process is to maintain the temperature inside the arena at 200C, which is provided by a wood-fired heater, dung. The heat output of the heater is calculated according to a well-known method, from the condition of maintaining the internal standard temperature. Artificial insemination is carried out by skilled workers.

The insemination process is carried out in the following sequence: preliminary, 1 ... 1.5 hours before the process, heating of the arena begins; the sheep selected for insemination are brought into the first corral through the left gate 3; the helper alternately feeds the sheep for insemination; the inseminated sheep is brought into the pen for the inseminated sheep, where they are kept for a certain time.

After the completion of the work, the item is disassembled, placed inside the LC, and the LC is installed on a trailer and delivered to the next flock. Dismantling is carried out in the following sequence: the barrier boards of the arena (21 pcs.) And the sheep pen (9 pcs.) are removed; the support posts of the umbrella (8 pcs.) and the canopy (2 pcs.) are removed; Having previously disconnected the central stand of the umbrella from the cable line, the umbrella is folded.

3.1.3 Substantiation of the choice of structures and materials

When choosing constructive and technological solutions, we proceeded from the following provisions:

- ensuring the required performance (daily, seasonal);

- compliance with technologies, temperature conditions (from 18°C to 23°C), sanitary and hygienic requirements and rules during the process of artificial insemination;

As the main premises in MAIS-1 and MAIS-2, a dome-circular structure is used, in which the main arena, laboratory, and vestibule for inseminated sheep are freely located (Figures 5 and 6).

| a) | b) | c) |
| --- | --- | --- |
| d) | e) | f) |

a) General view of PPIO-1; b) inside view of the arena; c) location of the laboratory; d) placement of sheep in the pen selected for insemination; e) a cabin for transporting equipment and sanitary and hygienic measures for personnel (equipped with a shower, sauna, toilet); f) inside view of the cabin.

Figure 5 - General view and device MAIS-1

| a) | b) | c) |
| --- | --- | --- |
| General view of MAIS-2 | Interior view of the laboratory cabin | MAIS-2 transportation process |

Figure 6 - Fragments of installation and general view of MAIS-2

In the autumn period, pedigree rams are kept under a canopy at the station. In this case, the premises of the point intended for keeping the rams are used to collect the seed from the producers. Here, a table for the preparation of an artificial vagina is installed and equipped, as well as a special machine of a simple design), where the ewes are brought.

The structure of the yurt is used as the main premises, in which the main riding hall, laboratory, and tambour for inseminated sheep are freely placed.

The structures are traditional and convenient for breeders engaged in pasture animal husbandry. Possesses high reliability proven for centuries. Maintains integrity during multiple assemblies and disassemblies. It is quickly assembled and disassembled and is resistant to strong winds. The coating keeps heat well, is thermo-drainable, is fireproof, does not emit poisonous gases during combustion, extinguishes extraneous sounds, which is important for maintaining silence when performing the insemination process.

There will be no special problems with the assembly and implementation of such mobile points, since yurts are produced by many enterprises in the republic.

All the above noted positive features of the design solution are important with frequent relocations between flocks. After the end of the insemination campaign, during the rest of the year, the main equipment, including: a room, a caravan, a collapsible fence, a heater with a water heater, showers, a toilet, can be used for the livestock breeder's own household needs. This will increase the equipment utilization rate and significantly reduce operating costs.

3.1.4 Research and justification of patterns

Based on the calculation of the heat balance of the room, which determines the heat load QP0T on the heating system, the parameters of the heating unit are justified. The calculation takes into account heat losses: through the barriers QО, for heating the infiltration air QЕВ, and the air entering the premises through the door openings, in accordance with the sanitary ventilation standard, as well as household heat emissions from people and animals Qб [29, 30].

| QP0T =QО + Q(Е,В) - Qб, Вт. | (1) |
| --- | --- |

Calculation of heat loss through the surface of the fence is made according to the well-known formula:

QО=K۰S۰(tВ-tНБ)۰n۰η, Вт. (2)

where K is the heat transfer coefficient of the enclosure, W/(m2 ° C);

S is the estimated surface area of the fences, m2;

tв - indoor air temperature in the room, (18-23 ° С);

tНБ - design temperature of the outside air, ° С;

n is a correction factor for the operating temperature difference;

η is a coefficient that takes into account additional heat losses.

Heat losses due to natural ventilation are defined as:

Qев=0,278∙Кв∙V∙cp∙ρ∙tв-tн ,                                 (3)

where Кв - air exchange rate; V is the volume of the room, m3;  cp - specific heat capacity of air (1.0 kJ (kg C °));

ρ is the density of air, (1.225 kg / m ^ 3).

The required amount of heat for heating the air in the arena is calculated by the formula:

Q = V * ΔT *· k , kcal / hour,	                                    (4)

where: V is the volume of the room; ∆T is the temperature gradient or the difference between the inside temperature of the arena (+20 degrees) and the ambient temperature (- 10 degrees).

k - coefficient of heat loss (for felt 3.84 kW / deg*h)

Q = 64,1*30*3,84 = 7384,3 kcal / hour or 8,6 kW/h.

The required amount of heat for heating the air in the arena is calculated by the formula:

| Q = V * ΔT * kcal / hour, | (5) |
| --- | --- |

where: V is the volume of the room; ∆T is the temperature gradient; k - coefficient of heat loss (for felt 3.84 kW / deg *h)

Q = 64,1*30*3,84 = 7384,3 kcal / hour or kW/h.

To ensure a power reserve in the event of a decrease in temperature during the period of artificial insemination and to ensure the possibility of heating the yurt in winter, a 10 kW heating unit was selected. The time required to raise the temperature of the arena room to the standard value is calculated as the ratio of the required amount of heat to the power of the heating unit.

t=QР=8,610=0,86 hour,

where: t - time, hour; Q is the required amount of heat for heating the air, kW / h; P is the power of the heating unit, kW.

To solve this problem, a simulation model of the heating and ventilation system of the main premises of the station has been developed [12].

The graphical code of the simulation model of the heating and ventilation system of the premises of the point, developed in the LabView development environment, is shown in Figure 7.

Figure 7 - Graphical code of the simulation model of the heating and ventilation system of the MAIS room.

The following data is entered into the model input:

- geometrical parameters of the yurt (radius and height of the base, and radii and height of the dome);

- thickness and thermal conductivity coefficient of the yurt material (felt);

- parameters of the air environment (temperature change during the day, temperature and air exchange rate in the room);

- the number of animals and people involved in insemination.

At the output of the model, we get a graph of changes in the heat balance during the day with an hour interval. The parameters of the yurt and the required temperature in the room are entered into the constant libraries.

The Sequence Stack library sets the temperature in the region for every hour of the day, Figure 8.

Figure 8 - Window for entering temperature parameters into the Sequence Stack library

The temperature for each hour is taken on the basis of statistical data from KazHydroMet in the Almaty region. Also, air exchange parameters are entered into the constant blocks. The multiplicity of air exchange agrees with Building regulations equals 2. The Source library contains information about the number of sheep and people in the area. The data obtained from the libraries for entering the initial parameters are sent to the SubSystem blocks, where, on the basis of mathematical models, the parameters of the building, its heat loss, heat transfer of animals are calculated and the heat balance is constructed.

3.1.5 Results and discussion

To check the adequacy of the model, the heat loss was calculated for two temperature values, the maximum (+ 4 ° С) and minimum (- 3 ° С) according to the formula (2) and on the simulation model of the heating and ventilation system of the room. Comparison of the data obtained as a result of calculations with the data obtained on the model showed approximately the same results, which confirms the validity of using the model to calculate the temperature and air exchange rate in a room under various environmental conditions.

Based on the calculations of the power and energy consumption of electrical appliances, the equipment of a solar photovoltaic station (SPS) was selected. The installed capacity of electricity consumers is 826 watts, the daily power consumption is 2.1 kW. [11].

A fragment of MAIS-1 tests is shown in the figure 9.

Figure 9 - Fragment of tests MAIS-1

The control and registration of temperature and humidity inside the premises was carried out using temperature and humidity sensors (thermohygrometer) of the IVTM-7N brand, the measuring range of the relative humidity is from 0 to 99%, the temperature measuring range is from - 45 ° C to + 120 ° C. The readings of the temperature and humidity sensors over time are shown in Figure 10.

Figure 10 - Installation of thermohygrometer sensors inside the PPIO-1 arena

For automatic recording of the thermohygrometer readings, a two-channel universal electronic recorder IS-203.3 was used, which provides the ability to connect to a PC for recording and displaying readings in real time. Fragments of graphs of temperature and humidity changes for one day are shown in Figure 11.

According to the readings of the sensors and graphs, the change in temperature and humidity inside the MAIS implies that during the artificial insemination of sheep, from 13 to 15 hours on November 15, 2019, the temperature (18-20 ° C) and relative humidity (36-42%) comply with zooveterinary standards.

Figure 11 - Fragments of graphs of temperature and humidity changes during artificial insemination of sheep.

The results of the timing of the operations of the technological process of artificial insemination of sheep are shown in Table 1.

Table 1 - Results of timing of operations of the technological process of artificial insemination of sheep

| Repetition experiences | Time spent on: |  |  |  |  | Total sec |
| --- | --- | --- | --- | --- | --- | --- |
|  | feeding sheep into c tank, sec | sheep fixation, sec | sheep insemination process, sec | De-fixing Sheep, sec | driving the sheep out of the pen, sec |  |
| 1 2 3 4 5 | 63 57 59 62 59 | fifteen eleven 15 14 11 | 64 62 56 55 57 | 12 10 12 14 9 | 5 7 4 8 5 | 95 85 90 98 84 |
| Amount | 300 | 66 | 294 | 57 | 29 | 452 |
| Avg. | 60 | 13.2 | 58.8 | 11.4 | 5.8 | 149.2 |

The timing results showed that it takes about 150 seconds to inseminate one sheep. Therefore, the productivity of the process of artificial insemination of sheep is 24 heads per hour. The main technical and technological indicators of the experimental sample of MAIS are shown in Table 2.

Table 2 - Results of technical and technological assessment

| # | Indicator name | Units | Indicator value |  |
| --- | --- | --- | --- | --- |
|  |  |  | According to calculations | According to experiment |
| 1 | Work shift performance, | heads | up to 30 | 26-28 |
| 2 | Seasonal performance, | heads | 500-1000 | 561 |
| 3 | Insemination room temperature | ° C | 15-20 | 18-25 |
| 4 | Maximum relative humidity of the insemination room | % | 70 | 35-52 |
| 4 | Insemination machine dimensions | m | 1.0x 0.7x0.9 | 1.0x 0.7x0.9 |
| five | Trailer cabin dimensions | m | 3.0 x 2.0 x 2.0 | 3.0 x 2.0 x 2.0 |
| 6 | Equipment weight | t | up to 1.5 | 1.46 |
| 7 | SPS rated power | W | 400 | 450 |
| 8 | Battery capacity | Ah | 80 | 80 |
| nine | Inverter rated power | W | 500 | 1000 |
| 17 | Labor intensity of installation | person-h | No more than 6 | 5 |

Comparative tests of an experimental sample of MAIS and a stationary point of artificial insemination of sheep were carried out during the period of the breeding company, from October 30 to November 28, at the kurozek and Kashara No. 1 sheepfolds of the experimental base of the Kazakh Research Institute of Livestock and Forage Production LLP, in which a level playing field was created.

The reproductive abilities of the sheep of both points were assessed according to: fertility of ewes from the first and second inseminations, the intensity of the arrival of ewes "into the hunt" and passing of the lambing company, the number and proportion of inseminated and unseminated sheep, the yield of lambs per 100 lambing and inseminated queens.

On the site "Kurozek" for 24 days, out of 561 heads of bright, 554 heads (98.75%) were fruitfully inseminated, while in "Kashar No. 1", where the stationary point worked, the process lasted 6 days more, where 604 were fruitfully inseminated. (98.69%).

Comparative dynamics of insemination of bright birds in the context of the day on MAIS and SAIS is shown in the diagram shown in Figure 12.

Figure 12 - Comparative dynamics of sheep insemination by day

Comparative dynamics of lambing of bright birds inseminated on MAIS and SAIS in the context of days is shown in the diagram shown in Figure 13.

Figure 13 - Comparative dynamics of the results of lambing is bright in the context of the day.

In general, at both points, there was a high proportion of sheep that came “to hunt”. In the first 4 five days of mating, the insemination process went through intensively, which, in general, indicates a fairly high preparedness of the sheep for the breeding season and their good fatness. Comparison of the results of the study of the performance indicators of the MAIS and the SAIS are given in Table 3.

Table 3 - Comparative indicators based on the results of the work of the AI points

| Indicators | MAIS |  | SAIS |  |
| --- | --- | --- | --- | --- |
|  | heads | % | heads | % |
| Number of sheeps at the beginning of mating | 561 | 100 | 612 | 100 |
| Inseminated total , including: | 554 | 98.75 | 604 | 98.69 |

Table 3 continuation

| from primary insemination | 531 | 94.65 | 578 | 94.45 |
| --- | --- | --- | --- | --- |
| from re-insemination | 23 | 4.10 | 26 | 4.25 |
| Non-seeded (barn) remained | 7 | 1.25 | 8 | 1.30 |
| Did not lamb among the inseminated | 8 | 1.44 | 12 | 1.99 |
| Sheep died from mating to lambing | 5 | 0.89 | 7 | 1.14 |
| Have sworn | 546 | 98.55 | 592 | 98.01 |
| Total lambs received | 584 | 100 | 645 | 100 |
| Per 100 inseminated sheep (first born) | - | 105.42 | - | 106.79 |
| Per 100 breech sheep (fertility) | - | 106.96 | - | 108.95 |

7 heads (1.25%) remained non-coincidental in terms of MAIS, 8 heads (1.31%) in terms of SAIS, due to the individual characteristics of the organism. High fertility rates were obtained (98.75% - MAIS, 98.69% - SAIS) for both points of IO of sheep, which is at the level of indicators of breeding farms where artificial insemination of sheep is systematically carried out. The intensity and synchronicity of insemination and brood of sheep indicate a high fertilization rate of ewes from the first insemination, and in general, a fairly high quality of organization and conduct of breeding and lambing campaigns. The fertility of first-born ewes was 105.42-106.79 %%, which also indicates the correct organization and implementation of breeding and lambing campaigns using mobile and stationary artificial insemination stations for sheep.

3.1.6 Analysis and conclusion of test results

1. Comparative tests of MAIS on a pasture, where there is no stationary artificial insemination station, provided the results of insemination and lambing comparable to the results obtained at the stationary station. The intensity and synchronicity of insemination and brood of sheep according to the results of artificial insemination indicate a high fertility of ewes from the first insemination, and in general, rather high quality of organization and conduct of breeding and lambing campaigns and the consistency of using a mobile artificial insemination station for sheep reproduction.

2. Tests have shown that the completeness of the mobile station for artificial insemination of sheep meets the zoohygienic requirements in the conditions of pastoral sheep breeding. Technological and technical parameters of the equipment of the mobile station for artificial insemination of sheep ensure the implementation of artificial insemination of sheep in personal and farms of distant-pasture sheep breeding with departure to flocks.

3. The use of technology and equipment MAIS provides the necessary zooveterinary conditions for the technical and technological modernization of pasture sheep breeding, improving the breed of the herd by artificial insemination with sperm of high-yielding sheep in distant sheep breeding and allows to reduce the unit operating costs by 20-25 percent compared with artificial insemination of sheep at a stationary point.

4. The developed simulation model makes it possible to study changes in the heat balance in the MAIS per day, depending on changes in the ambient temperature and the presence of additional heat sources in the room with an hourly interval, which allows you to adjust the operating mode of the heating and ventilation system of the point to ensure the required conditions in the room. Comparative analysis of the data obtained on the model and the data obtained as a result of experimental studies practically coincide, therefore, the developed simulation model gives adequate results and can be used to simulate thermal processes in similar premises built on the basis of a yurt.

3.2 Research and development of a mobile shearing station

3.2.1 Literature review, justification of technology and technical solution

The results of the literature review, the choice of the direction of research and the rationale for the constructive and technological solution and the principles of the functioning of the MSS were detailed in the interim R&D report for 2018 [21].

The development of the optimal circuit and design of the device, experimental research, measurements of parameters and the establishment of the sought regularities were carried out by processing and interpreting the data. The results obtained were used to develop technical requirements (TR) for the projected sample of the mobile station.

The step-by-step manufacture and study of laboratory, prototype and experimental samples pursued the goal of finding fundamental (constructive, schematic, etc.) product solutions that would give a general idea of the product design and principles and optimal operating modes. At the stage of development of the draft design, the options of its component parts were considered, which were necessary to ensure the requirements for the product and allow to establish fundamental solutions.

The acquisition and manufacture of the experimental sample was carried out according to the developed drawing documentation. The economic tests were carried out according to the developed program and methodology, where the design, operational, technological and economic indicators were assessed and their compliance with the approved technical specifications.

Analysis of scientific research, recommendations, traditional technologies and technical means of machine shearing of sheep, as well as work on promising technologies in these areas shows that the previously widely used and produced serial technological equipment for machine shearing of sheep is currently not in demand for a number of reasons. The R&D review for 2018 covered more than 300 literary sources, patent materials. Ten technologies and devices were identified as the closest analogs and prototypes [21].

3.2.2 Results of research and testing of MSS

When designing the MSS, the following requirements were taken into account: to divide the flock received for shearing into groups according to wool quality, first shearing sheep with less quality wool; place wool of different groups in separate bales. At the same time, the following tasks are solved:

- by shearing, first of all, sheep with a lower quality wool, the shearer and auxiliary personnel are able to restore their working tone. Sharpeners of cutting pairs of shearing machines enter the rhythm, since the performance of the shearer depends on the high-quality sharpening of the knife and comb and the correct setting of the machines. Manages to get a rune safe and sound. The cut of the wool is smoother and lower, avoiding areas with highly clipped wool, which reduces clipping and reduces the quantity and quality of the fiber due to its shorter length;

- varietal mixing of wool is excluded, since wool of different groups falls into separate bales.

The layout of the proposed mobile shearing station (MSS) is shown in Figure 14.

Figure 14 - Scheme of a mobile shearing station

The mobile shearing station consists of the following main parts: collapsible fences forming sections 1, 2, 3 and 4 for the separate keeping of groups of sheep, with entrance 5 and internal gates 6 for moving animals, splits 7 for the organized movement of animals for shearing, and also a transformable container 8 and a container for loading and removing bales of wool 9.

Sheep received for shearing are divided into groups and kept in separate sections of the corral. The first to be sheared are sheep from section 2, with a lower quality wool, the second - from section 1, following in wool quality. Then from section 3 and the last from section 4, with the highest quality wool. In the process of moving inside the corrals, the sheep are passed through the inner gate 6 and are brought to the shearers in an organized manner through the splits-passages 7. For the convenience of the movement of animals, section 2 has a wedge-shaped shape so that the sheep can be guided and fall into the split-passages 7. And the splits having a width of 0 , 4 ... 0.5 m, allow you to line up the sheep in one row one after another.

Figure 14a - Scheme of the main site

Along each side of the transformable container 8 there are work stations of shearers 10. They are equipped for the simultaneous operation of ten shearers, five on each side, as shown in auxiliary figure 14a. The clippers are powered by an autonomous power plant 13, which also supplies electricity to lighting fixtures.

When the sheep reach workplaces 10 (Figure 14a), they shear through the gates 11, which are installed opposite each shear, they bring out the next sheep, shear it and release it into the space under the shearing platform. The trimmed fleece is transferred to the main platform 12. Classification, pressing and marking of wool bales are carried out here. Further, the finished bales are loaded into container 9 using a special stacker trolley.

The main site is formed by transforming the container. To do this, its side walls, folded and fixed in the transport position vertically, as shown in Figure 15, are unfolded upon arrival at the destination. For this, the outer wall 1 pivotally rises up around the axis 2, forming a canopy over the platform. The inner wall 3 pivotally lowers down around the axis 4 to a horizontal position, forming an extension of the container floor. In the transport position, the side walls are folded and brought to their original position.

Figure 15 - Layout of side walls in transport position

The processes of lifting and lowering the side walls are carried out using one manual cable winch 1 (Figure 16).

To raise or lower the lower wall, the winch hook is hooked onto the lower load eye 2, and to raise the upper wall, the hook is hooked onto the upper eye 3.

Figure 16 - Scheme of the hoisting winch

The work of the shearing station is carried out as follows.

Removable convertible container delivered to the place of deployment by car. All the necessary equipment is loaded into it: sections of a collapsible sheep pen, a bathing bath, removable fences for the working area, a wool press, shearing machines, a machine for sharpening machine knives.

Upon arrival at the set point, the shearing station is deployed and brought into working condition. The side walls are removed from the latches and brought into a horizontal position. The front wall of the container body is also brought into a horizontal position and forms a ladder.

Then the entire enclosing structure, the sheep pen, is installed, an autonomous power plant is installed, the wires are connected to power the shearing machines.

According to the previously drawn up schedule, the flocks of sheep stay at the point, are driven into the corral, and they are divided into four groups according to the quality of wool. The sheared wool is picked up by balers and bales are pressed with a wool press. Finished bales of wool are loaded into container 5. After filling, container 5 is delivered by a car to the storage site.

3.2.3 Description of device technology and wool press

Figure 17 shows general views (front view (a), side view (c) and general views (b, d) of an improved model of a mechanical press for wool WP-2, where: 1 - lower bale chamber, 2 - upper loading chamber, 3 - pressing plate, 4 - clamp, 5 - power handle.

Main technical characteristics of WP-2:

- press plate stroke - 785 mm;

- compression force of the pressing plate - 20.4 kN;

- maximum torque on the power handle - 600 Nm.

Static rope "Shturm 10" used in industrial mountaineering is used as a rope for the chain hoist. Diameter - 10 mm; breaking load, 3100 kgf; elongation (50-150 kg), no more than 1.5%.

The work of the wool press is carried out as follows.

A cover for wool is installed in the lower chamber. The four edges of the case expand and attach securely to the edge of the camera. Fastening is carried out with special rods (4pcs), with which the edges of the cover are twisted and fixed around the edge of the lower bale chamber. With the help of the power handle 5, the press plate 3, suspended on the ropes of the chain hoist, rises to the upper extreme position. The upper loading window opens, through which the wool is loaded until the lower and upper chambers are completely filled.

| a) | b) |
| --- | --- |
| c) | d) |

Figure 17 - Front view (a), side view (c) and general views (b, d) of an improved model of a mechanical press for wool

After filling, the pressing process begins by activating the press plate, which compresses the wool to a certain density. The chambers are re-filled and re-pressed. Such actions continue until a normally compressed bale is obtained with a compression force of up to 2.0 tons.

During operation, the rope should be laid on a power reel (200 mm diameter) and a reel for lifting the press plate in one layer. The ends are attached to the sides of the reels in place using the holes provided. Full press plate stroke (785 mm) corresponds to 5 turns of the power coil.

Figure 18 shows an assembly drawing of the press

Figure 18 - Assembly drawing of an improved sample of a mechanical press for wool

During operation, the rotating blocks are periodically lubricated with industrial oil of the I-20 brand. Do not get grease on the rope.

After assembly, restore all damaged paintwork on metal surfaces.

The power handle is stored and transported in a personal case. Allows to perform work from either side of the working shaft.

3.2.4 Results and discussion

The dimensions (power), main and auxiliary equipment of the MSS were determined by the given initial conditions. The list of the main set of equipment for MSS used in the testing process is given in Table 4.

Table 4- Set of equipment for MSS

| Name and brand | Qty. | Name and brand | Qty. |
| --- | --- | --- | --- |
| Shearing machine Heiniger ONE 722-610 230 V / 300 W EU | 12 | Autonomous power plant Honda EG5500CXS (6 kW) | 1 |
| Classification table STANDARD CC SRO-15 / 6STSK | 2 | Machine for sharpening knives for shearing Heiniger the Easy Grinder | 2 |
| Bale stacker Prolift the MS 1016 | 1 | Workbench WB 120Sh + WD1 | 1 |
| Multifunctional bench scales MK 6.2 A21 | 1 | Mechanical wool press | 2 |
| Scales Hercules 1000 |  | Collapsible Sheep Pen | 69 |

Operational and technological assessment

ES tests were carried out in economic conditions, according to the developed programs and standard test methods. The typical list of evaluations performed during testing included the determination of technical parameters (technical expertise), operational and technological, energy, ergonomic indicators, reliability and economic indicators. The test results are shown in the following tables.

Table 5 - Operational and technological indicators

| Indicator names | Units | Indicator value |  |
| --- | --- | --- | --- |
|  |  | according to technical requirements | when testing MSS |
| Number of days (τ) of the shearing | day | 25-30 | 26 |
| Shift duration | h | 8 | 8 |
| The number of simultaneously working shearers | people | up to 12 | 10-12 |
| The number of sheep sheared per shift: - skilled shearer - unskilled shearer | heads | up to 100 up to 60 | 80-100 40-50 |
| Amount of clipped wool (Q n ): - skilled shearer - unskilled shearer | kg / shift | 250 150 | 275 125 |
| Medium cut from one head | kg | not less than 2.5 | 2.6 |
| The proportion of cases of skin cuts: |  |  |  |
| light: - skilled shearer - unskilled shearer | % | 5 15 | 5 15 |

Table 5 continuation

| heavy: skilled shearer unskilled shearer | % | 0.5 3 | 0.5 2.5 |
| --- | --- | --- | --- |
| Cutting height of wool: - skilled shearer - unskilled shearer | mm | 5 6 | 5 7 |
| Mass fraction of cutting and re-cutting; - skilled shearer - unskilled shearer | % | up to 5 to 10 | 4-5 8-10 |
| The number of sheep sheared with one cutting pair from sharpening to sharpening: - skilled shearer - unskilled shearer | heads | 10 12 | 11 13 |
| Wool press |  |  |  |
| Bale size: | mm | 980x700x700 | 980x700x700 |
| Bale weight | kg | 110 | 110 |
| Bale density | kg / m 3 | 250 | 250 |
| Press cycle time | min | 28 | 25 |
| Fabric consumption per bale | m 2 | 3.8 | 3.8 |

| Fragments of sheep shearing |  |  |
| --- | --- | --- |
| Fragments of wool pressing and bale formation |  |  |

Figure 19 - Fragments of MSS tests

The main processes that determine the productivity of the station are: fitting, feeding sheep for shearing, shearing, grading, wool pressing, packaging, weight measurement, electronic registration of quality, weight of wool in a bale, applying an electronic barcode on bales, transportation, loading and stacking bales into a shipping container.

Auxiliary processes are: sharpening and adjustment of shearing knives, installation, dismantling, operation of the MSS, fencing of corrals, electrical equipment.

Additional measures: labor protection, safety measures, accommodation, meals and rest of staff.

The shear performance (nгол), which depends on the degree of perfection of the cutting methods and techniques, as well as the level of its qualifications, is characterized by the following performance indicators:

ηсм is the utilization rate of the shift time (ηсм=τч.рτсм where τч.р is the time of clean work, h; τсм is the duration of the shift, h);

ηр.х is a coefficient that takes into account the time spent on working passes by a machine (ηр.х=τр.хτч.р, where τр.х is the time of working passes, h);

ηв - coefficient that takes into account the use of the constructive width (grip) of the knife (ηв=bрb, where bр is the working width of the grip, m; b is the constructive width of the knife, m).

Shear productivity, or its hourly operating time, nч head / h, with an individual shearing is determined by the following relationship:

nч=3600∙ηр.х∙ηсм∙ηв∙b∙vпSo	      (6)

where: vп - machine feed speed, m / s;

So- area of the sheared surface of one sheep, m2.

The following values of the coefficients   ηв are accepted: for highly skilled shearers   ηв= 0.88-0.92; average   ηв= 0.78-0.87; low   ηв= 0.69-77.

Based on the results of time observations, the average time spent on the process was established:

- sheep feeder - 0.375 min per one sheep;

- wool classifier (with an assistant) - 0.371 min for one fleece;

- grinder of cutting pairs - 1.35 minutes per pair;

- fitter-adjuster - 2.05 minutes per call.

At the same time, one filler, as well as a fitter-adjuster, served 12 shearers;

- grinder of cutting pairs - can be more than 12 shear.

Depending on the breed, age and keeping conditions of the sheep, the shearing of wool from one sheep varies within a fairly wide range - 1.5 ... 4.0 kg.

The loss of wool due to poor-quality shearing (re-cutting, cutting) was up to 5% in skilled shearers and up to 10% in unskilled shearers.

Performance reliability assessment

Operational reliability was assessed in accordance with industry standards [8]. At the same time, such indicators were determined as:

1) machine time between failures

2) the specific labor intensity of current repairs and scheduled maintenance

3) factors of availability and technical utilization of the machine, etc.

Information about machine failures during operation was systematized by units, systems, assemblies and working bodies.

During the tests, the general timing of the elements of the operating time, maintenance and routine repair of machines was carried out, that is, the net operating time, the time for carrying out maintenance, for the elimination of technological failures, etc., were recorded separately.

The operating hours of the machines were measured in hours of main work. If the duration of the use of the machines is more than one month per year, it is permissible to determine the time of the main work based on the operating time in hectares or tons for the entire test period and according to the results of selective timing.

When calculating the reliability indicators, we took into account defects, failures and malfunctions that were identified during dismantling, initial technical examination, and running-in.

The results of assessing the reliability indicators are shown in Table 6.

Table 6 - Indicators of reliability

| Indicator names | Units | Indicator value |  |
| --- | --- | --- | --- |
|  |  | according to technical requirements | when testing MSS |
| MTBF | h | more than 240 | 280 |
| Installation time | h | no more than 8 | 8 |
| Labor intensity of installation | person h | no more than 24 | 20 |
| Number of refusals: |  |  |  |
| I group of difficulty | units | no more than 5 | 3 |
| II group of difficulty | units | no more than 3 | 1 |
| III group of difficulty | units | no more than 1 | - |
| Labor intensity of maintenance | man-h | no more than 3.5 | 3.0 |
| Specific labor intensity of current repair | person h | no more than 5 | five |
| Average recovery time | h | no more than 0.5 | 0.3 |
| Availability ratio | unit | not less than 0.98 | 0.98 |

The tests showed that the reliability indicators for 10 established criteria met the technical requirements: MTBF was 280 hours; time spent on installation of the installation up to 8 hours; labor intensity of installation up to 20 people • h; 4 failures of the 1st and 2nd complexity were recorded; the average recovery time for failures of the 1st and 2nd complexity did not exceed 0.3 hours.

Maintenance was carried out at the end of the work shift every day, the labor intensity of which did not exceed 3 people • h. The availability factor was 0.98, which does not exceed the values established by the technical requirement.

Safety and ergonomics assessment

The assessment of the safety and ergonomics of products was carried out according to the relevant STATE STANDARDS, depending on the type of product [9]

Table 7 - Indicators for assessing working conditions

| # | Indicators for TR, SLSS | Indicator value |  |
| --- | --- | --- | --- |
|  |  | according to RTD | at tested bandwidth |
| 1 | Physical load on the employee's musculoskeletal system | no more than 50 kg | no more than 50 kg |
| 2 | Sensory load on the central nervous system and the employee's sense organs | - | no |
| 3 | Microclimate: |  |  |
|  | - air temperature, С | 20 | 15-28 |
|  | - relative humidity, % | 35 | 25-43 |
|  | - air speed, m / s | 0.1 | 0.1-0.3 |
| 4 | Illumination, lx | not less than 50 | 50-100 |
| 5 | Chemicals and mixtures | - | no |
| 6 | Serviceability | conveniently | conveniently |
| 7 | Convenience of technological service | conveniently | conveniently |
| 8 | Number of working movements per minute | ten | ten |
| 9 | Working posture | rational | rational |
| 10 | Convenience of mounting and dismounting | conveniently | conveniently |

Energy rating

Energy assessment is carried out in order to determine the energy consumption for technological operations. The assessment is carried out on operating modes in which technological operations are stably performed, while the number of modes should be established by standards for specific types of machines. The nomenclature of indicators and methods of energy assessment were determined in accordance with STATE STANDARDS R 52777.

Table 8 - Indicators of energy assessment

| Indicator name | Indicator value |
| --- | --- |
| The amount of active energy spent on the execution of the technological process, kW h / day | 33 |
| The amount of reactive energy spent on the execution of the technological process, kW h | 3 , 77 |
| Average power factor | 0.95 |
| Specific energy consumption, MJ / kg | 0.052 |

Monitoring environmental conditions

Microclimate, environmental parameters were determined using a Vantage Pro2 6152RU weather station. The weather station automatically measures and displays the parameters of the state of the environment on the console display, and also stores the maximum, minimum values of the main meteorological data in the memory of the console for a day, month, year.

Figure 20 - Vantage Pro2 weather station instrument set

The graphs of the state of the environment on the days of control measurements (24.06.19-07.07.19) are shown in Figures 21 ... 23.

Figure 21 - Graphs of changes in ambient temperature

Figure 22 - Graphs of changes in relative humidity

Environment

Figure 23 - Graphs of wind speeds

The economic assessment of the new technology was carried out by comparison with the base case (analogue). The best examples of foreign equipment (Russia) used in zonal agricultural technologies were taken as the basis for comparison. As the initial information for the economic assessment, the test results obtained by the compared parameters of technology in a specific soil-climatic zone under strictly comparable conditions are used. The economic assessment of the effectiveness of new technology is multi-criteria in nature, and the choice of one or another efficiency criterion is determined by economic, social and environmental problems that require their solution in agricultural enterprises of all forms of ownership. The main indicators of efficiency are operating costs, annual economic effect.

Table 9 - Indicators of economic assessment

| # | Indicator names | Units | MSS |
| --- | --- | --- | --- |
| 1 | Capital expenditures | tenge | 8 000 000 |  |
| 2 | Capital costs with installation | tenge | 10 000 000 |  |
| 3 | Operating costs for: | tenge | 1153 250 |  |
|  | Depreciation deductions | tenge | 500,000 |  |
|  | Overhaul | tenge / per season | 1 000 000 |  |
|  | Fuel consumption, etc. | tenge / per season | 173250 |  |
|  | Expenses for food, accommodation | tenge / per season | 480,000 |  |
| 4 | The sum of the reduced costs | tenge / per season | 1 000 000 |  |
| 5 | Annual economic benefit | tenge / year | 511750 |  |

3.2.5 Analysis and conclusion of test results

The tests of the mobile shearing station were carried out in real conditions during the period of the shearing company (April-May) for 26 days.

MSS equipment is well suited for prompt moving, installation and dismantling. In the transport position, the set is transformed into a container, the disassembled equipment is placed inside the container. In working order, the equipment is deployed on a level surface. Installation time does not exceed 8 hours, labor intensity, no more than 24 people ∙ h.

The shift performance of a qualified shearer was up to 80-100 sheared sheep, unskilled up to 50 heads. Losses of wool due to poor-quality shearing (re-cutting, chopping) were up to 5% in skilled shearers and up to 10% in unskilled shearers. Changeable productivity of the station - up to 800 goals, seasonal - 16,000 goals. The size of the pressed bales is: 980x700x700 mm, weight 150 kg, density 250 kg / m3, pressing time of one bale is not more than 28 minutes.

The annual economic effect due to the reduction in operating costs is 511,750 tenge / year.

The results of tests and studies of MSS confirm the effectiveness of the new design and technological solutions.

3.3 Research and development of wool storage equipment

3.3.1 Literature review, justification of technology and technical solution

The results of the literature review were detailed in the interim R&D report for 2018 [21]. Also, the choice of the direction of research and the substantiation of the constructive and technological solution and the principles of functioning of the WSE were carried out. The R&D review for 2018 covered more than 100 literary sources, patent materials. Three technologies and devices were identified as the closest analogs and prototypes [21].

The results obtained were used to develop technical requirements (TR) for the designed sample of wool storage equipment. The selection and development of the optimal circuit and design of the device, subsequent experimental studies were carried out by processing and interpreting the data.

At the stage of development of the draft design, options for its component parts were considered, giving a general idea of the device and the principles and optimal modes of operation necessary to meet the requirements and allowing to establish fundamental solutions. The subsequent manufacture and study of laboratory, prototype and experimental samples pursued the goal of finding fundamental (constructive, schematic, etc.) product solutions.

The acquisition and manufacture of the experimental sample was carried out according to the developed drawing documentation. The economic tests were carried out according to the developed program and methodology, where the design, operational, technological and economic indicators were assessed and their compliance with the approved technical specifications.

Analysis of scientific research, recommendations, traditional technologies and technical means for storing wool, as well as work on promising technologies in these areas shows that the previously widely used and produced serial technological equipment is not acceptable in terms of the main technical and operational, energy and technical and economic indicators.

One of the main disadvantages of the existing technology is that the normalized microclimate is not maintained, which leads to uncontrolled reproduction of moths - the main pest of wool and wool products. To kill moths, high (up to + 60 ° C - + 70 ° C) and low temperatures (below + 12 ℃) are recommended.

In the proposed technology, the wool bales are loaded into a container at the shearing station. The bales loaded into containers are transported to the PPW by a container ship. Containers loaded with bales are installed in a designated area, even outdoors. At the same time, unloading and stacking of bales is not performed. The bins are only opened for bale removal just prior to processing in the factory.

Thus, multi-tiered storage of containers will ensure a better use of the area allocated for the storage of raw materials in the factory. Warehousing investment is reduced as bales are stored directly in containers. There is a saving of the wage fund, due to the reduction of workers employed in warehouse work, heavy physical labor in the operations of unloading and storing bales, as well as the loss and damage of bales, is eliminated.

The container transportation system is based on strict standardization and unification of technical means, which has led to its international character. Wool storage equipment is quickly and conveniently transported, meets the requirements for wool storage in the conditions of distant sheep breeding. Component parts can be easily repaired and replaced in case of failure.

3.3.2 Results of research and testing of WSE

The equipment consists of a general-purpose cargo container, the chamber of which can hold 56 pressed bales of wool in the size: 980x700x700 cm. The moisture content of the loaded bales should not exceed 17% (Figure 24). It includes: container 1, consisting of front, back, side, vertical walls and a heat-cooling unit 2, as well as a stacker. The stacker is designed to mechanize the process of stacking bales, weighing 110-150 kg, in vertical rows in the container chamber. A detailed description of the developed model of the stacker will be given below in the text.

| 3 |  |
| --- | --- |
| a) | b) |
| 2 |  |
| c) | d) |

Figure 24 - Equipment for the transportation and storage of wool

a) diagram of air movement inside the chamber; b) side view; back view;

d) top view

The heat-cooling unit (HCU) consists of a heat pump 1 of the "water-to-water" type, consisting of a compressor, an evaporator, and a condenser. Detailed functional, energy and technical characteristics of the heat pump are given in our article published in the cited international journal [37].

Figure 25 shows the layout of the main elements of HCU. Temperature control is carried out by a microprocessor according to signals from temperature and humidity sensors 6, installed in the upper zone of the container chamber (Figure 24).

Figure 25 - Layout of the main elements of HCU

HCU works as follows. If the temperature in the container exceeds 10 ° C or the moisture content of the bales exceeds 17%, the heat pump is switched on. In this case, by means of the evaporator, excess heat and moisture from the chamber is absorbed. Further, the absorbed heat, through the condenser, heats the water in the water heater installed on the roof of the container. Thus, autonomous and remote monitoring of the wool condition is carried out during long-term storage.

Figure 26 - General view of the stacker structure

As can be seen from Figure 26, the stacker structure consists of: main frame - 1, lifting frame - 2, carriage - 3, pitchfork - 4, rack and pinion jack - 5 and static rope "Shturm 12" - 6.

Table 10 - Technical characteristics of the stacker

| Indicator names | Indicator values |
| --- | --- |
| Carrying capacity no more, kg | 300 |
| Minimum pitchfork height from the floor, m | 0.08 |
| Maximum lifting height with a mass not exceeding 300 kg, m | 1.5 |

Table 10 continuation

| Maximum lifting height with a mass of not more than 150 kg, m | 1.97 |
| --- | --- |
| Load lifting step, m | 0.04 |
| Hoist drive type | mechanical |
| Maximum moment on the drive stick, Nm | 180 |
| Minimum outer turning radius, m | 1.19 |
| Dimensions of the lifted load, mm | 700x700x900 |

The hoist drive is mechanical. A rack jack with a rack length of 1200 mm and a lifting capacity of 2 tons is used as a drive. The lift height is discrete, a multiple of 42 mm. To increase the lifting height up to 1960 mm, the scheme of a classic telescopic frame is used. The extendable frame lifts the load to a predetermined height by means of a traction line. In contrast to the classical model, instead of the traction chain, a static rope "Shturm 12" with a breaking strength of at least 20 kN and a chain hoist system are used. To compensate for the operational elongation, an adjusting lanyard is used in the carriage design.

The technical characteristics of the wool storage equipment are given in the table.

Table 11 - Technical characteristics of the installation (form А.1 STO AIST 32.4-2008)

| Indicator name | Indicator value |
| --- | --- |
| Type | mobile |
| Overall dimensions, mm: - width - length - height | 2438 6058 2896 |
| Bale size | 980x700x700 mm |
| Bale weight | 110 kg |
| Bale density | 250 kg / m 3 |
| The number of wool bales in the container | 56 |
| Number of rows of bales in stack height | 4 |
| Number of bales in 1 row in width |  |
| Number of bales in 1 row in length |  |
| Weight of bales in container | up to 6.5 tons. |
| Fan characteristics: | MAICO ER 60 VZС, WITH TIMER |
| A type | Centrifugal |
| Mounting type | Recessed |
| Thermal overload protection | there is |
| Electrical connection | 220 V / 50 Hz |
| Degree of protection | IPX5 |
| Power consumption, W | 50 |
| Maximum air consumption, m³ / h | 62 |
| Sound pressure level | 36 dBA |
| Weight, kg | 2.0 |
| Container service life, years | 8 |
| Serviceability factor | 0.98 |

The operational and technological indicators include: temperature, relative humidity, air velocity in the container, as well as the state of the surrounding air in which the container is contained. The temperature regime inside the container chamber is regulated by a heat pump (HP). The problem of heating the air in the container chamber can be considered as the process of heating the body of internal heat sources, constant power P. In this case, the following assumptions are made:

- body temperature at any time is the same at all points of the volume ϑ;

- the heat capacity of the body C does not depend on temperature;

- the heat transfer coefficient kто practically does not depend on the excess of temperature and is the same over the entire surface of the body.

During the time ∂τ, the energy generated in the body will be spent on increasing body temperature (C ∙ ∂τ), and part of it (kто∙t∙∂τ) will be released into the environment:

P∙∂τ=C∙∂t+ kто∙S∙∂τ                                       (7)

After appropriate transformations, equation (7) is reduced to the form:

∂t∂τ+kто∙SC ∙t- PC=0

The general solution (7) would be:

t= tуст∙e-τ/T+ Pkто∙S∙(1-e-τ/T)                                              (8)

Or

t= tнач∙e-τ/T+ tуст∙(1-e-τ/T)

where at τ = ∞; t = tуст - steady-state temperature rise when the power released in the chamber becomes numerically equal to the power given to the HP evaporator and into the environment from the container chamber surface (kто∙S∙tуст).

In the process of testing and research, temperature, humidity and air speed inside the container are measured using special devices (Figure 27).

- b)                               c)

- hot-wire anemometer for measuring the volumetric flow rate and air temperature; b) an aspiration psychrometer for measuring air humidity; c) Fluke multifunction infrared thermometer

Figure 27 - Instruments for measuring the microclimate inside the container

Many modern models of electronic anemometers allow measuring not only wind speed (this is the main purpose of the device), but also equipped with additional convenient service functions for calculating: measuring air temperature (hot-wire anemometer); air humidity (hot-wire anemometer with humidity measurement function) (Figure 27a).

The air humidity was measured with an aspiration psychrometer (Figure 27b). The surface temperature was measured with a contact electrothermometer (Figure 27c).

Figure 28 - Graphs of changes in ambient temperatures, air inside the container chamber and the inner surface of the walls on days

control measurements (09.30.19-16.10.19)

Figure 29 - Graphs of changes in the relative humidity of the ambient air and inside the container chamber on the days of control measurements (09.30.19-10.16.19)

Figure 30 - Graphs of the speed of air movement of the environment and inside the container chamber on the days of control measurements (09.30.19-16.10.19)

Figures 28-30 show graphs of the parameters of the external environment (temperature, humidity and air velocity), as well as the air inside the container chamber, as well as the inner surface of the chamber walls, obtained by experimental measurements on the days of control measurements.

Table 12 - Indicators of operational and technological assessment

| Indicators | Units | Indicator value |  |
| --- | --- | --- | --- |
|  |  | Basic technology | when tested |
| Microclimate of the environment |  |  |  |
| Temperature | ° C | 20 | 10-23 |
| Relative humidity | % | 35 | 30-75 |
| Air flow speed | m / s | 0.1 | 0-1.0 |
| Microclimate inside the container chamber |  |  |  |
| Temperature | ° C | 20 | 10-12 |
| Relative humidity | % | 50-60 | 40-50 |
| Air flow speed | m / s | 0.1 | 0.1 |
| Bale density | kg / m 3 | 250 | 250 |
| Duration of loading 1 bale | min | 5.0 | 3.0 |
| Duration of vehicle loading | min | 280 | 170 |
| The percentage of damage to the packaging of bales during loading and unloading operations | % | up to 2 | 0 |
| The percentage of damage to the packaging of bales when stacked in the warehouse | % | up to 2 | 0 |
| Loss of wool during transportation and storage | % | until 3 | 0 |

From the results obtained it follows:

- with changes in the external climate, the parameters of the air environment inside the container chamber are maintained within acceptable limits, without sharp fluctuations;

- air temperature inside the chamber varies within 10-12 ° С, relative air humidity within 40-50%;

- the ventilation system ensures the movement of air flows inside the container chamber at a speed of 0.1 m / s;

- the temperature of the inner surface of the chamber walls varied within 15-30 ° С. The walls were cooled at night. On sunny days, the walls were heated to 30 ° C.

- the duration of loading works is reduced by 60% in comparison with the basic technology;

- damage to the packaging of the bales during loading and unloading operations was not found;

- losses of wool during transportation and storage have not been established.

Reliability tests were carried out in real operation conditions or accelerated according to the current normative document, in real operation conditions of products or as close as possible to them within the values specified in the technical documentation for the product [25]

Table 13 - Indicators of reliability

| Indicators | Units | Indicator value |  |
| --- | --- | --- | --- |
|  |  | According to TR | when tested |
| MTBF | h | - | 160 |
| Number of refusals: |  |  |  |
| I group of difficulty |  | - | 2 |
| II group of difficulty |  | - | - |
| III group of difficulty |  | - | - |
| MTBF: |  |  |  |
| I group of complexity |  | - | 8 |
| II group of difficulty |  | - | - |
| III group of difficulty |  | - | - |
| Labor intensity of every shift maintenance, man-h |  | - | 0.5 |
| Specific labor intensity of current repair | person h | - | 4 |
| Duration of elimination | h | - | 1.5 |
| Average recovery time | h | - | 1,2 |
| Availability ratio |  | 0.98 | 0.998 |

In order to conduct a special assessment of working conditions, the following harmful and dangerous factors of the labor process were subject to testing and measurement:

- the severity of the labor process - indicators of physical load on the musculoskeletal system and on the functional systems of the employee's body;

- tension of the labor process - indicators of sensory load on the central nervous system and the employee's sense organs.

The factors of the working environment were also measured and investigated:

- physical factors - aerosols of predominantly fibrogenic action, noise, infrasound, airborne ultrasound, general and local vibration, non-ionizing radiation (electrostatic field, constant magnetic field, including hypogeomagnetic, electric and magnetic fields of industrial frequency (50 Hertz), alternating electromagnetic fields , including radio frequency range and optical range (laser and ultraviolet), ionizing radiation, microclimate parameters (air temperature, relative humidity, air velocity, infrared radiation), parameters of the light environment (artificial lighting (illumination) of the working surface);

- chemical factors - chemicals and mixtures measured in the air of the working area and on the skin of workers, including some substances of a biological nature (antibiotics, vitamins, hormones, enzymes, protein preparations), which are obtained by chemical synthesis and (or) to control the content of which chemical analysis methods are used;

- biological factors - producer microorganisms, living cells and spores contained in bacterial preparations, pathogenic microorganisms - causative agents of infectious diseases.

Table 14 - Indicators for assessing working conditions

| # | Indicators for TR, SLSS | Indicator value |  |
| --- | --- | --- | --- |
|  |  | according to RTD | under test machine |
| 1 | Type of lifting and transport vehicles | stacker | stacker |
| 2 | The load on the body of working personnel |  |  |
|  | - physical load on the musculoskeletal system and on the functional systems of the employee's body | no more than 50 kg | no more than 50 kg |
|  | - sensory load on the central nervous system and the employee's sense organs | - | no |
| 3 | Workplace microclimate: |  |  |
|  | - air temperature, С | 22-25 | no more than 25 |
|  | - relative humidity, % | 40-60 | no more than 60 |
|  | - air speed, m / s | 0.1 | no more than 0.1 |
| 4 | Illumination of the workplace, lx | not less than 50 | 50-100 |
| 5 | Chemicals and mixtures | - | no |
| 6 | Convenience of technological service | conveniently | conveniently |
| 7 | Number of working movements per minute | 2 | 2 |
| 8 | Working posture | rational | rational |

3.3.3 Development of software and hardware for the climate control system in the container storage chamber for wool bales

This hardware unit is capable of reading data from a DHT22 temperature and humidity sensor using an analogue of the OneWire protocol with an adjustable frequency from 0.5 s to 1 min. Having data on threshold values ​​of temperature and humidity, the device turns on or off the exhaust fans by controlling the built-in relay on the same board through the control port.

The main logic functions are performed by the 8-bit central microprocessor ATMEGA2560. In addition to the central processor itself, the circuit contains ZigBee modules for wireless communication with other devices, which can be connected to synchronize the system operation. The JTAG and ISP interfaces are used to update the central microprocessor software for programming the processor. Port B (PORTB) of the microprocessor is used to read data from the DHT22 temperature and humidity sensor, while port D (PORTD) transmits signals to control the relay.

Figure 31 - Conceptual architecture of the control unit

Figure 32 shows the complete schematic diagram of the central unit.

Figure 32 - Schematic diagram of the central unit

It fully corresponds to the architecture shown above, except for the presence of several LEDs for ease of debugging, processor restart circuits also to simplify debugging, elements to increase reliability and stability (additional capacitors and resistors), a power supply module (stabilizers) for separate power supply of individual components. This diagram was created using the Altium Designer 14.0 tool.

There are more than 50 individual elements in the schematic diagram, a list of which can be seen in Table 2 below.

Based on the developed approved schematic diagram, a schematic model of the control unit was developed, which consists of 2 layers and is shown in Figure 33.

| a) | b) |
| --- | --- |

Figure 33 - Layer 1 (a) and layer 2 (b) of the circuit model

central unit

In the schematic model, you can see the location of all components with symbols, holes and connecting tracks. The dimensions of the future components are shown in yellow, the electrical areas of layer 1 are in red, the electrical areas of layer 2 are in dark blue, the holes are in dark green, the solder area is in gray, and the solder mask is purple. It is worth noting that polygon was applied to layer 1 to reduce the influence of electromagnetic waves on the circuit and simplify the layout of the board.

3.3.4 Analysis and conclusion of test results

Determined that:

- WSE carries out a technological process with indicators corresponding to technical requirements for development;

- container capacity is 56 bales. With a bale weighing 150 kg, the total weight of the loaded wool is 8.5 tons;

- the indisputable advantage is the ability to regulate and control the microclimate, which makes it possible to exclude the vital activity of moths and their larvae;

- the equipment is well suited for quick and convenient loading and unloading of wool bales using a special stacker. Excludes heavy physical labor during unloading and storage of bales. The load on the body of the working personnel, in the form of physical load on the musculoskeletal system and on the functional systems of the employee's body, as well as the sensory load on the central nervous system and the employee's sense organs, meets the requirements of the SLSS.

- significantly speeds up the production of cargo operations. Compared to traditional technology, vehicle loading times are reduced by 60%;

- loss and damage of bales is almost completely excluded. For comparison, with traditional technology, up to 2% damage to the packaging material is allowed during loading and unloading, while wool loss during transportation and storage can reach 3%.

4 Research Feasibility Study

The assessment of the economic efficiency of the studied system of machines (MAIS, MSS and WSE) was carried out on the basis of N.M. Morozov [40, 41] and N.S. Vlasov [42].

The basic options are: a previously used mobile shearing station based on a MSS-12/200 tractor trailer, a standard stationary artificial insemination station (AIS) in operation; a real technology for transportation, storage, sorting and classification of wool today.

As an alternative mobile shearing station, a mobile shearing station (MSS) on the basis of a container (20 tons) folding platform for 10 shearing workplaces is proposed, developed by the Scientific and Production Center of Agroengineering. The assembled platform with the cutting equipment is delivered to the cutting site by a standard container ship, which is used only for the period of transportation of the MSS. A distinctive feature of sheep shearing technology is the presence of a collapsible sheep pen, consisting of four sections, for preliminary distribution of sheep of different breeds according to wool classes before shearing. This will eliminate the attraction of additional labor for sorting and grading wool at a primary wool processing factory (WSP).

The need to breed fine-wooled sheep breeds in order to obtain high-quality wool requires the adaptation of breeding work to the real conditions prevailing in sheep breeding today. To cover this work with scattered personal, peasant and farm households, where more than 90% of the sheep livestock are concentrated in their personal backyards, a mobile artificial insemination station (MAIS) has been developed, which, together with the equipment, is delivered to the location of the ewes.

The next bottleneck in the technology of sheep breeding today is the transportation and storage of wool, which requires temporary storage warehouses (TSW) at procurement points (from resellers) and WPF factories and, accordingly, an increase in the cost of loading and unloading operations. To solve this problem, it is proposed to use a container system for transporting and storing wool both at the sheep shearing station and at the WPF factory. The use of the container system will also allow several times to reduce the cost of loading and unloading operations.

The economic efficiency of the proposed types of technologies and technical means was determined on the basis of comparing direct operating costs for new and basic (existing) options. In order to optimize the presentation of the material, the research results are summarized and presented below in the form of diagrams.

Figure 34 shows the total unit costs and its main components for the basic (MSS-12/200 /) and proposed (MSS) options for sheep shearing technologies.

An analysis of the costs of shearing sheep shows that the use of the proposed MSS allows reducing the total direct operating costs by 32.8%, including depreciation charges by 47.2% and wages by 32%. This is explained by the fact that the book value of the proposed MSS is 29.3% lower than that of MSS-12/200 and provides 10 shearing jobs instead of 12 [31]. It should also be noted that the container ship for the transportation of the proposed MSS is involved only for the period of its redeployment, and the rest of the time it is used for its intended purpose.

Figure 34 - Unit cost of sheep shearing

Knowing that one proposed MSS per season can shear up to 23 thousand heads of sheep, we will determine the economic effect of its use:

тг.            (9)

where: and - respectively, the specific direct operating costs for the MSS -12/200 and the proposed MSS, = 402 tg/head, = 270 tg/head.

m1 = 23000 head - the number of sheep sheared on the proposed MSS for the season [31].

Data on the unit costs of artificial insemination of ewes according to the applied and proposed technologies are shown in Figure 35.

The cost diagrams for artificial insemination for the compared technologies show that the total specific direct operating costs for the proposed artificial insemination technology are up to 20% lower, which is explained by the lower book value of the proposed MAIS and the possibility of using it at the end of the insemination season for other purposes during the calendar year.

Figure 35 - Unit costs for artificial insemination of ewes

The economic effect from the use of one MAIS, provided that during the season it provides insemination of up to 2000 heads of ewes will be:

(603 – 480)*2000 = 246000 tg.             (10 )

Where  and are the specific direct operating costs for an artificial one sheep on a stationary PIP and PIP, respectively. = 603tg/head, =480tg/head.

m2 is the number of ewes inseminated by one MAIS per season, m2 = 2000 heads.

The results of the analysis of the existing technology of wool production made it possible to reveal that in the total unit costs, the share of the costs of loading and unloading and transport operations associated with the delivery of bales of wool to the WPF factory and its storage is quite significant.

The total unit costs for the listed types of work and their components are shown in Figure 36.

Figure 36 - Unit costs for handling, transportation and storage of wool

The use of a container system for transporting and storing wool both at the sheep shearing station and at the WPF factory can significantly reduce the unit costs of its production. So the proposed technology of wool production, due to the preliminary classification of sheep by wool classes, eliminates the need for sorting and classifying at the factory, which currently attracts up to 30 ... 40 people. At the same time, the costs of sorting and grading wool at the factory account for over 40% of the total specific direct operating costs for handling and transport operations, sorting and grading wool at the factory and its storage. Due to the fact that when using the container system, there is no need for intermediate temporary storage warehouses and warehouses at the factory, the costs of loading and unloading decreased by 4.6 times and the costs of transport work by 29%. Accordingly, the total specific direct operating costs for these types of work have also decreased by more than 4 times.

The economic effect from the use of container technology for transporting and storing wool on only one MSS will be:

(186-45) * 23000 = 3243000 tg.                   (11 )

Where and are the specific direct operating costs for handling, transportation and storage of wool with the applied and proposed technologies, respectively. = 186 tg/head, =45tg/head.

Considering that the proposed shearing technology provides for the preliminary separation of sheep according to wool classes before shearing, there is no need for additional sorting (at least 30 people) and classification (2 people) of wool at the PPW factory, which will make it possible to obtain additional savings in the form of their monthly salary in the amount:

Э4 = Зс∙nс + Зк∙ nк = 50000∙30 + 195000 ∙2 = 1890000 tg.          (12)

where Зс and Зк - respectively, the salary of the sorter and the classifier, Зс = 50,000 tenge and Зк = 195,000 tenge;

nс and nк - respectively, the number of working sorters and classifiers, nс = 30 people, nк = 2 people.

Thus, the total annual savings in direct operating costs from the implementation of the proposed modernized technologies and technical means for shearing and artificial insemination of sheep and a container system for transporting and storing wool in conditions of distant sheep breeding will be:

Эг= Э1 + Э2+ Э3+ Э4 = 3036000 + 246000 + 3243000 + 1890000 = 8415000 tg.

Of which the share of savings from the use of a mobile sheep shearing station is 3,036,000 tenge. or 36.07%, a mobile artificial insemination station 24600 tenge. or 2.92% and a container system for transporting and storing wool 3,036,000 tenge. or 38.54%.

In addition, it should be noted that the introduction of the machine system additionally provides for:

- as a result of the corresponding breeding work envisaged by the project, making a profit from the production and sale of high-quality wool, the price of which on the world market reaches $ 18.3 US per kilogram;

- the ecological and economic effect of reducing the trampling (degradation) of pastures by a large livestock of sheep around stationary shearing and insemination points; it is necessary to spend at least $ 40 US to restore only 1 hectare of them in Kazakhstan.

CONCLUSION

1 The Program for the Development of the Agro-Industrial Complex for 2018-2021 sets the tasks to increase the volume of wool production to 37.3 thousand tons / year, to harvest 65% of all wool produced. For these purposes, more than 300 mobile shearing stations and equipment for storing wool and up to 1,500 mobile stations for artificial insemination of sheep will be required.

2 During the reporting period, studies were carried out, parameters were justified, design documentation, technical specifications were developed, prototypes of a mobile artificial insemination station for sheep MAIS-2 of a mobile shearing station (MSS) and wool storage equipment (WSE) were manufactured and tested. Prototypes have been introduced in the farms of the Almaty and Turkestan regions.

3 The technical solutions of MAIS are protected by the RK patent No. 5032 dated 06/12/2020, as well as "Mobile station for artificial insemination of sheep", copyright certificates. Tests have confirmed that the unit is adapted for operational moving, installation and dismantling in pasture conditions, and is transported on a trailer. Operational and technological, functional, energy indicators of MAIS correspond to the indicators of technical requirements and provide conditions for high-quality zooveterinary measures for artificial insemination of sheep. The total weight of MAIS equipment does not exceed 1500 kg. Consists of: an arena heated by a heating boiler, a cabin, a transport trolley, laboratory instruments and instruments. Changeable productivity of the station is up to 30 heads, seasonal, up to 600 heads. For the organization of autonomous power supply, the MAIS is equipped with a photovoltaic station. The item is universal - after the end of the insemination season, it can be used for other important economic purposes. The trailer is used to transport goods (animals, hay, wool). A cabin equipped with a heating boiler as a kitchen, dining room in the pastures.

4 The technical solutions of the MSS obtained in the process of R&D are protected by patents of the Republic of Kazakhstan No. 4610. dated July 18, 2019 "Mobile shearing station for sheep" and No. 4603 dated January 17, 2020 "Sheep shearing machine with a circular knife." Tests have confirmed that the unit is adapted for operational moving, assembly and disassembly in pasture conditions, transported by container ship. The technological and technical indicators of the MSS correspond to the indicators of the technical requirements and provide conditions for the high-quality conduct of veterinarian measures for haircuts and wool preparation. The MSS includes: the main transformable platform, 10-12 workplaces for shearers, classifying tables, a wool press, a stacker trolley for transporting bales. In the transport position, the set is transformed into a container, the disassembled equipment is placed inside the container. Work at the MSS is associated with physical stress on the musculoskeletal system of the working personnel. Tests have confirmed that the working conditions meet the requirements of modern safety standards.

5 The WSE technical solution is protected by the RK patent No. 5295 dated 06/12/2020 "Complex for the transportation and storage of wool", copyright certificate for intellectual property: No. 12430, dated 11/7/2020. Hardware "Control unit for the climate control system in the container for storing bales of wool "And 12432, dated November 7, 2020. Software" Algorithm of work for the climate control system in the container for storing bales of wool. " A set of WSE equipment can consist of 7 containers, adapted for long-term storage of wool bales using heat pump technology. The capacity of one container is 56 bales. With a bale weight of 150 kg, the total weight of loaded wool is 8.5 tons. The equipment is adapted for quick and convenient loading and unloading of wool bales using a special stacker. Excludes heavy physical labor during unloading and storage of bales. The load on the body of the working personnel, in the form of physical load on the musculoskeletal system and on the functional systems of the employee's body, as well as the sensory load on the central nervous system and the employee's sense organs, meets the requirements of the modern safety standards. Compared to traditional technology, vehicle loading times are 60% lower. Loss and damage of bales is almost completely eliminated. For comparison, with the traditional technology, up to 2% damage to the packaging material is allowed during loading and unloading operations, the loss of wool during transportation and storage can reach 3%.

6 The total annual savings in direct operating costs from the implementation of the proposed technologies and technical means will amount to 8,415 thousand tenge, of which the share of savings from the use of MSS, up to 36%, MAIS, up to 3%, and TCR, up to 38.6% and 22, 4% - by reducing the cost of manual labor for sorting wool at PPW.

7 According to the research results, 13 scientific articles were published, of which 8 in the journal from the list recommended by the Science Committee of the Ministry of Education and Science of the Republic of Kazakhstan, 2 in an international journal from the Scopus database, 2 articles were submitted to an international cited journal (examination stages are underway). 4 patents of the Republic of Kazakhstan for invention and 4 certificates for copyright were received. Recommendations on the main results of R&D under the Program were issued. The results were reported and approved at the II Congress of Kazakhstan Sheep Breeders, held by the Ministry of Agriculture of the Republic of Kazakhstan, NASEC, (August 28-29, 2019), at seminars-meetings on the problems of sheep breeding development in Zhanarka District, Karaganda Region (June 10-11, 2019) ) and the K.U. Medeubekov on the topic: "Innovative technologies in sheep breeding" (April 10-12, 2019). Conducted 2 online consultations for farmers on sheep breeding machines.

REFERENCES

1 Message from the President of the Republic of Kazakhstan to the people of Kazakhstan: ("The third modernization of Kazakhstan: global competitiveness", dated January 31, 2017)

2 State program for the development of the agro-industrial complex of the Republic of Kazakhstan for 2017-2021. / http://mgov.kz.

3 Agro-industrial complex maps - a new level of development of the agricultural industry. /http://mgov.kz

4 On veterinary medicine: Law of the Republic of Kazakhstan dated 10.07.2002 No. 339-II (as amended and supplemented as of 29.12.2014) // online.zakon.kz.

5 On state regulation of the development of the agro-industrial complex and rural areas: Law of the Republic of Kazakhstan dated July 8, 2005 No. 66-III (as amended and supplemented as of December 29, 2014) //online.zakon.kz.

6 On livestock breeding: Law of the Republic of Kazakhstan dated July 9, 1998 No. 278-I //mgov.kz.

7 On the protection of breeding achievements: Law of the Republic of Kazakhstan dated July 13, 1999 No. 422 I (as amended and supplemented as of September 29, 2014) // online.zakon.kz.

8 Law of the Republic of Kazakhstan dated February 20, 2017 No. 47-VI "On pastures" / https://online.zakon.kz/Document/?doc_id=34242826#pos=3;-106.

9 Topical issues of the development of pasture animal husbandry in Kazakhstan. Brochure. Published by the Public Fund "Farmer of Kazakhstan". - Almaty, 2016 44p.

10 Development of the wool cluster in Kazakhstan. https://www.youtube.com/watch?v=EZdFepY01Do&feature.

11 In the world market, sheep wool has become the “golden fleece”. https://agrosektor.kz/agriculture-news/na-mirovom-rynke-ovechya-sherst-prevratilas-v-zolotoe-runo.html.

12 Shulenbaeva F., Daribaeva A., Nurlankyzy J. Trends in the sheep breeding industry in the economy of Kazakhstan and Mongolia. Economics and Statistics No. 3, 2016. С 55-64.

13 ANALYSIS OF THE LIVESTOCK INDUSTRY / Analytical Service of the RFCA Rating Agency / http://agrotnk.kz/press-tsentr/ekspertnoe-mnenie/2015/.

14 New Zealand sheep breeding system as the basis of modern farming / http://spcae.kz/index.php?page=publications

15 ANALYSIS OF THE LIVESTOCK INDUSTRY / Analytical Service of the RFCA Rating Agency / http://agrotnk.kz/press-tsentr/ekspertnoe-mnenie/2015/.

16 ST RK state standard R.15.011-2005. System of product development and launching into production. Patent research. Content and procedure. - Introduction. 2006.07.01.- Astana, 2005.- 24 p.

17 ST RK 4.8-2003 Development of technical specifications and technical specifications for the production of agricultural machinery. - Introduction. 2002-12-26.-M .: Standartinform, 2003 .-- 21 p.

18 State Standard 2.601 - 2006 Unified system for design documentation. Operational documents. –M .: IPK Publishing house of standards, 2006. -64s.

19 State Standard R 54783-2011. Agricultural machinery tests. Basic provisions

20 Conducting field trials of agricultural machinery: https://mehanik-ua.ru/lektsii-po-tekhnicheskim-temam/34-provedenie-polevykh-ispytanij-s-kh-tekhniki.html

21 Report on the research work of the Central Scientific and Technological Center of the Ministry of Agriculture of the Republic of Kazakhstan for 2018-2020: “Research and development of technologies and systems of machines for distant sheep breeding. (intermediate). State registration number 0118РК01342, Inv. No. 0218РК01252. Oct 2018

22 State Standard R 52777-2007 Agricultural machinery. Energy assessment methods

23 SP 4282-87 Sanitary rules for the construction of tractors and agricultural machines.

24 State Standard R 52778-2007 Tests of agricultural machinery. Operational assessment methods

25 AIST 2.8-2010 Tests of agricultural machinery. Reliability. Methods for assessing indicators

26 State Standard R 53056-2008 Agricultural machinery. Economic valuation methods.

27 Rules for the preparation, execution and consideration of an application for an industrial design. On approval of the Rules for drawing up, processing and considering an application for an industrial design, entering information into the state register of industrial designs of the Republic of Kazakhstan, as well as issuing a title of protection

28 State Standard 7.32-2001. Report on research work. - Introduction. 2002.07.01.– Minsk: Interstate Council for Standardization, Metrology and Certification, 2001.– 16 p.

29 Khrustalev B.M. Heat supply and ventilation. Design. M .: Publishing house of the association of building universities, 2008. - 784 p.

30 Methodology for calculating and researching the parameters of the heating and ventilation system of a mobile station for artificial insemination of sheep. Research, results. –Almaty. –2019. –No3. –S.400-405. Shynybai Zh.S., Alikhanov D.M., Moldazhanov A.K., Omarov R.A., Omar D.R.

30 Methodology for calculating and researching the parameters of the heating and ventilation system of a mobile station for artificial insemination of sheep. Research, results. –Almaty. –2019. –No3. –S.400-405. Shynybai Zh.S., Alikhanov D.M., Moldazhanov A.K., Omarov R.A., Omar D.R.

31 Patent of the Republic of Kazakhstan for utility model No. 5032. dated 06/12/2020 / Omarov R.A., Kuder K.M., Dosymbekov T.D. Sheep Artificial Insemination Station

32 Patent of the Republic of Kazakhstan for utility model No. 4610. dated 18.07.2019 / Omarov R.A., Isakhanov M.Zh., Kuder K.M. Sheep Shearing Station

33 Patent of the Republic of Kazakhstan for utility model No. 5295 dated 12.06.2020 / Omarov R.A., Isakhanov M.Zh., Kuder K.M. Complex for transportation and storage of wool

34 Patent of the Republic of Kazakhstan for utility model No. 4603 dated 17.01.2020 / Kuzmin Yu. V., Alikhanov DM, Mogilnikov GI. Tsyba Yu.A. "Sheep shearing machine with circular knife."

35 Copyright certificate No. 12430, dated November 7, 2020 / Omar D.R. Hardware "Control unit for the climate control system in the storage container for wool bales"

36 Copyright certificate No. 12432, dated November 7, 2020 / Omar D.R. Software "Algorithm of work for the climate control system in the container for storing bales of wool"

37 Omarov Rashit, Ivaylo Stoyanov, Demessova Saule, Omar Dauren Results of experimental studies of a heat pump with compressor self-cooling. International Journal of Mechanical and Production Engineering Research and Development (IJMPERD) ISSN (P): 2249-6890; ISSN (E): 2249-8001 Vol. 10, Issue 1, Feb 2020, 175–184

38 Copyright certificate for intellectual property: No. 12430, dated November 7, 2020 / Omar D.R. Hardware "Control unit for the climate control system in the container for storing bales of wool"

39 Copyright certificate for intellectual property: 12432, dated November 7, 2020 / Omar D.R. Software "Algorithm of work for the climate control system in the container for storing bales of wool"

40 Morozov N.M. Economic efficiency of complex mechanization of animal husbandry. - M., 1986. - 224 p.

41 Methodology for assessing the economic efficiency of the use of equipment and innovative technologies in animal husbandry / Morozov N.M., Khusainov I.I., Tsoi L.M. and others - Podolsk, 2011. - 99s

42 Vlasov N.S. Methodology for the economic assessment of agricultural machinery. - M., Kolos, 1968. - 223p.

APPENDIX A

Calendar work plan (Russian , English)

Приложение 2 к договору с МСХ РК №23 от «10» сентября 2018 года

Appendix No 2

to Additional Agreement № 1 of «___»___________ 2018 y.

Appendix No 2

to Agreement №23 of  September 10, 2018 y.

Work schedule of «Scientific Production Center of Agricultural Engineering» Ltd on budget program 267 «Improving the availability of knowledge and research» subprogram 101 «Program-targeted financing of scientific research and projects» for 2018-2020 yy.

| No. | Name of work | Duration, year |  | Expected results | Amount, tenge |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 |  |  |
| 1 | Research and development of technologies and machines systems for distant sheep farming | 2018- 2020 yy. |  | Prototypes of a mobile artificial insemination station (MAIS), a mobile shearing station (MSS), and wool storage equipment (WSE) will be made. The technical specifications for the prototypes development, design documentation for the prototypes of MAIS, MSS, WSE will be approved. Acts and economic testing protocols of prototypes are drawn up. The developed machines will be introduced in farms of the Almaty oblast. Application Acts of prototypes MAIS, MSS, WSE in farms are drawn up. 2 applications for the invention will be filed, 4 certificates of intellectual property have been received. 12 scientific articles were published, including 2 articles in peer-reviewed journals included in the Scopus database. Recommendations on the effective MAIS, MSS, WSE use in sheep farms of the Republic of Kazakhstan are published. The final report for 2018-2020 yy. will be presented in accordance with State Standard 7.32-2001 y. | 181 968 000 |  |  |
| 1 | R&D of technologies and machines systems for distant sheep farming. |  | 2018 г. | Programs and research methods for 2018 will be developed and approved. A literature review and patent research on technologies and equipment of a mobile artificial insemination station (MAIS), a mobile shearing station (MSS), and wool storage equipment (WSE) were carried out, research goals and objectives were formulated. The optimal structural and technological schemes will be demonstrated the need by base on the analysis of the literature review and patent research,; the technical requirements for the design of MAIS, MSS, WSE will be developed and approved. Experimental samples sketch drawings of MAIS, MSS, and WSE will be developed. The equipment selection, the purchase of components, materials, devices will be carry out. The annual report for 2018 y. will be presented in accordance with State Standard 7.32-2001. | 60 656 000 |  |  |
| 2 | R&D of technologies and machines systems for distant sheep farming. |  | 2019 y. | Work programs and research methods for 2019 y. will be compiled. Tests of experimental samples will be made and carried out, and test certificates for MAIS, MSS, and WSE will be drawn up. Technical specifications for the design of prototypes have been developed. The design documentation of MAIS, MSS, WSE prototypes will be developed; equipment, components and devices will be purchased. The prototypes production has begun. 3 scientific articles will be published, 1 application for an invention has been filed; 2 certificates of intellectual property have been received. An annual report for 2019 y. will be presented in accordance with State Standard 7.32-2001. |  | 60 656 000 |  |
| 3 | R&D of technologies and machines systems for distant sheep farming. |  | 2020 y. | Work programs and research methods for 2020 y. will be drawn up. MAIS, MSS, WSE prototypes will be manufactured and introduced into production. Application Acts of prototypes are drawn up. A recommendation will be issued on the developed machines using in the farms of the Republic of Kazakhstan. 4 scientific articles will be published, 2 applications for the invention will be submitted, and a PhD dissertation work will be prepared. The annual report for 2020 and the final report for 2018-2020 yy. will be presented in accordance with State Standard 7.32-2001. |  |  | 60 656 000 |

Director of

Strategic Planning Department and Analysis of

the Ministry of Agriculture of the Republic of Kazakhstan 								S.S. Moldabayeva

Acting Director General of

«SPC of Agricultural Engineering» Ltd 											S.A.Keshuov

APPENDIX B

Technical tasks 1

Наименование и область применения

###### 1.1 Передвижной пункт искусственного осеменения овец (ППИО).

###### 1.2 Область применения – овцеводство, личные и фермерские хозяйства, проведение искусственного осеменения овец.

2 ОСНОВАНИЯ ДЛЯ разработки

2.1 Основанием для выполнения НИОКР служат государственный заказ МСХ РК, решение Национального научного совета от 22 августа 2018 года по научно-технической Программе «Исследования и создание технологий и системы машин для отгонного овцеводства».

2.2 Результаты НИОКР выполненные по Программе в 2018-2019 годы

3 Цель и назначение разработки

3.1 Целью является создание передвижного пункта искусственного осеменения овец для ускоренного и эффективного улучшения племенного поголовья овец личных и фермерских хозяйств мясо-шерстных пород.

3.2 Назначение – предназначен для проведения искусственного осеменения овец личных и фермерских хозяйств с выездом в отары, в пастбищных условиях

3.3 Устройство разрабатывается впервые.

4 Источники Разработки

  - 4.1 Материалы литературного обзора, патенты.

  - 4.2 Нормативные документы, справочники, рекомендации.

5 ТЕХНИЧЕСКИЕ ТРЕБОВАНИЯ

5.1 Конструктивное исполнение.

5.1.1 Состав и характеристика комплектующих.

ППИО должен содержать: основное помещения, где должны находиться манеж для проведения основного процесса, лабораторию, загоны для содержания партии отобранных на осеменение овец и баранов, два смежных тамбура для не осемененных и осемененных животных, автоприцеп для перевозки комплекта оборудования ППИО, душевую кабину и кабину туалета для персонала. В центре манежа должен устанавливаться станок для осеменения.

5.1.1.1 Конструкция основного помещения и комплектующего функционального оборудования ППИО должно быть сборно-разборным. Должно рассчитываться на многократный монтаж и демонтаж, сохраняя надежность в эксплуатации.

Для этих целей рекомендуется использовать конструкцию юрты. Юрта хорошо удерживает тепло, ее конструкция устойчива против сильных ветров. Она быстро монтируется и демонтируется и компактна при транспортировке.

В ней должны размещаться манеж, лаборатория, тамбур для осеменяемых овец.

Общая площадь круга, занимаемого юртой, должна быть 28 м2. Площадь лаборатории 8м2, площадь манежа 20м2.

Общая масса комплектующих изделий основного помещения не должно превышать 200-250 кг.

Для обеспечения нормативной освещенности помещения должны быть снабжены окнами. Для обеспечения требуемой освещенности в темное время, помещения манежа и лаборатории должны оснащаться переносными светодиодными светильниками общей мощности 30 Вт.

Напольное покрытие манежа должно обеспечить надежное перемещение животным без скольжений. То есть создавать надежное сцепление ногам животных. Оно также должно легко чиститься от навоза и мочи животных. Для этих целей рекомендуется сетчатый пластик.

Обогреватель с нагревателем  воды должны быть совмещены и работать на дровах и кизяке. Вес отопительного агрегата не должен превышать 80 кг. Температуру в помещении манежа необходимо поддерживать в пределах 18-23 градуса.

5.1.1.2 Лаборатория должна быть оснащена всеми необходимыми приборами, а также средствами санитарной гигиены – раковиной, горячей и холодной водой для мойки приборов, рабочих инструментов.

Для поддержания соответствующих санитарно-гигиенических условий, внутреннее покрытие стен манежа и лаборатории должна легко чиститься при влажной и сухой уборке. Рекомендуется изготавливать покрытие из светлой пластиковой пленки.

5.1.1.3 Загоны и тамбуры должны собираться из модульных сборно-разборных ограждений с воротами для впуска и выпуска животных и калитками для прохождения персонала.

Загон для содержания партии отобранных на осеменение овец и тамбур должны иметь общее ограждение и разделены между собой подвижным ограждением. По мере убывания поголовья первых и прибавления количества осемененных овец она должна перемещаться вручную, обеспечивая равную плотность размещения овец находящихся по обеим сторонам.

5.1.1.4 Душевая кабина и кабина туалета должны устанавливаться на некотором расстоянии от основного помещения, быть удобными для человека, содержать необходимые гигиенические приспособления. Их конструкция должна быть сборно-разборной и удобной при перевозках.

5.1.1.5 Для обеспечения функционирования ППИО должен иметь источник электрической энергии. Он должен обеспечить внутренние потребности в электрической энергии.  Расчетная установленная мощность потребителей электрической энергии составляет около 400 Вт.

Для этих целей может использоваться автономный электрогенератор с приводом от ДВС или фотоэлектрические солнечные модули (ФЭСМ).

В состав ФЭСМ входят:

- солнечные панели по 100 Вт - 4 шт;

- необслуживаемые аккумуляторные батареи специально для ФЭС (12 В, 80 Ач.) - 1 шт.;

- инвертор – 500 Вт, контроллер (10 А; 12/24В).

Вес комплекта составляет около 150 кг.

Общий вес комплектующих изделий ППИО не должен превышать 1500 кг. Для транспортировки ППИО необходим одноосный автоприцеп грузоподъемностью в пределах 1,5 тонны.

5.2 Состав продукции – передвижной пункт искусственного осеменения овец в условиях отгонного овцеводства.

5.3 Основные параметры ППИО:

| № | Показатель | Ед. измер. | значение |
| --- | --- | --- | --- |
| 1 | Сменная производительность, | голов | до 30 |
| 2 | Сезонная производительность, | голов | 500-1000 |
| 3 | Температура помещения для осеменения | град | 15-20 |
| 4 | Диаметр кругового помещения | м | 6 |
| 5 | Площадь кругового помещения | кв. м | 28 |
| 6 | Габариты кабины | м | 3,0×2,0× 2,0 |
| 7 | Масса оборудования | т | до 1,5 |
| 8 | Грузоподъёмность автоприцепа | т | 1,5 |
| 9 | Номинальная мощность ФЭСМ | Вт | 400 |
| 10 | Номинальная мощность солнечной панели | Вт | 100 |
| 11 | Количество солнечных панелей | шт | 4 |
| 12 | Емкость аккумулятора | А·ч | 80 |
| 13 | Номинальная мощность инвертора | Вт | 500 |
| 14 | Напряжение инвертора на входе, | В | 12/24 |
| 15 | Напряжение инвертора на выходе | В | 220(+2% -10%) |
| 16 | Масса ФЭСМ | кг | Не более 150 |

5.4 Требования к технологичности.

Комплектующие изделия должны легко поддаваться ремонту и замене в случае выхода из строя.

5.5 Требования безопасности и требования по охране природы.

Конструкция ППИО должна отвечать системе стандартов по безопасности труда и требований пожарной безопасности ГОСТ 12.1.004-91.

5.6 Эстетические и эргономические требования.

ППИО должна отвечать требованиям промышленной эстетики.

Конструкция должна обеспечивать удобство обслуживания и эксплуатации.

5.7 Требования к патентной чистоте.

Конструкция ППИО и его компоненты должны обладать патентной чистотой в отношении Республики Казахстан.

5.8 Требования к составным частям продукции, сырья, исходным и эксплуатационным материалам.

Материалы ППИО и комплектующие изделия должны быть пожаробезопасными, стойки к воздействию атмосферных осадков.

5.9 Условия эксплуатации

ППИО должен отвечать требованиям ГОСТ 31343-2007. санитарно-профилактическим и технологические требования.

ППИО предназначен для работы в условиях, соответствующих исполнению «У» и «ХП», по ГОСТ 12.1.004-91.

Режим работы – сезонный.

5.10 Требования к транспортированию и хранению

ППИО транспортирует одноосный автоприцеп размерами 1,5х3 м, грузоподъемностью 1,5 тонны и должен соответствовать правилам перевозки грузов, действующих на этих видах транспорта.

6. ЭКОНОМИЧЕСКИЕ ПОКАЗАТЕЛИ

6.1 Ожидаемый годовой экономический эффект от внедрения до 1,0 млн. тенге

6.2 Срок окупаемости до 2,0 лет.

Примечание: экономический эффект от применения ППИО будет уточняться по результатам испытаний.

7 СТАДИИ И ЭТАПЫ РАЗРАБОТКИ

7.1 Разработка и утверждение технического задания 2019 г.

7.2 Разработка конструкторской документации на изготовление экспериментального образца 2020 год.

7.3 Изготовление экспериментального образца 2019 г.

7.4 Проведение производственных испытаний 2020 г.

7.5 Корректировка конструкторской документации по результатам производственных испытаний 2020 г.

7.6 Предприятие-изготовитель опытных образцов ТОО "НПЦ АИ".

8 ПОРЯДОК КОНТРОЛЯ И ПРИЕМКИ

8.1 При проектировании и разработке чертёжной документации должны выполняться требования ГОСТ 2.001 – 2.111 ЕСКД. Конструкторская документация утверждается уполномоченным лицом.

8.2 Разработка чертёжной документации должна базироваться на современных принципах проектирования с использованием программ автоматического проектирования и компьютерной графики.

8.3 При проектировании должно обеспечиваться использование современных материалов, энерго- и ресурсосберегающих технологий изготовления узлов, деталей и продукции в целом.

8.4 Контроль качества изготовления должен осуществляться изготовителем в процессе выполнения контрольных операций, являющихся частью технологического процесса изготовления в соответствии с требованиями настоящего технического задания.

8.5 По результатам испытаний должен быть оформлен акт.

8.6 Результаты испытаний должны быть оформлены протоколом

8.7 Вид производства – единичный.

8.8 Предполагаемое предприятие-изготовитель – ТОО "НПЦ АИ".

Научный руководитель Программы,

д-р техн. наук							Р.А. Омаров

Руководитель мероприятия,

к.т.н, профессор	                                                            Д.М. Алиханов

Заведующий лаборатории

проектирования с.-х. техники, к.т.н.                             Д.К. Карманов

1 Наименование и область применения

###### 1.1 Передвижной стригальный пункт (ПСП).

###### 1.2 Область применения – овцеводство, личные и фермерские хозяйства, проведение стрижки овец.

2 ОСНОВАНИЯ ДЛЯ разработки

2.1 Основанием для выполнения НИОКР служат государственный заказ МСХ РК, решение Национального научного совета от 22 августа 2018 года по научно-технической Программе «Исследования и создание технологий и системы машин для отгонного овцеводства».

2.2 Результаты НИОКР выполненные по Программе в 2018-2019 годы

3 Цель и назначение разработки

3.1 Целью является создание передвижного стригального пункта для стрижки овец в условиях личных и фермерских хозяйств.

3.2 Назначение – предназначен для проведения стрижки овец личных и фермерских хозяйств с выездом в отары, в пастбищных условиях

3.3 Устройство разрабатывается впервые.

4 Источники Разработки

  - 4.1 Материалы литературного обзора, патенты.

  - 4.2 Нормативные документы, справочники, рекомендации.

5 ТЕХНИЧЕСКИЕ ТРЕБОВАНИЯ

  - Конструктивное исполнение

5.1.1 Состав и характеристика оборудования

ПСП (рисунок 1) содержит контейнерную раскладывающуюся платформу, боковые стороны которой используется для рабочих мест стригалей и заточника, а на платформе размещены стригальные машинки и шерстопрессы.

В состав входят: сборно-разборный загон для овец 1, состоящий из 4-х секций, для предварительного распределения овец по классам шерсти, с центральной секцией 2, откуда животные по 2-м расколам-проходам 3 направляются на стрижку, основная площадка 4, стригальные машинки 5, классировочные столы 6, шерстопресс 7, штабелер тюков 9, автономная электростанция 10, юрта для персонала 11, душевая кабина 12, биотуалет 13. Грузовой контейнер 8 для транспортировки и хранения прессованных тюков устанавливается рядом и стыкуется с основным контейнером, так чтобы площадки контейнеров находились на одном уровне. При этом, совместно с центральной площадкой, образуется единая рабочая зона для выполнения классировки, прессования шерсти, а также перемещения и погрузки тюков грузовой контейнер.

5

9

6

13

8

7

10

3

4

4

4

1

2

12

11

Рисунок 1 – Схема передвижного стригального пункта

Основная площадка состоит из центральной платформы, боковых, передней и задней частей. В транспортном состоянии боковые, передние и задние стенки поднимаются и закрепляются в вертикальном положении. В рабочем состоянии стенки раскладываются в стороны и устанавливаются в горизонтальном положении, где под каждую стенку устанавливаются опорные стойки. При этом, совместно с центральной платформой они образуют единую рабочую зону. Крыша контейнера остается в своем первоначальном положении и служит в качестве навеса.

Расколы-проходы 3 начинаются с центральной секции загона и устанавливаются вдоль боковых стенок платформы 4. При этом, между ними оставляется продольная площадка, на которой располагаются рабочие места стригалей, со стригальными столами и машинками. В расколах-проходах напротив каждого места имеется калитка, через которую овца поступает на стрижку. Стригальные машинки получают электропитание от автономной электростанции 10, к которой также подключаются другие электроприборы.

Работа стригального пункта осуществляется следующим образом.

В контейнер загружается все необходимое оборудование, включая: секции сборно-разборного ограждения загонов для овец, стригальные машинки, станок для заточки ножей машинок, классировочные столы, шерстопрессы, штабелер тюков, которые доставляются до места дислокации автомобилем.

По прибытии в точку дислокации стригальный пункт разворачивается и приводится в рабочее состояние. Снимаются с фиксаторов и приводятся в горизонтальное положение боковые стенки контейнера, которые совместно с центральной площадкой образуют рабочую зону. Под ними устанавливаются соответствующие опорные стойки. Задняя стенка откидывается и стыкуется с задней стороной контейнера 8, который устанавливается, напротив, с предварительно открытыми задними дверями. На образованную задней стенкой площадку устанавливаются шерстопрессы 7.

Затем устанавливается вся ограждающая конструкция, в том числе внешняя ограда загона для овец, внутренние перегородки между секциями, расколы-проходы 3. Устанавливается автономная электростанция 10, прокладываются проводки для питания стригальных машинок.

По предварительно составленному графику отары овец пребывают на пункт. Овец загоняют в загон с разделением их на 4 секции по качеству шерсти. Из секций овцы предварительно поступают в центральную секцию и далее направляются по расколам 3, где они, перемещаясь по проходу, постепенно выстраиваются в цепочку.

Стрижка должна осуществляться на земле, рядом со стригальной площадкой. Стригали (10 человек) должны располагаться параллельно боковой кромке платформы, на площадке между платформой и расколами, на расстоянии удобном для выполнения стрижки и подачи состриженного руна на площадку. Поэтому, расстояние между стригалем и расколом, а также между стригалем и платформой должны быть на расстоянии «вытянутой руки».

Состриженную шерсть забирают классировщики. После обработки на классировочном столе 6 шерсть складывается в специальные накопители по сортам. Прессовщики, используя шерстопрессы 7 осуществляют прессование ее в тюки. Готовые тюки при помощи штабелера 9 загружаются в контейнер 8. После заполнения контейнер доставляется на фабрику первичной обработки, где тюки хранятся до их обработки. При этом, в отличие от известной технологии, из технологической цепочки исключаются склад для хранения шерсти, перегрузочные работы.

    - Основная стригальная платформа изготавливается на базе контейнера грузоподъемностью 20 тонн.  В транспортном состоянии в контейнере помещаются все перечисленное оборудование ПСП.

В рабочем состоянии основной контейнер и грузовой контейнер располагается на ровной площадке, устанавливаются на собственные опорные стойки высотой не менее 0,8 м.

В транспортном состоянии задняя и боковые стенки закрепляются в вертикальном положении. Задняя стенка, в рабочем положении, служит одновременно платформой-трапом соединяющейся с грузовым контейнером. В рабочем положении крыша  остается в первоначальном положении и служит в качестве навеса.

5.2 Показатели назначения

Производительность, гол/ч                                          до 100

Число электростригальных машинок, шт                     10-12

Численность обслуживающего персонала, чел             16

5.3 Требования к надежности

Коэффициент готовности, не менее                             0,98

Срок службы, лет                                                          8

5.4 Требования к технологичности

Конструкция ПСП должна быстро и удобно собираться и разбираться, легко поддаваться ремонту и замене в случае выхода из строя.

5.5 Требования безопасности и требования по охране природы

Конструкция ПСП должна отвечать системе стандартов по безопасности труда и требований пожарной безопасности ГОСТ 12.1.004-91.

5.6  Эстетические и эргономические требования

ПСП должен отвечать требованиям промышленной эстетики.

Конструкция установки должна обеспечивать удобство обслуживания и эксплуатации.

5.7 Требования к патентной чистоте

Конструкция установки должна обладать патентной чистотой в отношении Республики Казахстан.

5.8 Требования к составным частям продукции, сырью, исходным и эксплуатационным материалам.

Материалы ППИО и комплектующие изделия должны быть пожаро-безопасными, стойки к воздействию атмосферных осадков.

5.9 Условия эксплуатации

ПСП должен отвечать требованиям ГОСТ 31343-2007.

Установка предназначена для работы в условиях, соответствующих исполнению «У» и «ХП», по ГОСТ 12.1.004-91.

Сезон работы – в течение стригального периода.

5.10 Требования к транспортированию и хранению

Установка транспортируется автомобильным транспортом в соответствии с ГОСТ 105.006-82 и правилами перевозки грузов, действующих на этих видах транспорта.

6. ЭКОНОМИЧЕСКИЕ ПОКАЗАТЕЛИ

6.1 Ожидаемый годовой экономический эффект от внедрения до 5,0 млн. тенге

6.2 Срок окупаемости до 2,0 лет.

Примечание: экономический эффект от применения ПСП будет уточняться по результатам испытаний.

7 СТАДИИ И ЭТАПЫ РАЗРАБОТКИ

7.1 Разработка и утверждение технического задания 2019 г.

7.2 Разработка конструкторской документации на изготовление экспериментального образца 2020 год.

7.3 Изготовление экспериментального образца 2019 г.

7.4 Проведение производственных испытаний 2020 г.

7.5 Корректировка конструкторской документации по результатам производственных испытаний 2020 г.

7.6 Предприятие-изготовитель опытных образцов ТОО "НПЦ АИ".

8 ПОРЯДОК КОНТРОЛЯ И ПРИЕМКИ

8.1 При проектировании и разработке чертёжной документации должны выполняться требования ГОСТ 2.001 – 2.111 ЕСКД. Конструкторская документация утверждается уполномоченным лицом.

8.2 Разработка чертёжной документации должна базироваться на современных принципах проектирования с использованием программ автоматического проектирования и компьютерной графики.

8.3 При проектировании должно обеспечиваться использование современных материалов, энерго- и ресурсосберегающих технологий изготовления узлов, деталей и продукции в целом.

8.4 Контроль качества изготовления должен осуществляться изготовителем в процессе выполнения контрольных операций, являющихся частью технологического процесса изготовления в соответствии с требованиями настоящего технического задания.

8.5 По результатам испытаний должен быть оформлен акт.

8.6 Результаты испытаний должны быть оформлены протоколом

8.7 Вид производства – единичный.

8.8 Предполагаемое предприятие-изготовитель – ТОО "НПЦ АИ".

Научный руководитель Программы,

д-р техн. наук							Р.А. Омаров

Руководитель мероприятия,

к.т.н, профессор	                                                      М.Ж. Исаханов

Заведующий лаборатории

проектирования с.-х. техники, к.т.н.                        Д.К. Карманов

1 Наименование и область применения

###### 1.1 Оборудование хранения шерсти (ОХШ).

###### 1.2 Область применения – овцеводство, личные и фермерские хозяйства, фабрика первичной обработки шерсти, транспортировка и хранение шерсти в тюках.

2 ОСНОВАНИЯ ДЛЯ разработки

2.1 Основанием для выполнения НИОКР служат государственный заказ МСХ РК, решение Национального научного совета от 22 августа 2018 года по научно-технической Программе «Исследования и создание технологий и системы машин для отгонного овцеводства».

2.2 Результаты НИОКР выполненные по Программе в 2018-2019 годы

3 Цель и назначение разработки

3.1 Целью является создание оборудования транспортировки и хранения тюков шерсти, снижающее потери шерсти в процессе транспортировки и последующего хранения на фабрике, а также снижение трудоемкости перегрузочных работ на фабрике.

3.2 Назначение – предназначен для транспортировки и хранения тюков шерсти.

3.3 Устройство разрабатывается впервые.

4 Источники Разработки

  - 4.1 Материалы литературного обзора, патенты.

  - 4.2 Нормативные документы, справочники, рекомендации.

5 ТЕХНИЧЕСКИЕ ТРЕБОВАНИЯ

5.1 Конструктивное исполнение.

5.1.1 Состав и характеристика комплектующих

Оборудование состоит из грузового контейнера общего назначения. Камера контейнера должна вмещать 56 прессованных тюков шерсти размером: 980х700х700 см. Влажность загружаемых тюков не должна превышать 17%.

На пункте стрижки, в процессе транспортировки и на фабрике ПОШ контейнеры будут располагаться на открытом воздухе. Длительность процесса хранения может составлять несколько месяцев, включая осенний, зимний периоды. Для исключения порчи шерсти не допускается промокание и переувлажнение шерсти. Влажность в камере контейнера должна поддерживаться в пределах  65-70%.

Для исключения хищений контейнеры должны закрываться на замок.

Для дистанционного контроля состояния шерсти контейнеры должны быть оснащены датчикам температуры и влажности.

Для поддержания температурно-влажностного состояния внутреннего воздуха камера должна снабжаться естественной и искусственной вентиляцией.

5.2 Основные параметры определяются исходя из выбранного по ГОСТу типоразмера контейнера.

5.3 Требования к технологичности

ОХШ должно быстро и удобно транспортироваться, обеспечивать требования к хранению шерсти в условиях отгонного овцеводства. Комплектующие изделия должны легко поддаваться ремонту и замене в случае выхода из строя.

5.4 Требования безопасности и требования по охране природы

Конструкция ОХШ должна отвечать системе стандартов по безопасности труда и требований пожарной безопасности ГОСТ 12.1.004-91.

5.5 Эстетические и эргономические требования

Конструкция должна обеспечивать удобство обслуживания и эксплуатации.

5.6 Требования к патентной чистоте

Конструкция и ее компоненты должны обладать патентной чистотой в отношении Республики Казахстан.

5.7 Условия эксплуатации. ОХШ должен отвечать требованиям ГОСТ Р 53399-2009. Режим работы – круглогодичный.

6. ЭКОНОМИЧЕСКИЕ ПОКАЗАТЕЛИ

6.1 Ожидаемый годовой экономический эффект от внедрения до 3,0 млн. тенге

6.2 Срок окупаемости до 2,0 лет.

Примечание: экономический эффект от применения ОХШ будет уточняться по результатам испытаний.

7 СТАДИИ И ЭТАПЫ РАЗРАБОТКИ

7.1 Разработка и утверждение технического задания 2019 г.

7.2 Разработка конструкторской документации на изготовление экспериментального образца 2020 год.

7.3 Изготовление экспериментального образца 2019 г.

7.4 Проведение производственных испытаний 2020 г.

7.5 Корректировка конструкторской документации по результатам производственных испытаний 2020 г.

7.6 Предприятие-изготовитель опытных образцов ТОО "НПЦ АИ".

8 ПОРЯДОК КОНТРОЛЯ И ПРИЕМКИ

8.1 При проектировании и разработке чертёжной документации должны выполняться требования ГОСТ 2.001 – 2.111 ЕСКД. Конструкторская документация утверждается уполномоченным лицом.

8.2 Разработка чертёжной документации должна базироваться на современных принципах проектирования с использованием программ автоматического проектирования и компьютерной графики.

8.3 При проектировании должно обеспечиваться использование современных материалов, энерго- и ресурсосберегающих технологий изготовления узлов, деталей и продукции в целом.

8.4 Контроль качества изготовления должен осуществляться изготовителем в процессе выполнения контрольных операций, являющихся частью технологического процесса изготовления в соответствии с требованиями настоящего технического задания.

8.5 По результатам испытаний должен быть оформлен акт.

8.6 Результаты испытаний должны быть оформлены протоколом

8.7 Вид производства – единичный.

8.8 Предполагаемое предприятие-изготовитель – ТОО "НПЦ АИ".

Научный руководитель Программы,

д-р техн. наук							Р.А. Омаров

Руководитель мероприятия,

к.т.н, профессор	                                                            М.Ж. Исаханов

З

аведующий лаборатории

проектирования с.-х. техники, к.т.н.                             Д.К. Карманов

APPENDIX C

Acts of economic tests of installations

APPENDIX D

Protocols of economic tests of installations

Министерство сельского хозяйства Республики Казахстан

НАО «Национальный аграрный научно-оброзовательный центр»

ТОО «научно-производственный центр АГРОИНЖЕНЕРИИ»

ЛАБОРАТОРИЯ ВОЗОБНОВЛЯЕМЫХ ЭНЕРГОРЕСУРСОВ И ВОДОСНАБЖЕНИЯ

Протокол № 01

От «15» октября 2019 г.

Хозяйственных испытаний

передвижного пункта искусственного осеменения овец (ППИО)

с. Мынбаев, 2019

Введение

| Заводской номер | Год изготов-ления | Дата поступления на испытания |  | Период испытаний | Объем работ, сут. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | по плану | факти-чески |  | по плану | факти-чески |
| 001 | 2008 | 10.10. 2019. | 15.09. 2019. | с 15. 09.19 по 15.10.19 | 30 | 30 |

Организация – разработчик – ТОО «Научно-производственный центр агроинженерии»	НАО «НАНОЦ»

Министерство – ведомство – МСХ РК

Испытания проводились в соответствии СТО АИСТ 32.4-2008 (Стандарта организации) Испытания сельскохозяйственной техники «КОМПЛЕКТЫ МАШИН И ОБОРУДОВАНИЯ ДЛЯ СОДЕРЖАНИЯ ОВЕЦ И КОЗ» Методы оценки функциональных показателей

- Характеристика испытываемого образца

  - Назначение и технические описание машины.

Проведение полных хозяйственных испытаний ППИО запланировано на ноябрь месяц в К/Х «Сенім» Кегенского района Алматинской области при осеменении отары из 500 овцематок. В настоящем отчете приведены результаты предварительных испытаний, которые проводились в НИИ овцеводства им. К.У. Медеубекова в течение октября месяца. Общий вид (а) и схема расположение оборудования ППИО (б) приведены на рисунке 1.

а)

б)

Рисунок 1 – Общий вид и схема расположение оборудования ППИО

Пункт искусственного осеменения состоит из манежа 1, лаборатории 2, загонов для содержания партии отобранных на осеменение овец 3 и баранов 4, двух тамбуров для не осемененных 5 и осемененных животных 6, автоприцепа 7 для перевозки комплекта оборудования пункта искусственного осеменения, душевой кабины 8 и кабины туалета 9. Для поддержания соответствующих санитарно-гигиенических условий, внутреннее покрытие стен манежа и лаборатории изготавливаются из светлой пластиковой пленки. Это позволит ежедневно чистить и протирать поверхность покрытия. Помещения снабжаются боковыми окнами 10, обеспечивающими необходимое естественное освещение для выполнения работ. В манеже, лаборатории, кабинах туалета и душевой предусмотрено светодиодное освещение. Загоны и тамбуры снабжаются сборно-разборными ограждениями 11 и воротами 12 для прохождения персонала, впуска и выпуска животных. Загон для содержания партии отобранных на осеменение овец 3 и тамбур 5 имеют общее ограждение и разделены между собой подвижной перегородкой 13. В центре манежа устанавливается станок для осеменения 14. По мере убывания поголовья первых и прибавления количества осемененных овец она перемещается, обеспечивая равную плотность размещения овец.  Для электроснабжения источников освещения, приборов и бытовой техники предусмотрен источник электрической энергии 15 и обогреватель с нагревателем воды 16.

1.2. Описание технологического процесса

Работа пункта искусственного осеменения осуществляется следующим образом. Автоприцеп 7 подвозит комплектующее оборудование пункта искусственного осеменения к месту дислокации отары. После этого осуществляется сборка и приведение в рабочее состояние: основного манежа 1, лаборатории 2 с комплектующими приборами, загонов 3 и 4, тамбуров для не осемененных 5 и осемененных животных 6, душевой кабины 8 и кабины туалета 9, окон 10, ограждения 11 и ворота 12 для загонов и тамбуров, подвижной перегородки между загонами 13, станка для осеменения 14, источника электрической энергии 15 и обогревателя с нагревателем воды 16.

Перед проведением основных мероприятий включается в работу обогреватель с нагревателем воды 16, осуществляется прогрев основных помещений. Тепловая мощность обогревателя рассчитывается из условия поддержания температуры внутри помещения манежа на уровне 20 0С. Осуществляется монтаж источников освещения и приборов в лаборатории и манеже и подключается к источнику электроэнергии 15.  Утром, перед выгоном животных на пастбища, техники-осеменаторы отбивают от стада овец пришедших в охоту и собирают их в загоне 3. При поголовье одной отары в 500 голов, ежедневное среднее количество таких овец, доходит до 30 - 40. Из этой партии 10-15 голов заводят в тамбур для содержания осеменяемых овец 6

Предварительно у баранов производителей, располагающихся в загоне 4, забирают сперму, которая хранится в лаборатории 2, при соответствующей теплой температуре. После этого осеменяемую овцу заводят в манеж и помещают в специальный станок 14. При этом внутренняя температура воздуха в манеже должна поддерживаться в пределах 18-23 градуса. Осеменатор, используя специальные инструменты, выполняет процесс осеменения. После этого овцу выводят в тамбур осемененных животных 6. Далее, отбирают сперму со второго барана и цикл осеменения партии овец повторяется.

  - Техническая характеристика ППИО

Техническая характеристика ППИО приведена в таблице 1 (форма А.1 ГОСТ 31343-2007)

Таблица 1 –Техническая характеристика ППИО

| № | Показатель | Ед. измер. | значение |
| --- | --- | --- | --- |
| 1 | Сменная производительность, | голов | до 30 |
| 2 | Сезонная производительность, | голов | 500-1000 |
| 3 | Температура помещения для осеменения | °С | 15-20 |
| 4 | Диаметр кругового помещения | м | 6 |
| 5 | Площадь кругового помещения | кв. м | 28 |
| 6 | Габариты кабины | м | 3,0×2,0× 2,0 |
| 7 | Масса оборудования | т | до 1,5 |
| 8 | Грузоподъёмность автоприцепа | т | 1,5 |
| 9 | Номинальная мощность ФЭСМ | Вт | 400 |
| 10 | Номинальная мощность солнечной панели | Вт | 100 |
| 11 | Количество солнечных панелей | шт | 4 |
| 12 | Емкость аккумулятора | А·ч | 80 |
| 13 | Номинальная мощность инвертора | Вт | 500 |
| 14 | Напряжение инвертора на входе, | В | 12/24 |
| 15 | Напряжение инвертора на выходе | В | 220(+2% -10%) |
| 16 | Масса ФЭСМ | кг | Не более 150 |
| 17 | Трудоемкость монтажа, чел.-ч | Час. | Не более 6 |

- Техническая оценка

2.1. Условия испытаний

Передвижной пункт искусственного осеменение овец был смонтирован на полигоне НИИ овцеводства. Для осуществления работ по искусственному осеменению овцематок подопытных отар пункт укомплектован чемоданом осеменатора мелких животных с набором инструментов и материалов, представленных в таблице 2.

Таблица 2 – Инструменты и материалы по осеменению овец

| № | Наименование предметов | Спецификация | Кол-во |
| --- | --- | --- | --- |
| 1 | Зеркало влагалищное для овец | Металлическое | 2 |
| 2 | Зеркало влагалищное для ярок | Металическое | 2 |
| 3 | Шприц-катетер для цервикального осеменения (полуавтомат, в комплекте) | Стеклянный | 2 |
| 4 | Цилиндр к искуственной вагине | Эбонитовый | 2 |
| 5 | Камера к искусственной вагине | Резиновая | 6 |
| 6 | Краник к искусственной вагине | Полиэтиленовый | 8 |
| 7 | Пробки к краникам | Резиновые | 8 |
| 8 | Спермоприемник | Стеклянный | 8 |
| 9 | Подставка для спермоприемника | Пластмассовая | 2 |
| 10 | Подставка для инструментов | Пластмассовая | 2 |
| 11 | Подставка для искусственной вагины | Пластмассовая | 2 |
| 12 | Термометр для искусственной вагины | ТС-2 | 2 |
| 13 | Термометр | Комнатный | 2 |

Результативность проводимых работ оценивается по оплодотворяемости овцематок от двухкратного осеменения в день, путем выбора маток в «охоте», с промежутком 6-7 часов (утром и после обеда).

Окончательная оценка результативности проведенных мероприятий по  плодовитости овцематок сравниваемых отар осуществляется в конце окотной компании.

При оценке эффективности результатов искусственного осеменение овец с использованием экспериментального образца ППИО также будут учтены показатели оплодотворяемости и плодовитости маток данного племенного хозяйства за последние 3 года и аналогичные средние параметры по региону и республике.

Основное помещение – юрта, выполнена в виде цилиндра, над цилиндром образован купол юрты в виде усеченного конуса. Имеет полную высоту 3.0 м, где высота основания, цилиндрической части, составляет 1,7 м., а высота купольной, конусной части 1,3 метра. Диаметр основания 6 м., диаметр шанырака 1,5 метра. По известным формулам расчитывается расчетная суммарная площадь поверхности ограждения (S), которая равна 47,31 кв.м. Объем помещения юрты (V) - 64.11 куб.м.

По зоотехническим нормам температура манежа должна поддерживаться в пределах 18-23 градуса, независимо от изменения внешней температуры. Температурный режим во время проведения ИО регулируется путем отопления от печки. В холодные дни отопление начинают с 5-6 часов утра, с учетом времени выборки маток «в охоте» и начала проведения ИО.

В осеменении участвуют техник-осеменатор и два подсобных рабочих, подающие ему маток «в охоте», а также учетчик по осеменению. Одновременно в помещении могут находится до 30 голов маток и 4 человека.

При расчете теплового баланса помещения, определяющего тепловую нагрузку QP0T   на систему отопления, учитываются теплопотери: через ограждения Q О, на нагревание инфильтрационного воздуха Q ЕВ, и воздуха, поступающего в помещения через проемы дверей и шанырак, в соответствии санитарной нормой вентиляции, а также бытовые тепловыделения от людей и животных Qб  [10].

| QP0T =QО + Q(Е,В) - Qб, Вт | (1) |
| --- | --- |

Расчет теплопотерь через поверхности ограждения, производится по известной формуле [11]:

| QО=K۰S۰(tВ-tНБ)۰n۰η, Вт | ((2) |
| --- | --- |

Где:  К – коэффициент теплопередачи ограждения, Вт/(м2°С);

S – расчетная площадь поверхности ограждений, м2;

tв – температура внутреннего воздуха в помещении, (18-23°С);

tНБ – расчетная температура наружного воздуха, °С;

n – поправочный коэффициент к рабочей разности температур;

η – коэффициент, учитывающий дополнительные теплопотери.

Коэффициент η рассчитывается по формуле [10]:

η=1+Р100                                              (3)

Сумму дополнительных потерь тепла через ограждения, принимаются в процентах к основным теплопотерям.

Основные потери тепла происходят в щелях проходов (дверей) и загонов.

При этом, учитываются следующие дополнительные теплопотери:

- для ограждений, ориентированных на С, С-В и С-З – 10% от основных теплопотерь; 5% - для ограждений, ориентированных на 3, ЮВ; для ограждений, ориентированных на Ю, ЮЗ – 0%;

- для наружных двойных дверей с тамбуром при высоте помещения (Н), равный 1,7 м, учитываем дополнительные теплопотери, которые составляют 27Н,%.

Тогда:

η=1+10+5+3+45,9100=1,64

Для определения теплопотерь через наружные ограждения расчитывается термическое сопротивление утеплителя по формуле:

R0=Rв+R1+R2+R3+R4+…Rn=1αв+δ1λ1+δ2λ2+δ3λ3+δ4λ4+…δnλn    (4)

где: Rв–сопротивление теплообмену на внутренней поверхности;

- коэффициент теплоотдачи внутренней поверхности стены Вт∕(м2С);

R1,2…n – сопротивления теплообмена каждого слоя;

δ – толщина слоя ограждения, м

λ –теплопроводность слоя ограждения, Вт/(м*˚С).

Так, как юрта сделана из одного слоя войлока толщиной δ1=0,01 м. λ1=0,04 Вт/(м*˚С), тогда:

R0=Rв+R1=1αв+δ1λ1=18,7+0,010,04=0,36 м2*°С/Вт

Определяем коэффициент теплопередачи:

K=1R0=10,36=2,77Втм2×∘С

Тепловые потери, обусловленные естественной вентиляцией определяются формулой [4]:

Qев=0,278∙Кв∙V∙cp∙ρ∙tв-tн ,                          (4)

где: Кв– кратность воздухобмена; V– объем помещения, м3;                     cp–  удельная теплоемкость воздуха 1.0 кДжкг С°;

ρ –плотность воздуха, (1,225кгм3).

Требуемое количество тепла на нагрев воздуха в помещении манежа вычисляем по формуле:

| Q = V * ΔT *· k , ккал/час, | (5) |
| --- | --- |

где: V - объём помещения;

∆T -  градиент температуры или разница между внутренней температурой манежа (принимает +20 град) и температурой окружающей среды (принимаем минус 10 град).

k – коэффициент теплопотерь (для войлока 3,84 кВт/град۰ч)

Q = 64,1*30*3,84 = 7384,3 ккал/час. или 8,6 кВт/ч.

Для обеспечения запаса мощности в случае понижения температуры в период проведения искусственного осеменения и обеспечения возможности отопления юрты в зимнее время целесообразно выбрать отопительный агрегат мощностью 10 кВт.

Время необходимое для повышения температуры помещения манежа до нормативного значения рассчитывается, как отношение требуемого количества тепла к мощности отопительного агрегата.

t=QР=8,610=0,86 часа,

где:

t, время  час.; Q – требуемое количество тепла на нагрев воздуха, кВт/ч.;

Р - мощность отопительного агрегата, кВт.

Для обеспечения функционирования ППИО требуется источник электрической энергии. Основные характеристики используемых потребителей электроэнергии приведены в таблице 3.

Таблица 3 – Основные потребители электроэнергии в ППИО

| Наименование электрооборуд. | Марка оборудования | Кол-во шт. | Потребляемая мощность, Вт. | Прод. работы, час | Суточный расход электроэнергии, кВтч/сут |
| --- | --- | --- | --- | --- | --- |
| Освещение манежа | СПБ-РОНДО IP40 | 4 | 30 | 4 | 0,48 |
| Освещение лаборатории | КМ 6655С | 1 | 10 | 3 | 0,030 |
| Освещение туалета | ECOLITE-А60-Е27-6 | 1 | 6 | 1 | 0,06 |
| Освещение душевой | ECOLITE-А60-Е27-6 | 1 | 6 | 1 | 0,06 |
| Наружное освещение | ECOLITE-А60-Е27-11 | 2 | 11 | 2 | 0,044 |
| Холодильник | Бирюса 8Е/8ЕК1/8Е-2/8 | 1 | 135 | 6 | 0,81 |
| Ноутбук | Asus | 1 | 150 | 2 | 0,30 |
| Стиральная машина | Малютка, модель-021 | 1 | 240 | 1 | 0,24 |
| Циркуляционный насос | LPm125 | 1 | 125 | 1 | 0,12 |
| Микроскоп | Lewinhuk DTX 500 Mobi | 1 | 12 | 1 | 0,012 |
| Итого |  |  | 826 |  | 2,1 |

На основании полученных результатов моделирования режимов солнечной станции установлено, что для автономного энергоснабжения в ППИО необходима солнечная фотоэлектрическая станция с суммарной вырабатываемой электроэнергией не менее 2,1 кВт*ч/сут. В условиях Алматинской области искусственные осеменение овец проводится в октябре-ноябре, продолжительность составляет 20-30 дней. В остальное время солнечная станция может быть использована для электроснабжения бытовых нужд фермерского хозяйства.

Требуемым параметрам соответствует солнечная электростанция 2,2 кВт/сутки (12В) со следующими характеристиками. Выработка электроэнергии 2,2 кВт/сутки (12В). Количество солнечных панелей – 3 шт. Мощность системы – 450 Вт. Напряжение постоянного тока – 12 В. Напряжение переменного тока – 220 В. Энергоэффективность системы – 2,25 кВт*ч/день. Энергетический потенциал аккумуляторных батарей – 2,4 кВт*ч. Запас энергии при 50% разряде батарей – 1,12 кВт*ч. Максимальная мощность подключаемой нагрузки – 1000 Вт. Пиковая мощность подключаемой нагрузки – 1500 Вт. Общая площадь солнечных панелей – 3 м2. Общая масса системы (включая АКБ) – 140 кг.

    - Методы и приборы контроля

а) Контроль габаритных размеров – линейные измерения производятся с помощи рулетки.

б) Контроль и регистрация температуры и влажности внутри помещений осуществлялся с применением датчика температуры и влажности (термогигрометр) марки  ИВТМ-7Н, с диапазоном измерения относительной влажности от 0 до 99% и диапазон измерения температуры от -45° С до +120°С. Абсолютная погрешность измерения температуры в диапазоне от -20°С до +60°С не превышает 0,2°С. Для автоматической регистрации показаний термогигрометра применялся измеритель-регистратор марки ИС-203.3. Двухканальный универсальный электронный самописец ИС-203.3 обеспечивает возможность подключения к ПК для регистрации и вывода показаний в режиме реального времени.

в) Контроль и регистрация параметров наружного воздуха осуществляется с использованием метеостанции для регистрации наружной температуры и скорости ветра, марка Davis Vantage Pro 2. Диапазон показателей температуры от-40° C до + 65° C, сбор данных каждые 6 с. с возможностью вывода информации на компьютер в режиме реального времени.

г) Контроль освещенности в помещениях ведется с помощью люксметра LX-1330B. Диапазон измерения от 0 до 20000 лк., воспроизводимость измерений 2%,  погрешность не более 3%, рабочая температура от 0° до 40° С.

2.2. Показатели качества выполнения технологического процесса

Перед определением показателей качества выполнения технологического процесса овцы заводятся в тамбур для неосеменённых овец. Результаты хронометража технологического процесса искусственного осеменения овец приведены в таблице 3.

Рисунок 4 – Фрагмент испытаний (процесс проведения искусственного осеменения овцы)

Таблица 3 - Результаты хронометража технологического процесса искусственного осеменения овец

| Повторность опытов | Время, затраченное на подачу овцы в станок, сек. | Время, затраченное на фиксацию, сек. | Время, затраченное на расфиксацию, сек. | Время, затраченное на выгон из станка, сек. | Итого |
| --- | --- | --- | --- | --- | --- |
| 1 2 3 4 5 | 63 57 59 62 59 | 15 11 15 14 11 | 12 10 12 14 9 | 5 7 4 8 5 | 95 85 90 98 84 |
| Сумма | 300 | 66 | 57 | 29 | 452 |
| Ср. знач. | 60 | 13,2 | 11,4 | 5,8 | 90,4 |

На проведение самого процесса искусственного осеменение одной овцы, зафиксированной в станке, отводится не более одной минуты. Следовательно, для осеменение одной овцы необходимо затрачивать около 150 секунд.

Основные технико-технологические показатели экспериментального образца ППИО приведены в таблице 4.

Таблица 4 – Результаты технико-технологической оценки

| № | Наименование показателя | Ед. измер. | Значение показателя |  |
| --- | --- | --- | --- | --- |
|  |  |  | По ТТ | По экспер. |
| 1 | Сменная производительность, | голов | до 30 | 26-28 |
| 2 | Сезонная производительность, | голов | 500-1000 | 593 |

Продолжение Таблицы 4

| 3 | Температура помещения для осеменения | °С | 15-20 | 18-25 |
| --- | --- | --- | --- | --- |
| 4 | Диаметр кругового помещения | м | 6 | 6 |
| 5 | Площадь кругового помещения | кв. м | 28 | 28 |
| 6 | Габариты кабины | м | 3,0×2,0× 2,0 | 3,0×2,0× 2,0 |
| 7 | Масса оборудования | т | до 1,5 | 1,46 |
| 8 | Грузоподъёмность автоприцепа | т | 1,5 | 1,5 |
| 9 | Номинальная мощность ФЭСМ | Вт | 400 | 450 |
| 10 | Номинальная мощность солнечной панели | Вт | 100 | 150 |
| 11 | Количество солнечных панелей | шт | 4 | 3 |
| 12 | Емкость аккумулятора | А·ч | 80 | 80 |
| 13 | Номинальная мощность инвертора | Вт | 500 | 1000 |
| 14 | Напряжение инвертора на входе, | В | 12/24 | 12 |
| 15 | Напряжение инвертора на выходе | В | 220(+2% -10%) | 220(+5% ) |
| 16 | Масса ФЭСМ | кг | Не более 150 | 140 |
| 17 | Трудоемкость монтажа, чел.-ч | Час. | Не более 6 | 5 |

Таблица 5 – Основные функциональные показатели

| Наименование показателя | Описание и значение |
| --- | --- |
| Общая характеристика |  |
| Место проведения испытаний | НИИ овцеводства им. К.У Медеубекова, пос. Мынбаева, Алматинской области. |
| Назначение и марка, испытуемого оборудования | Экспериментальный образец ППИО |
| Направление деятельности хозяйства | Научные исследования, разведение племенных овец породы казахский архаромеринос. |
| Способ содержания животных | пастбищное |
| Наличие типовых помещений в т.ч. | Типовые помещения, предназначенные для содержания овец разные периоды года и ветобработки животных имеются |
| - стригальных пунктов | Специализированный стригальный пункт отсутвтвует. |
| Показатели обеспеченности: |  |
| - электроэнергией | К пункту подведена электическая сеть 380/220 В, обеспеченность 100% Имеется также генератор по выработке электроэнергии, который используется по мере необходимости (во время окота, вынужденного осмотра животных и т. д.). |
| - кормами | Страховой запас стойловых кормов на 2 мес. имеется. |
| - водой | Обеспеченность животных водой - 100%. |

Продолжение Таблицы 5

| - квалифицированными кадрами | Обеспеченность квалифицированными кадрами – 80 %. |
| --- | --- |
| Характеристика помещений |  |
| Основного помещения (манежа), лаборатории, оцарок для содержания овец и баранов производителей | Сборно-разборное помещение круглой формы (Юрта) для проведения осеменения. Лаборатория располагается внутри кабины для транспортировки оборудования ППИО. Навесы, земляной пол для размещения баранов производителей. |
| Номер типового проекта | Экспериментальный образец |
| Линейные размеры помещения манежа, м | диаметр юрты 6 м |
| Линейные размеры кабины для лаборатории, м | 3×2×1,9 |
| Площадь помещения манежа, м2 | 28 |
| Площадь, занятая технологическим оборудованием, м2 | 8 |
| Вместимость, гол. | 30 |
| Удельная площадь пола, м2/гол.: |  |
| - помещения | 0,93 |
| - площадки | 0,66 |
| Коэффициент использования площади пола помещения, % | 85 |
| Характеристика животных |  |
| Вид | Тонкорунные племенные овцематки породы АК со средней живой массой 57,7 кг, возраст животных 2,5-4,5 года |
| Порода | Казахский архаромеринос (АК) |
| Пол | Самки, бараны -производители |
| Возраст, лет | 2,5-4,5 |
| Упитанность | Выше-средняя и высшая |
| Средняя масса маточного поголовья, кг | Отара состоит из 500 овцематок, со средней живой массой 57,7 кг |
| Микроклимат помещения |  |
| - температура воздуха, ºС | 18-25 |
| - относительная влажность воздуха, % | 50-60 |
| Содержание вредных веществ в воздухе: - углекислого газа, % - аммиака, мг/м3 - сероводорода, мг/м3 - пыли, мг/м3 - скорость движения воздуха, м/с | Замеры не производились. Оценка визуальная |
| Климатические условия |  |

Продолжение Таблицы 5

| - температура наружного воздуха, ºС - относительная влажность, % - скорость ветра, м/с - атмосферное давление, кПа - количество осадков, мм | 8-15 70-90 0,5-0,8 - Нет |
| --- | --- |
| Технологическое оборудование для содержания животных: | щиты ограждений, изгороди; станок для фиксации овец; отопительный агрегат; стол для размещения приборов. |
| Станки для зооветеринарных мероприятий | Универсальный станок-фиксатор |
| Линейные размеры, м: - длина - ширина - высота | 100 см 70 см 90 см |
| Площадь, занимаемая станком, м2 | 0,9 |
| Количество обслуживающего персонала, чел. | 3 человека (техник-осеменатор и 2 помошника) |
| Вместимость, гол | 1-4 гол, в зависимости от половозрастного состава животных |
| Характеристики препарата, применяемого для ветеринарно-санитарной обработки |  |
| - вид препарата | двууглекислая сода или стиральный порошок |
| - концентрация рабочей жидкости, % | 3 |
| - температура рабочей жидкости до подогрева, ºС | 20-25 |
| Характеристика станка для проведения зооветеринарных мероприятий: |  |
| - количество одновременно осеменяемых овец, гол. | 1 |
| - количественная доля травмированных животных, % | 0 |
| - надежность фиксации животных, % | 97-100 |
| - удобство проведения зооветеринарных мероприятий | удобное |
| - оборудование для освещения | Естественное и искусственное освещение |

3 Анализ и заключение по результатам технической оценки

Установка ППИО поступила на испытания комплектной, полнота документации соответствует требованиям ГОСТ Р 54783-2011. Испытания сельскохозяйственной техники. Основные положения, СТО АИСТ 2.8-2010 Испытания сельскохозяйственной техники. Надежность. Методы оценки показателей.

Установлено, что:

- ППИО выполняет технологический процесс с показателями, соответствующими техническим требованиям на разработку;

- комплект оборудования ППИО состоит из помещения манежа, кабины, транспортной тележки, лабораторных приборов и инструментов, отопительного котла.  ППИО снабжен автономными источниками энергии на основе ВИЭ;

- оборудование хорошо приспособлено для оперативного переезда, монтажа и демонтажа; Нагрузка на организм рабочего персонала, в виде физической нагрузки на опорно-двигательный аппарат и на функциональные системы организма работника, а также сенсорная нагрузка на центральную нервную систему и органы чувств работника отвечает требованиям ССТБ.

- сменная производительность пункта составляет до 28 голов,   сезонная производительность – 590 голов;

- технологические и технические показатели установки соответствуют показателям технических требований и обеспечивают условия для качественного проведения искусственного осеменение овец;

- полученные данные технической оценки соответствуют требованиям качества выполнения технологического процесса искусственного осеменение овец;

- для организации автономного энергоснабжения в ППИО снабжена фотоэлектрической станцией, со следующими характеристиками: площадь солнечных панелей – 3 м2, с суммарная вырабатываемая электроэнергия не менее 2,1 кВт۰ч/сут; мощность – 450 Вт; напряжение переменного тока – 220 В; объем заряда аккумуляторных батарей – 2,4 кВт۰ч; запас энергии при 50% разряде батарей – 1,12 кВт۰ч; допустимая мощность подключаемой нагрузки не более – 1000 Вт, допустимая пиковая мощность подключаемой нагрузки – 1500 Вт.

Общая масса оборудования ППИО не превышает 1500 кг.

По результатам испытаний и экспертизы установлено, что экспериментальный образец передвижного пункта искусственного осеменение овец пригоден к эксплуатации.

Испытания проводились в соответствии СТО АИСТ 32.4-2008 (Стандарта организации) Испытания сельскохозяйственной техники «Комплекты машин и оборудования для содержания овец и коз» Методы оценки функциональных показателей.

Исполнители:                                        Д.М. Алиханов

Т. Кенжкбвев

МИНИСТЕРСТВО СЕЛЬСКОГО ХОЗЯЙСТВА РЕСПУБЛИКИ КАЗАХСТАН

Некоммерческое акционерное общество

«Национальный аграрный научно-образовательный центр»

ТОО «Научно-производственный центр агроинженерии»

Протокол № 02

От « 07 »  07.2019 г.

Хозяйственных испытаний

передвижного стригального пункта для овец

Алматы, 2019

Введение

| Заводской номер | Год изготов-ления | Дата поступления на испытания |  | Период испытаний | Объем работ, сут. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | по плану | факти-чески |  | по плану | факти-чески |
| 001 | 2019 | 15.06.19. | 24.06.16 | с 24.06.19 по 07.07.19 | 15 | 15 |

Организация – разработчик – ТОО «НПЦ АИ»

Министерство – ведомство – МСХ РК

Испытания проведены на соответствие машины требованиям ТТ по рабочей программе и методике, утвержденной 04.06.19 .

- Характеристика испытываемого образца

1.1 Назначение и технические описание машины.

Передвижной стригальный пункт для овец (в дальнейшем ПСП) предназначен для проведения стрижки овец всех пород. ПСП применяется для стрижки овец фермерских, крестьянских личных подсобных хозяйств в условиях отгонного овцеводства.

1

2

4

8

3

10

6

7

9

11

12

13

Общий вид установки показан на рисунке 2.

5

4

4

Рисунок 2 – Схема передвижного стригального пункта

В состав входят: сборно-разборный загон для овец 1, состоящий из 4-х секций, для предварительного распределения овец по классам шерсти, с центральной секцией 2, откуда животные по 2-м расколам-проходам 3 направляются на стрижку, основная площадка 4, стригальные машинки 5, классировочные столы 6, шерстопресс 7, штабелер тюков 9, автономная электростанция 10, юрта для персонала 11, душевая кабина 12, биотуалет 13.Грузовой контейнер 8 для транспортировки и хранения прессованных тюков устанавливается рядом и стыкуется с основным контейнером, так чтобы площадки контейнеров находились на одном уровне. При этом, совместно с центральной площадкой, образуется единая рабочая зона для выполнения классировки, прессования шерсти, а также перемещения и погрузки тюков грузовой контейнер.

Основная площадка состоит из центральной платформы, боковых, передней и задней частей. В транспортном состоянии боковые, передние и задние стенки поднимаются и закрепляются в вертикальном положении. В рабочем состоянии стенки раскладываются в стороны и устанавливаются в горизонтальном положении, где под каждую стенку устанавливаются опорные стойки. При этом, совместно с центральной платформой они образуют единую рабочую зону. Крыша контейнера остается в своем первоначальном положении и служит в качестве навеса.

Расколы-проходы 3 начинаются с центральной секции загона и устанавливаются вдоль боковых стенок платформы 4. При этом, между ними оставляется продольная площадка, на которой располагаются рабочие места стригалей, со стригальными столами и машинками. В расколах-проходах напротив каждого места имеется калитка, через которую овца поступает на стрижку. Стригальные машинки получают электропитание от автономной электростанции 10, к которой также подключаются другие электроприборы.

1.2. Описание технологического процесса

Работа стригального пункта осуществляется следующим образом.

В контейнер загружается все необходимое оборудование, включая: секции сборно-разборного ограждения загонов для овец, стригальные машинки, станок для заточки ножей машинок, классировочные столы, шерстопрессы, штабелер тюков, которые доставляются до места дислокации автомобилем.

По прибытии в точку дислокации стригальный пункт разворачивается и приводится в рабочее состояние. Снимаются с фиксаторов и приводятся в горизонтальное положение боковые стенки контейнера, которые совместно с центральной площадкой образуют рабочую зону. Под ними устанавливаются соответствующие опорные стойки. Задняя стенка откидывается и стыкуется с задней стороной контейнера 8, который устанавливается, напротив, с предварительно открытыми задними дверями. На образованную задней стенкой площадку устанавливаются шерстопрессы 7.

Затем устанавливается вся ограждающая конструкция, в том числе внешняя ограда загона для овец, внутренние перегородки между секциями, расколы-проходы 3. Устанавливается автономная электростанция 10, прокладываются проводки для питания стригальных машинок.

По предварительно составленному графику отары овец пребывают на пункт. Овец загоняют в загон с разделением их на 4 секции по качеству шерсти. Из секций овцы предварительно поступают в центральную секцию и далее направляются по расколам 3, где они, перемещаясь по проходу, постепенно выстраиваются в цепочку.

Стрижка должна осуществляться на земле, рядом со стригальной площадкой. Стригали (10 человек) должны располагаться параллельно боковой кромке платформы, на площадке между платформой и расколами, на расстоянии удобном для выполнения стрижки и подачи состриженного руна на площадку. Поэтому, расстояние между стригалем и расколом, а также между стригалем и платформой должны быть на расстоянии «вытянутой руки».

Состриженную шерсть забирают классировщики. После обработки на классировочном столе 6 шерсть складывается в специальные накопители по сортам. Прессовщики, используя шерстопрессы 7 осуществляют прессование ее в тюки. Готовые тюки при помощи штабелера 9 загружаются в контейнер 8. После заполнения контейнер доставляется на фабрику первичной обработки, где тюки хранятся до их обработки. При этом, в отличие от известной технологии, из технологической цепочки исключаются склад для хранения шерсти, перегрузочные работы.

1.3 Техническая характеристика установки

Техническая характеристика передвижного стригального пункта приведена в таблице 4 (форма А.1 СТО АИСТ 32.4-2008)

Таблица 4 –Техническая характеристика установки

| Наименование показателя | Значение показателя |
| --- | --- |
| Тип | передвижная |
| Количество овец за смену | до 800 гол |
| Количество овец за сезон | до 16000 голов |
| Объем шерсти за сезон | до 60 тонн |
| Количество персонала | 16 чел |
| Масса оборудования | до 3,5 тонн |
| Марка стригальной машинки, ее масса и другие технические показатели | Heiniger ONE 722-610 230V/300W EU |
| Материал изготовления классировочных столов и пресса | Металл |
| Количество остриженных овец за смену одной гребенкой от заточки до заточки | 12 |
| Средний настриг шерсти с одной головы | 2,5 кг |
| Размер кипы | 980х700х700 см |
| Масса кипы | 110 кг |
| Плотность кипы | 250 кг/м3 |
| Продолжительность цикла прессования | 28 мин |
| Габаритные размеры (в транспортным положение), мм: - ширина - длина - высота | 2438 6058 2896 |
| Габаритные размеры (в рабочем положение), мм: - ширина - длина - высота | 8630 11850 2896 |
| Срок службы, лет | 8 |
| Коэффициент эксплуатационной надежности | 0.85 |

2.1. Условия испытаний

Оценка качества технологии стрижки овец.

Качество стрижки овец зависит от многих факторов. При квалифицированной стрижке шерсть снимается целиком, без уступов. При этом сохраняется вся длина шерсти, что очень важно, так как длина - самый ценный показатель качества шерсти. Уменьшение длины шерсти - очень распространенный дефект стрижки. Если срезать шерсть не у самого основания штапеля, а выше, го она будет искусственно укорочена и такую шерсть вместо, например, первого класса отнесут ко второму или к третьему. Неопытный стригаль, оставляя на овце неостриженные участки шерсти, напоминающие борозды вспаханного поля, старается устранить этот недостаток и остригает оставшуюся на овце короткую шерсть. При этом получается дефектная шерсть - подстрижка.

Стрижка овец должна проводиться в период безветренной и сухой погоды Общая ее продолжительность 10-15 рабочих дней. Календарные сроки зависят от природно-климатических условий.

Весенняя стрижка должна начинаться до естественной линьки. Запаздывание приводит к большим потерям. Осенью стрижку необходимо  начинать с таким расчетом, чтобы к зиме у овен вырос покров шерсти, способный  предохранить их от холода.

За 12-15 ч. до стрижки овен не поят и не кормят, так как неестественное положение во время стрижки может привести к нарушению деятельности пищеварительных органов, что вызывает заболевание, а иногда и гибель овец. Кроме того, напоенные овцы выделяют много пота и мочи, шерсть портится. Нельзя также стричь овец с влажной шерстью, так как она самонагревается и преет в тюках, теряя свою крепость и цвет.

Стрижку овец в зависимости от пола и возраста ведут в следующей очередности: первыми стригут овцематок зимнего окота, затем молодняк, валухов, овцематок весеннего окота и баранов-производителей.

При стрижке необходимо соблюдать следующие основные правила:

- бережно обращаться с овцой, не причиняя ей травм. Травмированные овцы значительно снижают свою продуктивность;

- срез шерсти вести возможно ниже и ровнее, не допуская участков е высокоостриженной шерстью, так как это уменьшает настриг и значительно снижает качество за счет более короткой длины волокон шерсти;

- не допускать повторных срезов шерсти (перестрига), гак как при этом образуется «сечка», которая значительно снижает качество шерсти. Её практически невозможно отделить от руна, попадая в пряжу, она уменьшает ее прочность;

- не допускать порезов, а при их появлении обязательно обработать рану;

- руно полученное после стрижки, должно оставаться цельным без разрывов.

Размеры (мощность), основное и вспомогательное оборудование ПСП определены заданными исходными условиями.

Исходными условиями являются:

- число одновременно работающих стригалей (кстр), количество дней (τ) кампании стрижки при односменной работе пункта (это время обусловлено зоотехническими требованиями, и для районов тонкорунного овцеводства оно составляет 25-30 дней)

- основные, вспомогательные технологические процессы, дополнительные мероприятия.

Основные процессы: подгон, подача овец на стрижку, стрижка, классировка, прессование шерсти, упаковка, измерение веса, электронная регистрация качества, массы шерсти в тюке, нанесение электронного штрих-кода на тюки, транспортировка, погрузка и укладка тюков в транспортный контейнер.

Вспомогательные процессы: заточка-наладка стригальных ножей, монтаж, демонтаж, эксплуатация ПСП, ограждения загонов, электрооборудования.

Дополнительные мероприятия: охрана труда, техника безопасности, проживание, питание и отдых персонала.

Производительность стригаля (nгол), которая зависит от степени совершенства способов и приемов стрижки, а также уровня его квалификации, характеризуют следующие эксплуатационные показатели:

ηсм -коэффициент использования времени смены (ηсм=τч.рτсм, где τч.р-время чистой работы, ч;  τсм -продолжительность смены, ч);

ηр.х - коэффициент, учитывающий затраты времени на рабочие проходы машинкой (ηр.х=τр.хτч.р,  где τр.х -время рабочих проходов, ч);

ηв - коэффициент, учитывающий использование конструктивной ширины (захвата) ножа (ηв=bрb,где bр -рабочая ширина захвата, м; b -конструктивная ширина ножа, м).

Производительность стригаля, или его часовая наработка, nч  гол./ч, при индивидуальном способе стрижки определяется следующей зависимостью:

nч=3600∙ηр.х∙ηсм∙ηв∙b∙vпSo

где:	vп- скорость подачи машинки, м/с;

So- площадь остригаемой поверхности одной овцы, м2.

При стрижке овец поточным способом часовая производительность nч, гол/ч, рассчитывается по формуле:

nч=3600∙ηсм∙ηп.о∙ηпр∙ηвSo

где 	ηп.о- коэффициент, учитывающий породу овец, состояние шерстного покрова и степень засоренности руна;

ηпр-  число приемов снятия шерстного покрова с одной овцы;

ηв-  средняя скорость подачи, достигаемая стригальщиками в поточной линии, м/с.

Приняты следующие значения коэффициентов ηв: для стригальщиков высокой квалификации   ηв= 0,88-0,92; средней     ηв= 0,78-0,87;    низкой

ηв= 0,69-77.  При поточном методе стрижки овец с настригом 4,5-5,0 кг/гол. и  ηпр =4  для случаев использования станков-тележек ηп.о= 0,47-0,51, а при использовании установок карусельного типа ηп.о= 0,26-0,42.

По результатам хронометражных наблюдений установлены средние затраты времени на процесс: подавальщик овец - 0,375 мин на одну овцу; классировщик шерсти (с помощником) - 0,371 мин  на одно  руно; точильщик режущих пар - 1,35 мин на одну пару; слесарь-наладчик -2,05 мин на один вызов. При этом один подавальщик, как и слесарь-наладчик, может обслуживать 16 стригальщиков, а точильщик режущих пар - 37 стригальщиков.

На выносном стригальном пункте ВСЦ-24/200 средняя выработка стригальщика на 1 ч сменного времени при стрижке тонкорунных и каракульских овец составляет 8 голов, а грубошерстных - до 12 голов.

Объем производства Gстр стригального пункта, выраженный через массу полученной сырой шерсти за одну стрижку, определится из равенства

Qн=g∙Gстр∙Nстр∙Tстр

где 	g — настриг шерсти с одной овцы, кг

В зависимости от породы, возраста и условий содержания овец однократный настриг шерсти колеблется в довольно широких пределах; g=1,5-4 кг. Потери шерсти по причине некачественной стрижки составляют до 10-20 %.

Таблица 2 –Показатели качества выполнения технологического процесса

| Наименование показателя | Значение показателя |  |
| --- | --- | --- |
|  | По ТТ | По испыты-ваемой машине |
| Показатели качества выполнения технологического процесса |  |  |
| Продолжительность смены при стрижке овец | 8…10 ч. | 10 ч. |
| Количество остриженных овец за смену одной гребенкой от заточки до заточки | 12 | 12 |
| Количество настриженной шерсти за смену | 2000 кг | 2260 кг |
| Средний настриг шерсти с одной головы | 2,5 |  |
| Количество случаев и тяжесть пореза кожи на 10 голов. | 40 | 44 |
| Количественная доля разрыва рун | - | 0,03 |
| Высота среза шерсти | 6 | 6 |
| Массовая доля сечки и перестрига | - | 0,09 |

Продолжение Таблицы 2

| Показатели качества выполнения технологического процесса пресса для шерсти |  |  |
| --- | --- | --- |
| Размер кипы | - | 980х700х700 см |
| Масса кипы | - | 110 кг |
| Плотность кипы | - | 250 кг/м3 |
| Продолжительность цикла прессования | - | 28 мин |

2.3 Анализ и заключение по результатам технической оценки

Испытания ПСП проводились в режиме экспериментальной стрижки. Время проведения испытаний 15 дней – с 24.06.19 по 07.07.19 г.

Производительность ПСП составляла: 800 гол/сут.

Полученные данные испытаний соответствуют требованиям качества выполнения технологического процесса при стрижке овец.

3 Результаты испытаний

3.1 Энергетическая оценка

Результаты энергетической оценки работы ПСП приведены в таблице 3 (ГОСТ Р 52777-2007).

Энергетическая оценка проводилась для определения расхода электрической энергии.

Таблица 3 – Энергетические показатели ПСП

| Наименование показателя | Значение показателя |
| --- | --- |
| Количество активной энергии, затраченной на выполнение технологического процесса, кВтч/сут | 33 |
| Количество реактивной энергии, затраченной на выполнение технологического процесса, квар.ч | 3.77 |
| Средний коэффициент мощности | 0.95 |
| Удельные энергозатраты, МДж/кг | 0.052 |

3.2 Показатели назначения

3.2.1 Результаты эксплуатационно-технологической оценки (таблица 4).

Таблица 4 – Показатели эксплуатационно-технологической оценки

| Наименования показателя | Ед. изм. | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по ТТ | при испытаниях |
| Микроклимат окружающей среды |  |  |  |
| Температура | °С | 20 | 15-28 |
| Относительная влажность | % | 35 | 25-43 |
| Скорость движения воздушных потоков | м/с | 0,1 | 0,1-0,3 |
| Продолжительность смены | ч | 8 | 8 |

Продолжение Таблицы 4

| Количество остриженных овец за смену: - квалифицированный стригаль - неквалифицированный стригаль | гол. | 100 60 | 110 50 |
| --- | --- | --- | --- |
| Количество настриженной шерсти за смену: - квалифицированный стригаль - неквалифицированный стригаль | кг | 250 150 | 275 125 |
| Средний настриг шерсти с одной головы | кг | 2,5 | 2,6 |
| Количественная доля случаев пореза кожи: - легких: квалифицированный стригаль неквалифицированный стригаль - тяжелых: квалифицированный стригаль неквалифицированный стригаль | % | 5 15 0,5 3 | 5 15 0,5 2,5 |
| Высота среза шерсти: - квалифицированный стригаль - неквалифицированный стригаль | мм | 5 6 | 5 7 |
| Массовая доля сечки и перестрига; - квалифицированный стригаль - неквалифицированный стригаль | % за смену | 0,09 0,095 | 0,09 0,093 |
| Количество остриженных овец одной режущей парой от заточки до заточки: - квалифицированный стригаль - неквалифицированный стригаль | гол. | 10 12 | 11 13 |
| Пресс для шерсти |  |  |  |
| Размер кипы: - длина - ширина - высота | мм | 700 700 980 | 700 700 980 |
| Масса кипы | кг | 110 | 110 |
| Плотность кипы | кг/м3 | 250 | 250 |
| Продолжительность цикла прессования одной кипы | мин | 28 | 25 |
| Расход ткани на кипу | м2 | 3.8 | 3.8 |
| Классировочные столы |  |  |  |
| Удобство работы классировщика |  | удобно | удобно |
| Удобство расположения руна на классировочном столе |  | удобно | удобно |
| Технологическая линия для содержания овец |  |  |  |
| Изгороди для загонного выпаса |  | удобно | удобно |
| Возможность и удобство перемещения изгороди |  | удобно | удобно |
| Проникновение животных через изгородь в смежные загоны |  | нет | Нет |
| Количественная доля травмированных животных по причине оборудования | % | 0 | 0 |
| Щиты ограждений |  |  |  |
| Удобство перемещения и крепления |  | удобно | удобно |
| Устойчивость |  | Устойчиво | Устойчиво |
| Удобство отдыха животных |  | удобно | удобно |
| Удобство перемещения |  | удобно | удобно |
| Количественная доля травмированных животных по причине оборудования | % | 0 | 0 |

3.3 Показатели надежности

Таблица 5 – Показатели надежности

| Наименования показателя | Ед. изм. | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по ТТ | при испытаниях |
| Наработка на отказ | ч | - | 140 |
| Время монтажа | ч | - | 8 |
| Трудоемкость монтажа | чел·ч | - | 24 |
| Количество отказов |  |  |  |
| I группа сложности |  | - | 2 |
| II группа сложности |  | - | - |
| III группа сложности |  | - | - |
| Наработка на отказ |  |  |  |
| I группа сложности |  | - | 28 |
| II группа сложности |  | - | - |
| III группа сложности |  | - | - |
| Трудоемкость ежесменного технического обслуживания, чел.-ч |  | - | 3,5 |
| Удельная суммарная трудоемкость текущего ремонта (отыскания и устранения отказов) | чел·ч | - | 7 |
| Продолжительность отыскания и устранения отказов | ч | - | 3,5 |
| Среднее время восстановления | ч | - | 1,2 |
| Коэффициент готовности |  | 0,98 | 0,998 |

3.3.1 Первичная техническая экспертиза

Установка поступила на испытания комплектной, полнота документации соответствует требованиям ГОСТ Р 54783-2011. Испытания сельскохозяйственной техники. Основные положения, СТО АИСТ 2.8-2010 Испытания сельскохозяйственной техники. Надежность. Методы оценки показателей. Несоответствия ТТ не выявлены.

3.4 Оценка условий труда

3.4.1 Показатели оценки условий труда

Таблица 6 – Показатели оценки условий труда

| № п/п | Показатели по ТТ, ССБТ | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по НТД | при испытываемой машине |
| 1 | Удобство техобслуживания | удобно | Удобно |
| 2 | Удобство технологического обслуживания | удобно | Удобно |
| 3 | Количество рабочих движений в минуту | 10 | 10 |
| 4 | Рабочая поза | рациональная | рациональная |
| 5 | Удобство монтажа и демонтажа | удобно | Удобно |

3.4.2 Перечень несоответствий конструкции машины требованиям ССБТ

Для обеспечения безопасности при эксплуатации установка оборудована следующими устройствами:

-  расколы-проходы;

-  рабочие места стригалей, со стригальными столами и машинками;

-  калитка, через которую овца поступает на стрижку.

Несоответствия требованиям ССБТ не обнаружено.

3.5 Перечень несоответствий машины требованиям ТТ

Несоответствия требованиям ТТ не обнаружено.

3.6 Анализ и заключение по эксплуатационно-технологической оценке

4 Экономическая оценка

4.1 Расчет экономической эффективности ПСП

ГОСТ Р 53056-2008 Техника сельскохозяйственная. Методы экономической оценки

СТО АИСТ 003-2010 Экономическая оценка сельскохозяйственной техники. Термины и определения

Таблица 7 – Показатели экономической оценки

| № п/п | Наименования показателя | Ед. изм. | ПСП |
| --- | --- | --- | --- |
| 1 | Капитальные затраты | тенге | 8 000 000 |
| 2 | Капитальные затраты с монтажом | тенге | 10 000 000 |

Продолжение Таблицы 7

| 3 | Эксплуатационные издержки на: | тенге | 1153 250 |
| --- | --- | --- | --- |
|  | Амортизационные отчисления (А) | тенге | 500000 |
|  | Капитальный ремонт | тенге/за сезон | 1 000 000 |
|  | Расход горючего и др. (Э) (35 л бензин* цена (145 тг)) | тенге/за сезон | 173250 |
|  | Расходы на питание. Проживание (Г) (1000 тг*персонал) | тенге/за сезон | 480000 |
| 4 | Сумма приведенных затрат | тенге/за сезон | 1 000 000 |
|  | Экономический эффект | тенге/год | 511750 |

4.2 Анализ и заключение по экономической оценке

Полученные данные показывают, что ПСП дает ожидаемый доход и экономический эффект составляет– 511750 тенге за сезон.

5 Анализ и заключение по результатам испытаний

В результате испытаний установлено, что ПСП выполняет технологический процесс с показателями, соответствующими ТТ на разработку.

Производительность установки составляет 800-1000 гол/сут.

Установка дает экономический эффект за счет снижения эксплуатационных затрат 511750 тенге/год.

Достоинства конструкции

Трансформируемость – достаточно простораскладывается из транспортного положенияв рабочее и собирается обратно.

Перевозится на прицепе автомобиля.

Созданы условия для высокопроизводительной и качественной работы персонала.

Универсальность – после оканчания сезона стрижки, в остальной период года, после соответствующих перестроений, может быть переделан в жилой дом. Созданы условия для качественной и высокопроизводительной работы – укомплектован современными стригальными машинками, шерстопрессом, штабелером тюков.

Недостатки конструкции

1. Доукомплектовать ПСП автономными источниками энергии на основе ВИЭ.

2. Вспомогательное оборудование щиты, загоны выполнить из легких композитных материалов.

Выводы и предложения

На основании материалов производственных испытаний ПСП можно сделать следующие выводы:

1. Установка соответствует требованиям ТТ.

2. Установка выполняет технологический процесс.

3. Установка соответствует требованиям ССБТ.

4. Рекомендуется разработать опытный образец ПСП и провести его предварительные и приемочные испытания.

Исполнители:                                                М.Ж. Исаханов

Р. Касым

МИНИСТЕРСТВО СЕЛЬСКОГО ХОЗЯЙСТВА РЕСПУБЛИКИ КАЗАХСТАН

Некоммерческое акционерное общество

«Национальный аграрный научно-образовательный центр»

ТОО «Научно-производственный центр агроинженерии»

Протокол №03

От «30» 06.2019 г.

Хозяйственных испытаний

оборудования хранения шерсти

Алматы, 2019

Введение

| Заводской номер | Год изготов-ления | Дата поступления на испытания |  | Период испытаний | Объем работ, сут. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | по плану | факти-чески |  | по плану | факти-чески |
| 001 | 2019 | .05.19. | .05.19 | с 01.06.19 по 30.06.19 | 15 | 15 |

Организация – разработчик – НПЦ АИ

Министерство – ведомство – МСХ РК

Испытания проведены на соответствие машины требованиям ТТ по рабочей программе и методике, утвержденной 04.06.19 .

- Характеристика испытываемого образца

  - Назначение и технические описание машины.

Кипы шерсти транспортируют в крытых железнодорожных вагонах, трюмах судов и автомашинах в соответствии с правилами, действующими на транспорте данного вида.

Кипы должны транспортироваться в условиях, исключающих их загрязнение, нарушение целостности упаковки, а также попадание атмосферных осадков. Кипы шерсти и пуха должны храниться в крытых помещениях с бетонированным или асфальтированным покрытием.

Допускается хранение кип шерсти на специальных площадках под навесом или укрытых брезентом, на настиле из подтоварника.

Основное назначение склада для шерсти, как и всякого склада, заключается в сохранении шерсти от порчи, убыли и всякого иного уничтожения. Порча шерсти при её хранении происходит главным образом от избыточной влажности воздуха внутри склада и от повреждений молью. Поэтому склады для шерсти прежде всего должны быть достаточно сухими. В связи с этим полы в них делают деревянными. В тех же случаях, когда шерсть приходится складывать на земляной пол, кипы кладут не непосредственно на землю, а на деревянные подкладки, называющиеся подтоварниками [2].

Подтоварники представляют собой деревянные бруски (слегка обтёсанные брёвна) средней толщины и длиной около 3 м, подкладываемые на земляной пол по 3 шт. под каждую кипу так, чтобы кипы шерсти не касались земли.

Особое внимание обращается на оборудование склада освещением. При отсутствии электрического освещения допускается пользование керосиновыми лампами и свечами в исправных металлических фонарях с сетками. Электропроводка должна производиться с соблюдением всех правил изоляции. Кроме того, склад должен быть оборудован противопожарными средствами.

Замена таких крытых складов открытыми контейнерными складами обеспечить, лучшее использование складских площадей в результате многоярусного складирования контейнеров, сокращает капиталовложения в складское хозяйство с учетом необходимых средств на механизацию в несколько раз.

Оборудование состоит из грузового контейнера общего назначения. Камера контейнера должна вмещать 56 прессованных тюков шерсти размером: 980х700х700 см. Влажность загружаемых тюков не должна превышать 17%.

На пункте стрижки, в процессе транспортировки и на фабрике ПОШ контейнеры будут располагаться на открытом воздухе. Длительность процесса хранения может составлять несколько месяцев, включая осенний, зимний периоды. Для исключения порчи шерсти не допускается промокание и переувлажнение шерсти. Влажность в камере контейнера должна поддерживаться в пределах  65-70%.

Для исключения хищений контейнеры должны закрываться на замок.

Для дистанционного контроля состояния шерсти контейнеры должны быть оснащены датчикам температуры и влажности.

Для поддержания температурно-влажностного состояния внутреннего воздуха камера должна снабжаться естественной и искусственной вентиляцией.

1.2. Описание технологического процесса

Контейнерная технология – одно из направлений технического прогресса в организации перевозок, складировании и хранении грузов. Контейнеризация являются одним из самых прогрессивных направлений развития, рационализации и оптимизации транспортных процессов. Контейнерные перевозки позволяют освободить грузовладельца от необходимости транспортной упаковки и маркировки, снижают затраты на погрузочно-разгрузочные и складские работы при смешанном сообщении [   ].

Контейнер представляет собой прямоугольную камеру, предназначенную для перевозки практически любых видов груза. При этом, он идеально приспособлен для оперативной и удобной погрузки на различные виды транспорта. Специально оборудованные погрузчики способны в считанные часы загрузить и надежно закрепить в транспортном средстве большую партию контейнеров. Неоспоримым преимуществом контейнера является также его высокая прочность и надежность. Все это позволяет гарантировать хранение, безопасную перевозку разнообразных грузов, в том числе упаковок шерсти различными видами транспорта [    ].

Техническую базу системы контейнерного хранения составят: парк контейнеров, автотранспорт, используемый для перевозки контейнеров, перегрузочное средства для тюков.

Применение контейнеров позволит комплексно механизировать и автоматизировать погрузочно-разгрузочные и складские операции, полностью исключить тяжелые ручные работы, в 2 раз снизить себестоимость перегрузочных работ, в 2 раза сократить затраты на тару и упаковку, повысить сохранность перевозимой продукции, ускорить на 25 % доставку грузов.

На рисунке 1 схематически представлен комплекс для контейнерной перевозки и хранения шерсти. Комплекс включает контейнер 1, состоящий из передней, задней, боковых, вертикальных стенок и систему вентилирования. Система вентилирования состоит из вентилятора 2, аэроионизатора и вентиляционного канала 3, расположенного под полом, состоящего из последовательно соединенных модулей, соединенного с фланцем 4, снабженным обратным клапаном, съемных заглушек и выходных окошек 5, снабженных обратными клапанами, датчиков температуры и влажности 6, солнечной панели 7 для электропитания датчиков, аэроионизатора и вентилятора. Нормированный микроклиматосуществляется во время складирования тюков в стригальном пункте и транспортировки и во время  длительного хранения на фабрике. В пункте стационарного хранения оборудованный контейнер по необходимости подключается к системе стационарного вентилирования, содержащей источник теплоты с помощью мягких рукавов.

Рисунок 1- Комплекс для контейнерной перевозки и хранения шерсти.  а) вид с боку, б) вид сзади, в) вид сверху

На пункте стрижки, в процессе транспортировки и на фабрике ПОШ контейнеры будут располагаться на открытом воздухе. Длительность процесса хранения может составлять несколько месяцев, включая осенний, зимний периоды. Для исключения порчи шерсти не допускается промокание и переувлажнение. Влажность в камере контейнера должна поддерживаться в пределах  65-70%.

Контейнер оснащен датчиками температуры, влажности, газа в камере для дистанционного контроля состояния шерсти в период длительного хранения.

Контейнерная система хранения шерсти, включая перевозки, снимает многие из недостатки, присущие существующим технологиям. Она позволяет:

– исключить тяжелый физический труд на операциях разгрузки и складирования тюков;

– значительно ускоряет производство грузовых работ;

– практически полностью исключить утрату и повреждения тюков;

–сократить простои автотранспорта;

–исключить затраты на строительство склада для хранения шерсти, так как тюки будут храниться непосредственно в контейнерах;

–значительно сократить себестоимость перевозок и хранения;

–ускорить доставку тюков с любой точки в пункты назначения без перегрузки и сократить объем грузовой массы, находящейся в процессе транспортировки.

Система контейнерных перевозок основана на строгой стандартизации и унификации технических средств, что обусловило ее международный характер.

Оборудование хранения шерсти быстро и удобно транспортируется, отвечает требованиям к хранению шерсти в условиях отгонного овцеводства. Комплектующие изделия легко поддается ремонту и замене в случае выхода из строя.

Техническая характеристика оборудования хранения шерсти приведена в таблице 1 (форма А.1 СТО АИСТ 32.4-2008)

Таблица 1 –Техническая характеристика установки

| Наименование показателя | Значение показателя |
| --- | --- |
| Тип | передвижной |
| Габаритные размеры, мм: - ширина - длина - высота | 2438 6058 2896 |
| Размер кипы | 980х700х700 см |
| Масса кипы | 110 кг |
| Плотность кипы | 250 кг/м3 |
| Продолжительность цикла прессования | 28 мин |
| Количество тюков шерсти в контейнере | 56 |
| Масса тюков в контейнере | до 6,5 тонн |
| Срок службы, лет | 8 |
| Коэффициент эксплуатационной надежности | 0.98 |

- Техническая оценка

2.1. Условия испытаний

Основное назначение склада для шерстя, как и всякого склада, заключается в сохранении шерстя от порчи, убыли и всякого иного уничтожения. Порча шерсти при её хранении происходит главным образом от избыточной влажности воздуха внутри склада и от повреждений молью. Поэтому склады для шерсти прежде всего должны быть достаточно сухими. В связи с этим полы в них надо делать деревянные.

В отношении требований, предъявляемых к строительным материалам, склад надо строить в соответствии с правилами строительного устава и противопожарной охраны.

Шерсть в складах должна храниться только упакованной в кипы. Хранение шерсти в неупакованном виде - навалом - запрещается.

Перед размещением шерсти в складе необходимо проверить, не упаковано ли слишком влажное сырьё. Если это обнаружится, то соответствующие кипы надо распаковать, шерсть просушить, снова запаковать и лишь после этого сложить её в склад. Равным образом необходимо следить за наличием на кипах полагающейся маркировки.

Складывать шерсть разрешается обязательно в сухое помещение с непропускающей дождевую воду крышей.

Шерсть на складах должна храниться так, чтобы кипы каждой партии лежали отдельными штабелями высотой в 6-7 рядов кип с достаточными проходами между штабелями для отгрузки шерсти из той или иной партии по мере надобности.

Следует систематически проверять состояние шерсти на контейнере. При обнаружении повышенной влажности, молеедин, согревания шерсти и прочих недочётов соответствующие кипы надо немедленно распаковать и принять все меры к устранению недочётов.

Таблица 2 –Показатели качества выполнения технологического процесса

| Наименование показателя | Значение показателя |  |
| --- | --- | --- |
|  | По ТТ | По испыты-ваемой машине |
| Показатели качества выполнения технологического процесса пресса для шерсти |  |  |
| Размер кипы | 980х700х700 см | 980х700х700 см |
| Масса кипы | 120 кг | 110 кг |
| Плотность кипы | 300 кг/м3 | 250 кг/м3 |
| Продолжительность цикла прессования | - | 28 мин |

2.3 Анализ и заключение по результатам технической оценки

Испытания ОХШ проводились в режиме экспериментального хранения. Время проведения испытаний 15 дней – с 24.06.19 по 07.07.19 г.

Вместимость ОХШ составляла: 6,5 тонн.

Полученные данные испытаний соответствуют требованиям качества выполнения технологического процесса при стрижке овец.

3 Результаты испытаний

3.1 Энергетическая оценка

Результаты энергетической оценки работы ОХШ приведены в таблице 3 (ГОСТ Р 52777-2007).

Энергетическая оценка проводилась для определения расхода электрической энергии.

Таблица 3 – Энергетические показатели ОХШ

| Наименование показателя | Значение показателя |
| --- | --- |
| Количество энергии, затраченной на выполнение технологического процесса, кВт.ч | 1,2 |
| Средний коэффициент мощности | 0.95 |
| Удельные энергозатраты, кДж/кг | 0.0036 |

3.2 Показатели назначения

3.2.1 Результаты эксплуатационно-технологической оценки.

Таблица 4 – Показатели эксплуатационно-технологической оценки

| Наименования показателя | Ед. изм. | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по ТТ | при испытаниях |
| Микроклимат окружающей среды |  |  |  |
| Температура | °С | 20 | 15-28 |
| Относительная влажность | % | 35 | 25-43 |
| Скорость движения воздушных потоков | м/с | 0,1 | 0,1-0,3 |
| Микроклимат внутри оборудования хранения шерсти |  |  |  |
| Температура | °С | 20 | 20 |
| Относительная влажность | % | 65-70 | 65 |
| Скорость движения воздушных потоков | м/с | 0,1 | 0,1-0,3 |
| Плотность кипы | кг/м3 | 250 | 250 |
| Продолжительность цикла прессования одной кипы | мин | 28 | 25 |
| Расход ткани на кипу | м2 | 3.8 | 3.8 |

3.3 Показатели надежности

Таблица 5 – Показатели надежности

| Наименования показателя | Ед. изм. | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по ТТ | при испытаниях |
| Наработка на отказ | ч | - | 160 |
| Количество отказов |  |  |  |
| I группа сложности |  | - | 2 |
| II группа сложности |  | - | - |

Продолжение Таблицы 5

| III группа сложности |  | - | - |
| --- | --- | --- | --- |
| Наработка на отказ |  |  |  |
| I группа сложности |  | - | 8 |
| II группа сложности |  | - | - |
| III группа сложности |  | - | - |
| Трудоемкость ежесменного технического обслуживания, чел.-ч |  | - | 0,5 |
| Удельная суммарная трудоемкость текущего ремонта (отыскания и устранения отказов) | чел·ч | - | 4 |
| Продолжительность отыскания и устранения отказов | ч | - | 1,5 |
| Среднее время восстановления | ч | - | 1,2 |
| Коэффициент готовности |  | 0,98 | 0,998 |

3.3.1 Первичная техническая экспертиза

Установка поступила на испытания комплектной, полнота документации соответствует требованиям ГОСТ Р 54783-2011. Испытания сельскохозяйственной техники. Основные положения, СТО АИСТ 2.8-2010 Испытания сельскохозяйственной техники. Надежность. Методы оценки показателей. Несоответствия ТТ не выявлены.

3.4 Оценка условий труда

3.4.1 Показатели оценки условий труда

Таблица 6 – Показатели оценки условий труда

| № п/п | Показатели по ТТ, ССБТ | Значение показателя |  |
| --- | --- | --- | --- |
|  |  | по НТД | при испытываемой машине |
| 1 | Удобство техобслуживания | удобно | удобно |
| 2 | Удобство технологического обслуживания | удобно | удобно |
| 3 | Количество рабочих движений в минуту | 2 | 2 |
| 4 | Рабочая поза | рациональная | рациональная |

3.4.2 Перечень несоответствий конструкции машины требованиям ССБТ

Конструкция ОХШ отвечает системе стандартов по безопасности труда и требований пожарной безопасности ГОСТ 12.1.004-91.

Несоответствия требованиям ССБТ не обнаружено.

3.5 Перечень несоответствий машины требованиям ТТ

Несоответствия требованиям ТТ не обнаружено.

4 Экономическая оценка

4.1 Расчет экономической эффективности ПСП

Таблица 7 – Показатели экономической оценки

ГОСТ Р 53056-2008 Техника сельскохозяйственная. Методы экономической оценки

СТО АИСТ 003-2010 Экономическая оценка сельскохозяйственной техники. Термины и определения

| № п/п | Наименования показателя | Ед. изм. | ПСП |
| --- | --- | --- | --- |
|  | Годовой экономический эффект | тенге | 3 000 000 |
|  | Прямые эксплуатационные затраты | тенге /т | 75 000 |
|  | Приведенные затраты | тенге /т | 30 000 |

4.2 Анализ и заключение по экономической оценке

Полученные данные показывают, что ОХШ дает ожидаемый доход и экономический эффект составляет– 3 000 000 тенге за год.

5 Анализ и заключение по результатам испытаний

В результате испытаний установлено, что ОХШ выполняет технологический процесс с показателями, соответствующими ТТ на разработку.

Вместительность контейнера составляет до 6,5 тонн шерсти.

Установка дает экономический эффект за счет снижения эксплуатационных затрат 3 000 000 тенге/год.

Достоинства конструкции

Перевозится на прицепе автомобиля.

Созданы условия для высокопроизводительной и качественной работы персонала.

Созданы условия для качественной и высокопроизводительной работы – укомплектован современными шерстопрессом, штабелером тюков.

ОХШ снабжен автономными источниками энергии на основе ВИЭ.

Выводы и предложения

На основании материалов производственных испытаний ОХШ можно сделать следующие выводы:

1. Установка соответствует требованиям ТТ.

2. Установка выполняет технологический процесс.

3. Установка соответствует требованиям ССБТ.

4. Рекомендуется разработать опытный образец ОХШ и провести его предварительные и приемочные испытания.

Исполнители:						М.Ж. Исаханов

Р. Касым

APPENDIX E

Acts of installation

#### APPENDIX F

#### List of publications

#### Список публикаций, патентов и авторских свидетельств

#### по теме НТП за 2018-2020 гг.

| № п/п | # Название статьи | Издательство, журнал (название, номер, год) или номер патента | Авторы |
| --- | --- | --- | --- |
|  | Статьи опубликованные в отечественных и зарубежных изданиях |  |  |
|  | Научно обоснованная модернизация пастбищного овцеводства – путь реализации экспортного потенциала шерстяного кластера | Международная агроинженерия /Научно-технический журнал ТОО «НПЦ агроинженерии». – выпуск 3 (23), – Алматы.– 2018. – С. 6-15. | Омаров Р.А., Кудер К.М., Омар Д.Р. |
|  | Задачи технико-технологической модернизации овцеводства | Современные тенденции развития овцеводства: Материалы международной научно-технической конференции посвященной II съезду овцеводов Казахстана. –Алматы, 2019, -350 с. (С.255-265) | Омаров Р.А., Досымбеков Т.Д. Ахметов Е.К. |
|  | Development of energy-saving ventilation system for agricultural buildings | Eurasian Physical Technical Journal. – Vol 15, 1(29), - Karaganda. – 2018. – P. 86-92. | Issakhanov M.Zh., Sakipova Sh.E., Alibek N.B., Dyusenbaev T.S. |
|  | Обоснование эффективности модернизации технологии и технических средств стрижки овец | Международная агроинженерия /Научно-технический журнал ТОО «НПЦ агроинженерии». – выпуск 1 (29), – Алматы.– 2019. – С. 25-35. | Омаров Р.А. Байжуманов С. Кудер К.М. Мухтар Ж.К. |
|  | Методика расчета и исследования параметров системы отопления и вентиляции передвижного пункта искусственного осеменения овец | Ізденістер нəтижелер/ исследования результаты /Научный журнал КазНАУ –Алматы. –2019. –No3. –С.400-405 | Шыныбай Ж.С., Алиханов Д.М., Молдажанов А.К., Омаров Р.А., Омар Д.Р. |
|  | Обоснование и исследование параметров автономной системы электроснабжения пункта искусственного осеменения овец | Ізденістер нəтижелер/ исследова-ния результаты /Научный журнал КазНАУ –Алматы. –2019. –No3. –С.343-348 | Алиханов Д.М., Молдажанов А.К., Муканали А.Б., Омаров Р.А., Кудер К.М. |
|  | К обоснованию конструктивно-технологического решения передвижного стригального пункта для овец | Ізденістер нəтижелер/ исследова-ния результаты /Научный журнал КазНАУ –Алматы. 2019 .№ 3 (83). ISSN 2304-3334 | Омаров Р.А., Исаханов М.Ж., Алибек Н.Б., Касым Р.Т., Талдыбаева А.С. |
|  | Обоснование параметров солнечной электростанции передвижного стригального | Совершенствование методологии познания в целях развития науки: Сборник статей Международной научно-практической конференции (17 апреля 2019 г., Волгоград), в 2 ч. Ч.2/ Уфа: OMEGA SCIENC. 2019. -280 c/ | Талдыбаева А.С., Омаров Р.А., Исаханов М. Ж. |
|  | Современное состояние отечественных стригальных машинок | Проблемы современных интеграционных процессов и пути их решения. Сборник статей по итогам Международной научно-практической конференции. 13 апреля 2019 г. Уфа -2019. С 56-60 | Алиханов Д.М. Цыба Ю.А. Кузьмин Ю.В. |
|  | Современное состояние зарубежных стригальных машинок | Проблемы современных интеграционных процессов и пути их решения. Сборник статей по итогам Международной научно-практической конференции. 13 апреля 2019 г. Уфа -2019. С 60-64 | Алиханов Д.М. Цыба Ю.А. Кузьмин Ю.В. |
|  | К обоснованию технологии и оборудования хранения шерсти | Ізденістер нəтижелер/ исследова-ния результаты /Научный журнал КазНАУ –Алматы.. 2020 №1(85) . –С.420-427 | Омаров Р.А., Исаханов М.Ж., Алибек Н.Б., Касым Р.Т., Талдыбаева А.С. |
|  | Обоснование эффективности использования передвижного пункта искусственного осеменения овец для развития отгонного овцеводства | Ізденістер нəтижелер/ исследова-ния результаты /Научный журнал КазНАУ –Алматы. –2020. –No3. – С.420 -425 | Омаров Р.А., Мусабаев Б.И., Кенжебаев Т.Е. , Алиханов Д. М. |
|  | К разработке и созданию передвижных стригальных пунктов и оборудования хранения шерсти | Ізденістер нəтижелер/ исследова-ния результаты /Научный журнал КазНАУ –Алматы.–2020. –No3. – С.414 -419 | Омаров Р.А., Исаханов М.Ж. |
|  | Исследования и создание технологий и системы машин для отгонного овцеводства | Рекомендации/ ТОО «НПЦ агроинженерии», -Алматы, 2020 -61с. | Омаров Р.А. Байжуманов С. Кудер К.М. |
|  | Статьи опубликованные в базе Scopus |  |  |
|  | Results of experimental studies of a heat pump with compressor self-cooling | International Journal of Mechanical and Production Engineering Research and Development (IJMPERD) ISSN (P): 2249–6890; ISSN (E): 2249–8001 Vol. 10, Issue 1, Feb 2020, 175–184 | Omarov Rashit, Ivaylo Stoyanov, Demessova Saule, Omar Dauren, Yerzhigitov Yerkin |
|  | Energy-Saving Ventilation System for Sheep Premises | International Journal of Advanced Science and Technology, 29(08)2020, 748 - 754. Retrieved fromtp://sersc.org/journals/index.php/IJAST/article/view/18815 | M. Zh. Issakhanov, N.B. Alibek, T.S. Dyusenbayev, A.S. Taldybayeva |
|  | Патенты РК |  |  |
|  | Передвижной пункт искусственного осеменения овец | Патент РК на полезную модель № 5032. от 12.06.2020 На веб-портале РГП «НИИС» | Омаров Р.А., Кудер К.М. Досымбеков Т.Д. Алиханов Д. М. |
|  | Передвижной стригальный пункт для овец | Патент РК на полезную модель № 4610. от 18.07.2019. На веб-портале РГП «НИИС» | Омаров Р.А., Кудер К.М. Досымбеков Т.Д. Исаханов М.Ж. |
|  | Комплекс для перевозки и хранения шерсти | Патент РК на полезную модель № 5295 от 12.06.2020. На веб-портале РГП «НИИС» | Омаров Р.А., Кудер К.М. Досымбеков Т.Д. Исаханов М.Ж. |
|  | Машинка для стрижки овец с дисковым ножом. | Патент РК на полезную модель № 4603 от 17.01.2020. На веб-портале РГП «НИИС» | Алиханов Д.М. Цыба Ю.А. Кузьмин Ю.В. |
|  | Авторские свидетельства о защите интеллектуальной собственности |  |  |
|  | Аппаратное обеспечение «Центральный блок автоматического управления системой энергообеспечения животноводческой фермы» | Авторское право. Свидетельство №5543 от 30.09.2019 | Омар Д.Р. |
|  | Программное обеспечение автоматизированной системы энергообеспечения животноводческой (овцеводческой) фермы с использованием возобновляемых источников энергии | Авторское право. Свидетельство №5550 от 30.09.2019 | Омар Д.Р. |
|  | Аппаратное обеспечение «Блок управления системы контроля климата в контейнере хранения тюков шерсти» | Авторское право. Свидетельство № 12430, от 7.11.2020 г. | Омар Д.Р. |
|  | Программное обеспечение «Алгоритм работы для системы контроля климата в контейнере хранения тюков шерсти» | Авторское право. Свидетельство № 12432, от 7.11.2020 г. | Омар Д.Р. |

Главный ученый секретарь ТОО «НПЦ АИ»,

канд. техн. наук                                                                                          Г. С. Жуматай

APPENDIX G

Copies of Published Articles

1. 2018 год

2.

3.

4  2019 г

5.

6.

7.

8.

9.

10.

2020 г

11.

12.

13.

14.

15.  Статьи опубликованные в базе Scopus

16.

APPENDIX H

Copies of patents and copyright certificates for the protection of intellectual property

| Патент РК на полезную модель №4610, от 18.07.2019 г. Передвижной стригальный пункт для овец | Патент РК на полезную модель №4610, от 18.07.2019 г. Комплекс для перевозки и хранения шерсти |
| --- | --- |
| Патент РК на полезную модель №5032, от 12.06.2020 г. Передвижной пункт искусственного осеменения овец | Патент РК на полезную модель № 4603. Машинка для стрижки овец с дисковым ножом |

Копии авторских свидетельства о защите интлектуальной собственности

| Аппаратное обеспечение «Блок управления системы контроля климата в контей-нере хранения тюков шерсти» Авторское право. Свидетельство № 12430, от 7.11.2020 г. | Программное обеспечение «Алгоритм работы для си-стемы контроля климата в контейнере хранения тюков шерсти» Авторское право. Свидетельство № 12432, от 7.11.2020 г. |
| --- | --- |
| Аппаратное обеспечение «Центральный блок автома-тического управления системой энергообеспечения животноводческой фермы» Авторское право. Свидетельство №5543 от 30.09.2019 | Программное обеспечение автоматизированной системы энергообеспечения животноводческой (овцеводческой) фермы с использованием возобновляемых источников энергии Авторское право. Свидетельство №5550 от 30.09.2019 |

Копия заявления на выдачу патента РК

APPENDIX I

Information about the online consultation

APPENDIX J

Copies of drawing documentation of installations

Сборочный чертеж и чертежи основных узлов ППИО

Сборочный чертеж и чертежи основных узлов ПСП

Сборочные чертеж и чертежи основных узлов усовершенствованного образца шерстопресса

Сборочный чертеж оборудования хранения тюков шерсти

Сборочные чертеж и чертежи основных узлов штабелера тюков шерсти

Программное обеспечение системы контроля климата в контейнере

хранения тюков шерсти

- Программа для ЭВМ –  Программное обеспечение «Алгоритм работы для системы контроля климата в контейнере хранения тюков шерсти»

- Дата создания – 06 июня 2020 года;

- Область применения – устройства автоматизированной системы контроля климата в помещениях хранения шерсти, фабрики первичной обработки шерсти, учебные программы подготовки бакалавров, магистрантов и докторантов ВУЗов, научно-исследовательские работы;

- Назначение – обеспечение корректной работы устройств и элементов автоматики выполняющих управление исполнительными устройствами в целях достижения оптимальных условий для хранения шерсти;

- Функциональные возможности – обеспечивает выполнения алгоритма работы на устройствах автоматики системы контроля климата в помещениях хранения шерсти. Поддерживает взаимосвязь между узлами и пользователем посредством технологии ZigBee, используя оптимальную маршрутизацию информации и тем самым создавая надежную связи. Создает необходимые условия (температура и влажность) в контейнере на основе заранее заданных пороговых значений параметров среды;

- Основные технические характеристики – минимальные требования для работы программы:

Intel® Core™ 2 Duo T7250  2 ГГц  и выше. Microsoft .Net FrameWork 4.0 и выше, аппаратное обеспечение для энергообеспечения животноводческой фермы;

- Язык программирования – С#, C++;

- Тип реализующей ЭВМ – персональные компьютеры, ноутбуки;

- Структура данных, условные названия и обозначения. Разрабатываемая система автоматики содержит 1 класс устройств - периферийный блок, который значительно отличается от центрального блока.. В то же время обладают схожими свойствами. Например, режимы работы, используемые протоколы и т.д. Каждый класс имеет свою структуру для хранения логически связанных множеств информации. Так как рассматриваемый блок управления относится к периферийным устройствам, то его структура описывается классом periphery_info. Спецификации типов данных можно найти в документации компилятора AVR-GCC. Ниже приведена структура periphery_info хранения данных для класса «периферийный блок» (таблица 1).

Таблица 1 – структура periphery_info

| Тип данных | Название данных | Тип данных | Название данных |
| --- | --- | --- | --- |
| uint8_t | xbee_id[8]; | struct | DHT22 DHT22[]; |
| uint8_t | xbee_na[2]; | uint8_t | DHT22_quantity; |
| uint8_t | periphery_name[4]; | struct | dim dim[]; |
| uint8_t | xbee_op[8]; | uint8_t | dim_quantity; |
| uint8_t | xbee_panid[8]; |  |  |
| uint8_t | associated; |  |  |
| uint8_t | connected; |  |  |
| uint8_t | time_sync; |  |  |
| uint8_t | enabled; |  |  |
| uint8_t | complete; |  |  |
| uint8_t | mf; |  |  |

В Таблице 2 приведены:

- xbee_id[8] - идентификационный адрес беспроводного модуля;

- xbee_na[2] - сетевой адрес модуля, название (версия) периферийного блока;

- periphery_name[4] - номер сети;

- xbee_op[8], xbee_panid[8] - текущий номер сети;

- associated - подключение к сети;

- connected - подключение к системе;

- time_sync - синхронизированность по времени;

- enabled - активизация периферии;

- complete - завершённость инициализации;

- mf - частота замеров;

- DHT22 –датчик температуры и влажности;

- dim – реле подачи напряжения 220В для включения и выключения вытяжных вентиляторов.

Подробная информация о каждом устройстве приведена ниже.

В таблицах 2 и 3 приведены структуры вышеперечисленных датчиков и исполнителей.

Таблица 2 – структура DHT22

| Тип | Название |
| --- | --- |
| uint8_t | local_id[2]; |
| uint8_t | rom[8]; |
| uint8_t | value[2]; |
| uint8_t | pin; |

Таблица 3 – структура dim

| Тип | Название |
| --- | --- |
| uint8_t | local_id[2]; |
| uint8_t | value[4]; |
| uint8_t | pin; |

Справедливо будет утверждать, что структуры вышеперечисленных устройств схожи. Отсюда напрашивается вывод об объединении их в один класс, но этот вывод будет ошибочным. В свете того, что система будет развиваться дальше, необходимо учесть грядущие модификации в структурах не только носящие данные о перифериях, но и о датчиках и исполнителях, поэтому решение об объединении их в один класс неприемлемо.

Из таблиц 2 и 3 можно выделить значения local_id, хранящее локальный идентификационный номер внутри периферийного блока, value – содержит текущее выдаваемое значение датчика или состояние исполнителя, pin – функциональная переменная, указывающая, к какому выводу подключено то или иное устройство.

- Алгоритм и схема хранения данных

Рисунок К1 – Алгоритм работы программного обеспечения

Рисунок К2 – Структурная диаграмма хранения данных на flash носителе

ПРИЛОЖЕНИЕ К3-2

Аппаратное обеспечение  системы контроля климата в контейнере хранения

тюков шерсти

- Программа для ЭВМ – Аппаратное обеспечение «Блок управления системы контроля климата в контейнере хранения тюков шерсти»

- Дата создания – 06 июня 2020 года;

- Область применения – овцеводство, фермерские хозяйства, фабрики первичной обработки шерсти, учебные программы бакалавров, магистрантов, докторантов.

- Назначение – Блок управления системы контроля климата в контейнере хранения тюков шерсти предназначен для сбора, обработки и хранения данных о температуре и влажности воздуха, беспроводная передача информации оператору.

- Функциональные возможности – данный аппаратный блок способен посредством аналога протокола OneWire считывать данные с датчика температуры и влажности DHT22 с регулируемой частотой от 0,5с до 1 мин. Имея данные по пороговым значениям температуры и влажности устройство включает или отключает вытяжные вентиляторы посредством управления встроенного реле на том же плате через управляющий порт.

- Основные технические характеристики – см. Таблицу 1

Таблица 1 – Основные технические характеристики

| Наименование параметров | Значения параметров |
| --- | --- |
| Название контроллера | ATMEGA2560 |
| Ядро | AVR |
| Битность шины | 8 bit |
| Максимальная частота | 16 МГц |
| Объем памяти для программы | 256 kB |
| Объем ОЗУ | 8 kB |
| Количество аналоговых выводов | 10 |
| Разрешение аналогового входа | 16 бит |
| Напряжения для питания | От 4.5В до 5.5В |
| Максимальная рабочая температуры | + 85 С0 |

Продолжение Таблицы 1

| Стиль монтажа | SMD |
| --- | --- |
| Интерфейсы | I2C, SPI, USART, ISP, JTAG |
| Минимальная рабочая температуры | - 40 С0 |
| Количество выводов ввода/вывода | 86 |
| Количество таймеров | 6 |
| Внутрикорпусный АЦП | Да |
| Тип программной памяти | Flash |

- Язык программирования –Objective C;

- Тип реализующей ЭВМ – персональные компьютеры, ноутбуки / AVR MC;

- Концептуальная архитектура блока управления системы контроля климата в контейнере хранения тюков шерсти:

Концептуальная архитектура блока управления приведена ниже на рисунке К3.

Основные логические функции выполняет 8 битный центральный микропроцессов ATMEGA2560. Кроме самого центрального процессора в схеме присутствуют ZigBee модули для беспроводной связи с другими устройствами, которые можно подключить для синхронизации работы системы. Интерфейсы JTAG и ISP используются для обновления программного обеспечения центрального микропроцессора для программирования процессора. Порт Б (PORTB) микропроцессора используется для считывания данных с датчика температуры и влажности DHT22, в то время как порт Д (PORTD) передает сигналы для управления реле.

Рисунок К3 – Концептуальная архитектура блока управления

- Принципиальная схема центрального блока

На рисунке К4 представлена полная принципиальная схема центрального блока.

Рисунок К4 – Принципиальная схема центрального блока

Она полностью соответствует архитектуре, показанной выше, за исключением присутствия нескольких светодиодов для простоты отладки, схем перезапуска процессора также для упрощения отладки, элементов для повышения надежности и стабильности (дополнительные конденсаторы и резисторы), модуля питания (стабилизаторы) для раздельного питания отдельных компонентов.  Данная схема была создана с помощью инструмента Altium Designer 14.0.

В принципиальной схеме присутствуют более чем 50 отдельных элементов, список которых можно наблюдать в Таблице 2 ниже.

Таблица 2 – BOM (Bill of Materials) – список элементов, необходимых для схемы

| Обозначение элемента | Название отпечатка | Номинал | Краткое описание |
| --- | --- | --- | --- |
| AMS1 |  |  | SOT223 |
| B_1 |  |  | B_SMD1 |
| C_1 | 100nF |  | C_SMD_0805 |
| C_2 | 10uF |  | C_SMD_0805 |
| C_3 | 330nF |  | C_SMD_0805 |
| C_4 | 100nF |  | C_SMD_0805 |
| C_5 | 100nF |  | C_SMD_0805 |
| C_6 | 22pF |  | C_SMD_0805 |
| C_7 | 22pF |  | C_SMD_0805 |
| C_8 | 100nF |  | C_SMD_0805 |
| C_9 | 100nF |  | C_SMD_0805 |
| C_10 | 100nF |  | C_SMD_0805 |
| C_11 | 100nF |  | C_SMD_0805 |
| DJK-1 |  |  | DJK02A |
| L1 |  | 5V | DPAK (Vin, GND, Vout) |

Продолжение Таблицы 2

| L_1 |  |  | L_3525 |
| --- | --- | --- | --- |
| L_2 |  |  | L_3525 |
| P2 |  |  | HDR2X5 |
| rj1 |  |  | rj45_v1 |
| rj2 |  |  | rj45_v1 |
| R_1 | 4.7kOhm |  | R_SMD_0805 |
| R_2 | 330Ohm |  | R_SMD_0805 |
| R_3 | 330Ohm |  | R_SMD_0805 |
| R_4 | 4.7kOhm |  | R_SMD_0805 |
| R_6 | 4.7kOhm |  | R_SMD_0805 |

12) Схемотехническая модель центрального блока

Основываясь на разработанной утвержденной принципиальной схеме, была разработана схемотехническая модель блока управления, которая состоит и 2-х слоев и представлена на рисунке К5.

| а) | б) |
| --- | --- |

Рисунок К5 – Слой 1 (а) и слой 2 (б) схемотехнической модели центрального блока

На схемотехнической модели можно наблюдать расположение всех компонентов с обозначениями, отверстий и соединяющих дорожек. Желтым цветом обозначены габариты будущих компонентов, красным – электрические области слоя 1, темно-синим - электрические области слоя 2, темно-зеленым – отверстия, серым – область припоя, фиолетовым – маска припоя. Стоит отметить, что к слою 1 был применено полигонирование для снижения влияния электромагнитных волн на схему и упрощения разведения платы.

APPENDIX K

New Zealand Mission Report

Новая Зеландия занимает ведущее место в мире по производству высококачественной,исключительно тонкой шерсти меринос.

Конференция и сопутствующая программа тура предоставили вам возможность увидеть, обсудит, изучить и проанализировать вопросы не только о тонкой шерсти, но и о новозеландских инновационных системах разведения овец и сельского хозяйства в целом.

Тур начался в г. Крайстчерче, расположенный на южном острове, и направится в северном направлении по всему Северному Кентербери, Мальборо и западному побережью. Затем направились на юг в г. Кромвель в Центральном Отаго, где состоялась Конференция. Она была приурочена к шоу A & P UpperClutha в Ванака. Центральный Отаго представляет собой фантастический регион для этого мероприятия с большим разнообразием достопримечательностей и развлечений, а также посещение заводов в регионе Маниотото.

Из Кромвеля отправились на север в знаменитую страну Маккензи. Здесь посетили несколько Мерино-хозяйств (отар), многие из которых расположены в районе впечатляющие Южных Альп с голубыми озерами. После ночи в Омараме путь продолжился дальше на север через разнообразные Кентерберийские равнинные платос обратным возвращением в г. Крайстчерч. На следующее утро 19 марта 2020 года вылетели в обратный путь с аэропорта Крайстчерч в Окланд, откуда предстоял полет в Токио в аэропорт Нарита.

Крайстчерч, самый большой город и ворота в Южный остров. Из-за обилия парков и скверов получил название «Город-сад». В настоящее время переживает фазу драматической модернизации после разрушительного землетрясения 2014 года. Многие здания в стадии строительства.

В день приезда, 7 марта, состоялся приветственный ужин, знакомство и общение с коллегами, специалистами и экспертами международного уровня из России, Новой Зеландии, Австралии, Аргентины.

8 марта 2020, после завтрака, отправились по маршруту КРИСТЧЕРЧ – БЛЕНХЕЙМ. Путь пролегал через увлекательные и красочные сельхозугодия Северного Кентербери в небольшую деревню Шевиот.

После небольшого перерыва, отправились в хозяйство “The Gums”, где расположены мерино стада-отары.

THE GUMS MERINO STUD – в Лимингтоне, к западу от Шевиот. Ферма организована  Стивенсоном в партнерстве с Яном Триш, их сыном Марк и его женой Джоан. Доход фермы Стивенсонов является результатом реализации шерсти Мерино и полуфабрикатов, а также продажи около 250 баранов, пяти разных пород каждый год.

Во второй половине дня состоялась поездка в Каикоуру, маленький городок в живописное место между горами высотой до 3000 м и морем, которое стало важным для туризма пунктом. Особенно интересен просмотр удивительной морской жизни, который можно наблюдать на берегу. После перерыва поездка продолжилась вдоль живописного побережья до Бленхейма в самом центре винной страны Мальборо.

Понедельник 9 марта 2020 г. Мальборо / BLENHEIM.

После завтрака экскурсия продолжилась по Мальборо с посещением ведущих хозяйств мериноса в долине реки Аватере (Awatere Valley). Сухой и теплый  климат Мальборо в сочетании с широкими речными долинами и гористой местностью, прекрасно подходит для производства шерсти. Состоялось посещение Blairich Stud, где содержатся породы: Awapiri, Glenlee, Isolation и Upcot и смешанные породы Middlehurst/Muller.

Последние 8 лет хозяйствами в долине Аватере, Мальборо, управляют Эрик и Салли Смит. Хозяйство было основано отцом Салли Грэмом Блэком в 1975 году. Эрик и Салли производят среднюю комбинацию рогатых мериносов в основном для обслуживания своей коммерческой паствы и продажи 25-30 баранов каждый год.

Обед состоялся на одной из ферм, чтобы гости могли почувствовать Гостеприимство организованное на природе с фермерами. Этим же вечером, на ужин, состоялось угощение барбекю на одной из станций. Позже вернулись в Бленхейм, где переночевали.

GLENLEE STATION. Располагается в 60 км через Awatere Valley на повороте от главной дороги. Хозяйство, зарегистрированное в 1969 году, находится в собственности семьи Гамильтонов (Hamilton family).

Роб и Шэрон Гамильтон осуществляют  производство мериносовой шерсти Flock # 197. Также содержат крупный рогатый скот ангус/герефорд (Аngus/Hereford). Разведение, отбор и классификацию осуществляет Роб Гамильтон.

MIDDLEHURST & MULLER STUDS.

В 1980 году, когда Стив и Мэри Саттервейт вступили во владение собственностью в 38 860 га в Awatere valley, тонина овечьей шерсти составляла в среднем 23,9 микрон, а настриг 5 кг/гол. Процент ягнения – 85-95%. Шерсть являлась основным источником доходов.

Сегодня поголовье овец увеличилось на 20%. Тонина шерсти доведена в среднем до 20,5 микрон с настригом 6,3 кг на голову. Чтобы обеспечить стабильность дохода, 70% шерсти продается по контракту на открытом аукционе и их закупают компании по производству одежды для отдыха, включая теплую одежду для проживания в исключительно холодных условиях.

MARLBOROUGH/BLENHEIM - FRANZ JOSEF

10 марта 2020 г. утром выехали из Бленхейма. Маршрут пролегал вдоль реки Вайау, вглубь суровых гор и речных долин Южных Альп через перевал Льюиса. Западное побережье исторически известно производством древесины, золота и добычей угля. Климат намного влажнее из-за распространения влаги западных ветров от Тасманского моря.

Южные Альпы образуют сильные осадки на западный склон гор и узкую полосу пологой земли, прилегающей к океану. Это привело к развитию тропического леса на большей части побережья. Некоторые преобразованы в пастбища для мясного и молочного животноводства. После перерыва на обед и остановки в Пунакайки и Блин путь продолжился на юг через города Греймут и Хокитика (Greymouth and Hokitika) в страну ледников и Франц Йозеф (Glacialcountry and Franz Josef).

BLAIRICH MERINO STUDS

Blairich Merino и Poll Merino Stud находятся в совместной собственности. Рон и Сью Смолл в партнерстве со своим сыном Тома и жены Клэр. Состоит из 800 овец от прекрасного среднего типа. Порода очень стойкая к гниению копыт с использованием asbv's. Овцы являются победителями многих конкурсов. Blairich Merinos известны своим качеством шерсти.

FRANZ JOSEF - CROMWELL

Ранним утром 11 марта 2020 г. представилась возможность посмотреть на Ледник Франца Йозефа. Далее путь пролегал на юг вдоль береговой линии, через тропический лес, а затем на восток через перевал Хааст в Центральный Отаго. На этой стороне климат намного суше горы с мериносовым овцеводством, особенно вокруг озер Хавеа, Ванака и Дунстан.

CROMWELL

В четверг 12 марта 2020 г., весь день - конференция MerinoExcellence 2020 в Кромвеле.

Краткие сведения о докладчиках.

СТИВ РОЧЕСТЕР

Руководитель компании Boehringer Ingelheim Animal Health NZ Ltd. Выпускник Университета Линкольна в области сельского хозяйства, пищевой биологии и химии. Стив Рочестер - признанный и уважаемый лидер с заметными достижениями в области ветеринарии. Имеет значительный международный опыт, лидер в индустрии здоровья животных за последние 35 лет.

Интеграция предприятий и команд Boehringer Ingelheim и Merial Animal Health, благодаря приобретению Boehringer Ingelheim бизнеса Merial - январь 2017 года.

В 2007 году в качестве главы Merial Oceania Стив содействовал приобретению Ancare это стабилизировало австралийский и новозеландский бизнес. Стив ведет бизнес, чтобы стать ветеринарным врачом, пока интегрируя бизнес Ancare в Merial, успешная интеграция привела к росту доходов и рыночной доли.

В 2000 году в качестве руководителя компании Merial Global Ruminant Enterprise, базирующегося в Лионе (Франция), а затем в Дулуте, США, Стивом выявлены и реализованы бизнес-стратегии для жвачных. Будучи региональным менеджером MSD Agvet NZ, Стив определил возможность и договорился с MSD Agvet о приобретении Nufarm. Бизнес по охране здоровья животных, в результате которого Nufarm произвела целый ряд продуктов IVOMEC для MSD  Agvet во всем мире в обмен на MSD Agvet получает контроль над рынком капсул для овец в Австралии и Новой Зеландии.

Д-р МАРК ФЕРГУСОН

В neXtgen Agri мы стремимся к тому, чтобы обширные отрасли животноводства добились оптимальных показателей генетического роста путем принятия своевременных и обоснованных решений. Мы знаем, что эффективный и прозрачный поток генов раскроет огромный потенциал отрасли животноводства во всем мире. Мы стремимся преобразовать сложный язык и системы генетической науки в интуитивные решения. Мы стремимся воплотить гены в жизнь.

Менеджер по науке производства: новозеландской компании Merino, Крайстчёрч, с ноября 2012 – по январь 2018, 5 лет 3 месяца.Управляет командой, которая меняет сельское хозяйство Новой Зеландии, позволяя фермерам быть болеевыгодными, устойчивыми и прозрачными. Текущие рабочие потоки включают генетическую устойчивость к Footrot, датчики на овец для мониторинга благосостояния и разработку ряда учебных материалов.

ДЖЕЙМИ УАЙТИНГ

Джейми является генеральным директором группы компаний Barkers и Max. Оба бренда являются знаковыми в Новой Зеландии и переживают взлеты и падениярозничной торговли. Он был вовлечен в розничную торговлю в течение последних 27 лет, пройдя через различные роли, изучил все тонкости отрасли. Сейчас он сталкивается с проблемой воздействия на окружающую среду швейной промышленности и играет ключевую роль в изменении производства брендов и бизнес-модели, чтобы быть частью решения.

CATHRINE STANGE

Катрин является генеральным директором DevoldofNorwayGroup. DevoldofNorway - вертикально интегрированный специалист по шерсти снаследием, восходящим к 1853 году. Сегодня DevoldGroup включает в себя несколько компаний, ориентированных на производство, продажии маркетинга, а также розничной торговли. Катрин присоединилась к Devold в 2014 году и возглавляет компанию с акцентом на качество, инновации и управление брендом.До прихода в Devold, Катрин работала в ExxonMobil в Норвегии, Великобритании и Бельгии, занимая различные руководящие должности в сфере розничного маркетинга, маркетинговых коммуникаций и развития бизнеса. Она также работала в ReitanGroup вНорвегии, возглавляя отдел продаж и маркетинга крупнейшей в Норвегии сети продуктовых магазинов, а также розничной торговли.

Катрин занимает несколько должностей в совете директоров, а также входит в состав совета Норвежской ассоциации спортивной индустрии.Катрин считает, что энтузиазм является величайшей движущей силой для достижения результатов, и она верит в шерсть, страсть и людей.

Д-р ДЭЙВ БОЛДУИН

Доктор Дейв, Летучий Доктор, является авиационным медицинским экспертом, пилотом и автором нескольких публикаций и книг по вопросам здоровья идуховного благополучия необходимого для хорошего здоровья. Увлеченный на природе человек и заядлый охотник, он может вспомнить множество приключений.

Доктор Дейв летает в самые отдаленные районы Новой Зеландии, предоставляя пилотам ежегодную медицинскую помощь, необходимую для продления лицензии. Прекрасный рассказчик смешных историй и анекдотов, которыми может развлекать часами.

ДЖОН ХИКФОРД

Джон - профессор животноводства и генетики в Университете Линкольна, читает лекции в Линкольне с 1989 года.Исследования Джона сосредоточены на молекулярной генетике крупного рогатого скота и овец, с акцентом на гены, которые а) лежат в основе молока,производстве мяса и шерсти, и б) которые делают животноводство более устойчивым к болезням и экологическим проблемам.

Ключевая цель его исследования заключаются в разработке генных маркеров для животноводства, и он был ответственным за доставку десятикоммерческих генных маркерных тестов на рынок.Джон возглавляет исследовательскую группу, в которую входят два постоянных сотрудника, сотрудники и иностранные посетители, а также студенты. Автор более 200 рецензируемых статей в международных журналах. Он поддерживает высокий уровень последипломного надзора содиннадцатью аспирантами AgMardt. Джон преподает 17 курсов на уровнях бакалавриатаи магистратуры в областях

животноводства, науки о шерсти, молекулярной биологии, генетики и селекции и устойчивости. Он был первымобязательного курса для всех студентов в Линкольне, и курса, на котором онсосредоточены на устойчивости в сельском хозяйстве и проблеме глобального изменения климата. Регулярно консультирует фермерские группы Новой Зеландии и Австралии.

Проживание:

In 7, out 8 March, 2020 - 1 Night

ChateauOnThePark - ChristchurchADoubletreeByHilton

189 Deans Avenue,Christchurch, New Zealand

In 8, Out 10 March - 2 Nights

Scenic Hotel Marlborough, Blenheim

65 Alfred StBlenheim 7201, New Zealand

In 10, Out 11 March - In 1 Night

Scenic Hotel Franz Josef Glacier

State Highway 6,Franz Josef Glacier, 7856,New Zealand

In 11, Out 17 March – 6 Nights

Harvest Hotel

The Gate 6 Barry Ave,Cromwell, New Zealand

In 17, Out 18 March

The Heritage

State Highway 8PO Box 20Omarama 944, New Zealand

In 18, Out 19 March

Chateau On The Park - Christchurch A Doubletree By Hilton

189 DeansAvenue,Christchurch,NewZealand

На конференции из стран СНГ участвовало 10 человек, в том числе 7 человек из России:

Егоров Михаил Васильевич – Генеральный директор Национального союза овцеводов Российской Федерации. Телефон/факс: (8652) 75-21-24, (8652) 35-44-61.

Email: info@rnso.net. Адрес: 355035, г. Ставрополь, ул. Мира, д. 337

Поздняков Александр – учредитель ООО «Вива-Лаб», председатель Союза овцеводов Хакасии.

Абурахманов Магомед – Президент Ассоциации фермерских хозяйств и сельскохозяйственных кооперативов Дагестана.

Чураев Абурахман – руководитель СХК «Агрофирма «Согратль», Дагестан

Бронникова Галина – генеральный директор АО "Забайкальский центр племенного животноводства", Забайкальский край РФ

Дондокова Дамира – генеральный директор сельскохозяйственного производительного кооператива "Родина". Бурятия

Корниенко-Жиляев Юрий Алексеевич, генеральный директор ООО «Венера-Вет», к.б.н.

Из Казахстана:

Мусабаев Б. – д. с.-х. наук, директор НИИ овцеводства им. К.У. Медеубекова, филиала ТОО «КазНИИЖиК»

Омаров Р. – д.т.н, зав лаборатории «МТПЖ» ТОО «НПЦ агроинженерии»

Кудер К. – специалист по коммерциализации лаборатории «МТПЖ».

Из Окланда вылетели на день позже – 20 марта 2020 года из-за отмены рейса в связи с карантином. В Алматы прилетели 23 марта, с задержкой на 2 дня.